554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

Fax: 985-641-5950

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

October 27, 2015

For: 	Tamie Johnson

56184 Redmill Drive

Slidell, La. 70461

Construction:

Single-story, wood frame with brick veneer and composition shingle roof on a pier and beam foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the residence.  No inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers Standards and Practices for Residential and Small Building Property Condition Surveys.  This report is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering received a request to perform a structural inspection of the referenced home due to possible elevation differences noted in an appraisal report.

Findings:

An inspection was made on the interior of the residence and found that they are noticeable differences in the ceramic flooring elevation thru out the home.

An inspection was made of the exterior of the residence, which revealed no new cracks in the brick veneer but could see where patching of the joints in previous years. The cinder blocks that support the beams have settled over time and it is visibly noticeable where it was attempted to re level with the use of a wood shims.

Analysis:

A zip level was used to measure the floor elevation differences that are visibly noticeable in the home.  The greatest difference in elevation measured was at -2.5 inches in the hall bathroom. See the attached floor plan.

Conclusion:

It’s important to note that all foundations settle with time.  Usually everything settles together, and the fastest rate of settlement usually occurs during the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

It is my opinion that the structural integrity of this home is currently sound, but the foundation will continue to expand and contract within the normal range of movement for this area.

The beams can be re leveled with the use of wood shims.

Sincerely,

Brian Mistich

PE:30187