554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

May 23, 2016

For: 	Alex Sullivan

1347 Greenlawn St.

Slidell, La. 70460

Construction:

Single-story, wood frame, brick veneer and siding, vinyl and wood flooring with a composition shingle roof on a conventional foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the residence. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to a home inspection report that noted some bricks that are cracked.

Findings:

An overall visual inspection of the exterior of the home was conducted and several hair line cracks in the brick veneer were noted.

A). Above the right side of garage, on corner B). Right side of garage wall, from top to bottom C). Rear patio wall, from window sill to patio foundation (patio foundation is blocking view to foundation). No cracks in the foundation were noted.

The interior walls of the home were inspected also and no cracks were noted in the sheetrock at the time of inspection.

Upon inspection, a zip level was used to verify the floor height conditions.  The level was set up at the front door at zero inches and measured throughout the home, there is a one and a half inch difference noted to the left rear of the home.  It is important to note that all floor height conditions are within acceptable tolerances.

The attic was also inspected and no broken roof rafters or ceiling joist were noted at the time of inspection.

Conclusion:

It is important to note that all house foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

It is my opinion that the structural integrity of this residence is currently sound.

Recommendation:

It is recommended that the cracks be sealed to prevent insects from entering the home.

Sincerely,

Brian Mistich, P.E.