554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

May 24, 2017

For: 	Robert Seaton

139 Everest Dr.

Slidell La. 70458

Construction:

One-story, wood frame, brick veneerl, ceramic and wood flooring with a composition shingle roof on a conventional foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the home due to a home inspection that noted some cracks in the brick veneer in the rear of the home. This home has had some flooding issues in the past due to improper yard drainage. To mitigate the flooding problem the previous owner installed a tar barrier on the bottom bricks to keep water out. Some areas of the home have gutters and subsurface drainage out to the street.

Findings:

Located in the rear of the home there is one long crack in the brick veneer mortar that you can visibly see. It starts at the bottom of the bathroom window and runs diagonally to the foundation of the home and appears to stop at the foundation.

The owner was able to dig down past the barrier and we were able to verify that it didn’t go into the foundation.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

Due to the fact that there are no visible signs of distress in the interior wall, ceiling or flooring in the bathroom that is adjacent to the outside crack, it is likely that the brick veneer settlement has subsided. There were no signs of the crack going into the foundation.  At this time it appears stable and does not appear to compromise the integrity of the structure.

- It is recommended where the crack is visible in the brick veneer be patched with mortar.

- Also the yard should have proper drainage to keep water away from the foundation.

Sincerely,

Brian Mistich, P.E.