554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

April 11, 2019

For: Kevin Sims

16156 Bali Rd.

Pearlington, Mississippi 39572

Construction:

One-story raised, wood frame, wood siding, ceramic and carpet flooring with a composition shingle roof on a post and concrete foundation.   The home is approximately 11 years old.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to conduct a structural inspection of the referenced home due to an inspection that noted sagging and warped beams under the home.

Findings:

An overall visual inspection under the exterior of the home was conducted and noted that four of the 	beams in the center of the home are spliced together between two columns and are sagging. At least 	two exterior columns are warped and only appear to be cosmetic. It was also noted that the exterior  	porch on the front and rear have floor joist that are scabbed onto the floor joist under the home and 	have failed.

Recommendation:

Either remove and replace or add onto the existing beams in question with a 2x12. The warped columns can be added onto with a single 2x8 and bolted together to eliminate anymore warping. The exterior porch floor joist should be replaced with floor joist all the way back to the beam of the house. It is my opinion that the structural integrity of this home will be sound with these repairs.

Sincerely,

Brian Mistich, P.E.