554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

Fax: 985-641-5950

dammonengineering.com

info@dammonengineering.com

August 18, 2023

Mr. Dallas Guidry

4501 Providence Pl

Apt 1B

New Orleans, La. 70126

Re:

1753 Marsha Dr

Slidell, LA. 70458

Dear Mr. Guidry,

You requested that we inspect the referenced residence for the removal of a load bearing wall.

A double 2x12 or Parallam 2.0E 3.5x9.5 beam that is not longer than 12 ft in length and constructed to meet the requirements of the 2021 IRC will be structurally sufficient to meet the loads applied to the load bearing wall.  See attached plan.

This report does not cover any other aspects of the building such as plumbing, electrical, gas, mechanical egress, general arrangement, site drainage, etc., and assumes that ground conditions are substantially the same as is under the residence.

Sincerely,

Brian A Mistich, P.E.

La. License No.  0030187