554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

October 3, 2017

For: 	Chad Newcomer

37160 Grist Mill Blvd.

Geismar, La. 70734

Ref:     2010 Green Ct.

Mandeville, La. 70448

Construction:

One-story, wood frame, Stucco, brick veneer, vinyl siding, ceramic and wood flooring with a composition shingle roof on a conventional foundation.  Home is approximately 18 years old.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to a home inspection that noted some roofing that is not flat on the right rear of the home.

Findings:

An overall visual inspection of the exterior roof of the home and attic was conducted and couple items were noted.

The decking on the right rear of the home is buckled over the ceiling joist causing a valley in that area. Inside the attic you can visually see the ceiling joist is not cut off to allow the roof decking to be level.

The current home owner had hired a contractor to install additional roof supports thru out the attic supporting the roof. No leaks were noted from inside the attic.

Conclusion:

Although it is visually apparent when looking at the roof from outside that roof is buckled in this area, the roof is structurally sound.

Recommendation:

The buckle in the roof does not appear to have compromised the structural integrity of the home.  It is my opinion that the structural integrity of this home is currently sound.

Sincerely,

Brian Mistich, P.E.