554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

June 16, 2020

For: Joan and John Ascherl

226 Hoffman Lane

Waveland, Ms 39576

Dammon Engineering was contacted to request a structural inspection of the referenced new home due to the homeowner noting rainwater build up on the raised covered rear porch and the concern of the structural attachments for it.

Findings:

The tongue and grove decking is installed overall level allowing water pockets to build up and drain towards the house and onto the floor below.

The 2x12 floor joists are installed from side to side with tongue and grove decking attached running front to back.  The 2x12 floor joists are attached to (3) 2x12’s in the center to create a beam (one of the 2x12’s is scabbed on and not in a joist hanger on either side).  Some of the 2x12 joist hangers are of different sizes.   One of the double 2x12 floor joist are spliced together but in a double joist hanger.

After reviewing the construction drawings it was found that the 2x12 floor joist @ 16” O.C. are designed from front to rear for the covered rear porch but does not state what type of decking to use.

Traditionally, deck boards are laid running perpendicular to the house with a slight slope to it between 1⁄8" and ¼" per linear foot for tongue-and-groove floors.  If the porch is deeper than 8' then a 1⁄8" per foot is recommended to avoid a pronounced slant.

Recommendation:

- Remove spliced double 2x12 on double joist hanger and install single 2x12 with proper joist hangers.

- Remove improperly sized, approximately 3 joist hangers, and install proper joist hangers for 2x12 floor joist.

- Remove double joist hangers from center beam and install joist hangers for triple 2x12 for a beam and bolt triple beam together using the pattern attached.

- Install caulking at top between decking and house to prevent water from draining onto the floor below.

- Saw cut the 2x4 bottom plate of the porch in four places to allow build up of water to runoff.

- It is recommended that this porch not be used for heavy loads i.e., spa/hot tub or pool.

Sincerely,

Brian Mistich, P.E.