554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

May 24, 2022

James Kevin Curley

2800 Gause Blvd.

Slidell, La. 70461

Construction:

Two-story, wood frame, brick veneer, with a composition shingle roof on a conventional foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the building. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced building due to a recent insurance inspection that noted some cracks in the rear brick veneer wall.

Findings:

An overall visual inspection of the exterior of the building was conducted and a couple items were noted.

- There are some bricks that have cracked mortar around them horizontally and vertically; the cracks in the brick veneer do not show up in the foundation.

- Two down spouts at the corners of the bldg are not lined up to flow onto the backsplash blocks.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

The cracks in the brick veneer do not appear to have compromised the structural integrity of the building.  It is my opinion that the structural integrity of this building is currently sound, but the foundation will expand and contract with the ground movement and any standing water.

Items to be addressed are as follows:

- Seal the cracks in the brick veneer with a non-shrinking grout, to prevent insects from entering the building.

- Align downspouts on back of building to flow onto the backsplashes provided.

Sincerely,

Brian Mistich, P.E.

Pictures attached