554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

March 22, 2023

Ed Lafferty

3007 S. Palm Dr.

Slidell, La. 70458

Construction:

Raised One-story, wood frame, brick/hardy veneer, ceramic & concrete flooring with a composition shingle roof.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to cracks in the ceramic flooring.

Findings:

The rear dining room and front master bedroom hallway flooring was noted to have cracks in them.  It was noted there are stress cracks in the sheetrock walls above the dining room floor that is cracking. None noted on the walls of the master bedroom hallway.

The exterior of the home was noted to have cracks in the stucco below the dining room wall and around the fireplace wall.  The front Master bedroom rear exterior wall was noted to have cracks in the brick veneer and the front stucco wall also. The front left bedroom exterior wall was noted to have cracks in the stucco.

It was also noted that there are no gutters or down spouts in these two areas of concern allowing water to settle under the foundation. Rainwater is splashing onto the exterior walls creating mold. This also allows for settlement of the soil around the home and will create uneven floors and crack sheetrock walls. Due to a high water table in this area it is necessary to keep the foundation dry.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

It is our opinion that the structural integrity of this home is currently sound, but the foundation will expand and contract with the ground movement and any standing water.  It is recommended when there is more than a 3” elevation difference (tripping hazard) to have the home stabilized with screw pilings.

Items to be addressed are as follows:

- Add gutters and downspouts and tie into subsurface drainage on all downspouts that are needed to allow for proper drainage of rainwater away from the foundation.

- Repair flooring

- Repair sheetrock.

Sincerely,

Brian Mistich, P.E.