554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

August 5, 2021

For: Timmy Gennusa

3256 William Tell

Slidell, La. 70458

Construction:

One-story, wood frame, vinyl veneer, vinyl plank flooring, composition shingle roof on a CMU pier and beam foundation.

Scope:

This inspection is limited to a visual inspection of the CMU piers of the home; No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the CMU piers due to a previous inspection that noted an elevation differences in the flooring of the home.

Findings:

Upon inspection of the home, the ground around the home and under the home was noted to be wet and holding water.

The CMU piers were inspected and it was noted that the right side exterior of the home has two piers that are out of alignment, there is also one CMU pier in the rear of the home that is cracked.

At the time of this inspection, the interior of the home was noted not have any cracks in the sheetrock ceilings or walls.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

The elevation differences do not appear to have compromised the structural integrity of the home.  It is my opinion that the structural integrity of this home is currently sound, but the foundation will expand and contract with the ground movement and any standing water.  It is recommended when there is more than a 3” elevation difference (tripping hazard) to have the home stabilized with screw pilings.

Items to be addressed;

Align and fix CMU blocks as noted.

Install gutters and down spouts to direct water away from and under the home.

Install fill under the home to keep the area dry.

Sincerely,

Brian Mistich, P.E.