554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

July 11, 2017

For: 	Glenda & Dave Schmidt

59344 Lacombe Harbor Road

Lacombe La. 70445

Construction:

One-story, wood frame, wood veneer, with a composition shingle roof on a conventional foundation that has been elevated.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to the owners concerns of home being raised and materials used in construction.

Findings:

The home was elevated as part as a federal grant and in doing so new stairs had to be added to give access to the home owners for safety. Also, the emergency generator and A/C’s had to be elevated and with new platforms and stairs for maintenance.

As part of the elevation the new enclosed area below the home, code requires either static vents or decorative vents be added per the square footage of the home as to not allow the flood waters to knock down the foundation of the home.

It was noted during the inspection that static vents are not operable (meaning they do not open) they appear to be held in the closed position due to the installer not properly removing the mortar around the frame of the static vent and most of the frames are bent at the top not allowing them to open in the advent of flooding.

The new stairs leading to the platforms are of wood construction and appear to not be installed with the proper fastening materials as the wood is warping up on the ends and not allowing for a safe access to the equipment for maintenance.

The front entrance concrete stairs have no static vents or decorative vents and the owner stated that no dowels are installed to connect the concrete stairs to the home which could fail during a flood.

Conclusion:

It is important to have some type of vents in the new enclosed area below the home. This will allow storm water to penetrate the enclosed area without knocking it down. The area below the home has two types of vents installed. The front wall has decorative and the remaining walls have static vents.

During the inspection the under the home it was noted that the concrete foundation above is sweating due to lack of proper ventilation and will eventually cause mould to build.

Recommendation:

Items to be addressed are as follows:

- Remove all static vents and install a decorative type vent for better ventilation.

- Remove and install new boards on the stairs that have warped and install with proper screws.

- Install new decorative type vents in the front concrete stairs and tie the stairs into the foundation of the home.

- ?

Sincerely,

Brian Mistich, P.E.