554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

December 22, 2021

Franklin Annerino

607 Florida St.

Mandeville, La.

Construction:

One-story, wood frame, vinyl veneer, ceramic flooring with a composition shingle roof on a pier and beam foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to a previous inspection that noted some hairline cracks in the CMU blocks under the home.

Findings:

This is a pier and beam foundation. The beams are placed where load bearing walls are located with CMU blocks on top of it to support the wood beams.  An overall visual inspection of the CMU blocks under the home was conducted and it appears to have three CMU blocks that have hairline cracks in them.

It was also noted that there are some downspouts that are draining out the rear of the home and some right onto the ground.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

The hairline cracks in the CMU blocks do not appear to have compromised the structural integrity of the home.  It is my opinion that the structural integrity of this home is currently sound, but the foundation will expand and contract with the ground movement and any standing water.  It is recommended when there is more than a 3” elevation difference (tripping hazard) to have the home stabilized with screw pilings.

Items to be addressed are as follows:

- Seal the cracks in the CMU blocks with a non-shrinking grout.

2)  Add subsurface drainage on all downspouts that are needed to allow for proper drainage of rainwater away from the foundation.

Sincerely,

Brian Mistich, P.E.