554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

06-20-2022

Jason & Rebecca Massa

61173 N. 15th St.

Lacombe, La. 70445

Construction:

One-story, wood frame, composition shingle roof on a CMU pier and beam foundation, carpet and wood flooring.

Scope:

This inspection is limited to a visual inspection of the interior of the home; No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the home due to large oak trees that fell on the roof of the home during hurricane Ida on 08-29-2021.

Findings:

Upon inspection of the home, two large oak trees had fallen on the front of the home causing severe structural damage to the roof, roof decking, roof rafters, ceiling joist and sheet rock in the master bedroom area and pulled the main electrical feed off the house. The open porch on the front of the home and side porch has several broken railings, steps and decking and is no longer level and not safe to walk on.

A zip level was used inside the home to verify the floor height conditions throughout. The level was set up at the front of home in the foyer. As per the attached sketch it shows that the home is not level

on its CMU piers.

The interior of the home shows signs of damage from the force of the tree falling on the home. All the floors are uneven; the doors into the bedrooms are loose from the door jambs. Master bedroom sliding glass door onto the porch leans outward and is not level for proper operation. The master bedroom closet ceiling has a hole in it and mold growing. Several areas throughout the home have cracks in the sheetrock walls and ceilings along with water stains.

Conclusion:

Due to the force of the impact from the trees falling on it, it has created severe damage to the structure of the home.

Recommendation:

It is recommended to have the home leveled prior to any repairs.

Items to be addressed are as follows:

- Stabilize and level CMU piers throughout home

- Replace top plates in the damaged area

- Replace cracked ceiling joist in the damaged area

- Replace roof rafters in the damaged area

- Replace roof decking in the damaged area

- Replace asphalt shingles throughout entire roof

- Replace air-conditioned ductwork in the damaged area

- Replace insulation in the damaged area

- Replace sheetrock in the master bathroom closet ceiling

- Repair sheetrock cracks throughout home

- Replace damaged flooring

- Paint all affected areas

Sincerely,

Brian Mistich, P.E.

See the attached