DAMMON ENGINEERING, INC.

dammonengineering.com


CONSULTING

DESIGN

STUDIES

EXPERT WITNESS


1095 Florida Ave.



   P.O. Box 2830



     985-649-5832

Slidell, LA 70458



Slidell, LA  70459


            FAX 985-641-5950

STRUCTURAL CERTIFICATION

Date:

 June 05, 2007

Location/


Mailing address:
Charles Penn                         Phone:  504-512-0734

1228 Louisa Street

                                 New Orleans, LA 

Building Description:
Wood frame, lap board siding, composition shingle roof with brick and concrete foundation with piers
Scope:

This inspection is limited to a visual inspection of the interior and exterior stud walls, ceilings, floors, roof and foundation.  No inspection of the mechanical or electrical systems was performed, but these systems appear to be in working order.
History:

Mr. Charles Penn contacted Dammon Engineering to request a structural inspection on the referenced residential property to survey damage caused when the adjacent house collapsed onto this residence.

Findings:

The roof is partially damaged in two places and it appears that two or more rafters are partially damaged and will need to be repaired or replaced.  One window in the middle section of the residence was destroyed when a large section of lumber fell through and penetrated an interior wall and ended in a bedroom where a child was sleeping.  From the middle section of the residence to the rear, the floor is sagging noticeably to the left where it was damaged.  A window in the kitchen is knocked out of square.  There are stress cracks in the plaster along the ceiling and down several interior walls along this side of the residence. The outside wall of the rear 1/3 of the residence is sagging and will need to be raised and properly supported to restore the building to a level condition.
Mr. Penn reported that a very large pecan tree had fallen on the rear of the house during Hurricane Katrina and that settling of the rear of the house was sustained at that time, including the kitchen window.

Analysis:

Although this residence has damage, it is capable of being restored. I certify the above residence is structurally sound and capable of being restored to its original design.

Sincerely,

Emmett Dammon

La. License 8796

Attachment:
Photos of damage
This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


