Site visit 4-29-08  pre-pour inspection

The following items were noted during the on-site visit:

1. Measure the pedestals to ensure they are in the proper place.  See attached for measurements.

2. Connect the rods inside the footing behind the pedestal.

3. The pedestals are to have #4 bars pointing toward the opposite pedestal. See detail H on S-2

4. The form and top of column pedestal are not the same height.

5. Washout was seen around parameter of foundation, clean the footings out and compact the foundation to meet 95% proctor.  The slab will be held by the foundation.

[image: image1.jpg]



6. A hole was dug from the foundation under the footing to drain water.  This entire area will have to be compacted in lifts of no more than_________.

[image: image2.jpg]



7. Rods from the footing are touching the form built from metal forms.  Any metal that is exposed to dirt foundation will introduce rust into the slab system.  Ensure all metal is separated from the possibility of moisture.

[image: image3.jpg]4
i i
:
r i
B
|
L
£
4
:
g \
, - -
v £
S
Beo
Z
2o
- i
- /)
&
A g
¢ S 7
i o 7
<
e 3 X, -
. : o
X \',"‘ Be.
% < ;

2





8. Two pedestals are not setup at the rear of the building.

[image: image4.jpg]



9. Rear ramp is not formed.

[image: image5.jpg]



10.  Re-dig the thicken slab at proper thickness and install #4 bars as shown on drawings.

11.  Pull grass from foundation.

12.  Lay WWF

David Dammon 

Dammon Engineering

