# PLANNING PROJECT REVIEW SUMMARY

# PROJECT NAME

# PROJECT ADDRESS

# Permit No. __19-1084_____

# DATE

# The following is a review and summary of the permitting plan set using the Planning Department’s Commercial Checklist.  Items outlined below are required to complete the Planning Department’s review for approval of construction permit.

# Cannot determine type of metal panels on elevation- must be 26-gauge or better and of a like color to match and compliment stucco.

# No details provided on overhead door. Must match or compliment stucco.

# At least 3 camellias needed on landscape plan; additional landscaping required to produce 3-tier effect; must have 10:1 ratio for shrubs to trees; please provide info on tree height at maturity.

# No dumpsters or litter receptacles shown on plans. Please provide details.

# Lighting details not provided; lighting not shown on elevations.

# No signage indicated.

# No indication of construction schedules or timelines- provide acknowledgement letters from owner and contractor; provide details on laydown yard.

# Provide information on backflow preventer.

# At least one window needed on front elevation at the office

# Confirm meter pan will be painted the same color as exterior wall.

# No roofing details provided.

# Please provide full-sized revised plan sheets to the Planning Department, Building & Safety, and Engineering, as necessary.  For additional information on the City’s Code of Ordinances, please follow the link here:  https://library.municode.com/la/slidell/codes/code_of_ordinances

At completion of the project, a final AS-BUILT site plan and floor plan must be provided to the Planning Department in PDF format and full size copy.

# The pages that follow identify the code requirements for each item of concern.  If you have any questions, please do not hesitate to contact the plan reviewer at (985) 646-4320.

# Thank you.

PLANNING DEPARTMENT REVIEW

# COMMERCIAL CHECKLIST

# (C1, C2, C4, C6)

This checklist is merely a preliminary review of the proposed site plan received by our office.  Please understand that our review is limited and shall not constitute approval development or specific plan layout until all components of this checklist are approved by the Planning Department.  Items that are highlighted are items that need to be corrected.

# FILE NAME:  				___Bouche Heating and A/C______

## APPLICANT:  				___Dammon Engineering__  __

DEVELOPMENT LOCATION:  		___117 Taos St______

DATE:  					___5/29___

REVIEWER:  				___Sydney Keller _____

ZONING DISTRICT:  			___C-4______

COUNCIL DISTRICT:			___E______

FLOOD ZONE/BFE:			___X___ __

PROPOSED USE:  				___Warehouse__ ___

PREVIOUS USE OF PROPERTY:  	___None______

GROSS FLOOR AREA:  			_    2400 sq. ft.________

I.	ZONING CONFORMANCE DETERMINATION:  The proposed use of the property is permitted within its zoning district.  (C1=Z.O. Sec. 2.16A03; C2=Z.O. Sec. 2.1601, C4=Z.O. Sec. 2.1801 & C6=Z.O. Sec. 2.18A01)

COMMENTS:  CONFORMS

II.	SUBDIVISION CONFORMANCE:  Construction across lot lines, except for single-family homes, the subdivision regulations require that no buildings be constructed across lot lines.  (Ord. No. 1679, Subsection 2:219)

COMMENTS:  CONFORMS

III.	CHANGE OF USE DETERMINATION:  When the use of a parcel of property or structure changes, that parcel or structure must comply with all applicable standards established by the Zoning Ordinance.

COMMENTS:  N/A

IV.	NON-CONFORMING USE DETERMINATION:

A.	Structures or uses lawfully permitted by previous Zoning Ordinances, but prohibited by the current ordinance, may not be enlarged or expanded unless the expansion conforms to the provisions of the current ordinance.  (Z.O. Sec. 6.201)

COMMENTS:  N/A

B.	No building or structure or premises where a nonconforming use has ceased for reasons other than those stated (damaged to the extent of not exceeding 50% or its replacement valuation by reason of fire, flood, explosion, earthquake, riot, war, or act of God) for a period of twelve (12) months or has changed to a permitted or conforming use shall again be used as a nonconforming use.  (Z.O. Sec. 6.203-6.204)

COMMENTS:  N/A

V.	REQUIRED YARD AREA SETBACKS:

A.	In addition to the setbacks previously identified, the Zoning Ordinance requires other applicable setbacks.  The applicable zoning classifications establish front, side, and rear yard setbacks.  ([C1=Z.O. Sec. 2.16A05 & 2.16A06]; [C2=Z.O. Sec. 2.1603 & 2.1604]; [C4=Z.O. Sec. 2.1803 & 2.1804]; [C6=Z.O. Sec. 2.18A03 & 2.18A04])

|  | Required | Submitted |
| --- | --- | --- |
|  | C1 C2 C4 C6 |  |
| Front | 50’ 25’ 25’ 25’ | 35’ |
| Side | 5’ 0/3’ 0/3’ 0/3’ | 10’ / 20’ |
| Rear | 0/10’ 0/20% 0/20% 0/10’ | 15’ |
| Height | 2-Story 45’ 45’ 60’ | 19’9” |

COMMENTS:  CONFORMS

VI.	ARCHITECTURAL FEATURES:  Design Guidelines:

A.	Single Use and Mixed Use

1) 	Section 13-303 Façade Treatment- Single use.

a) 	Elevations

i) 	Front Elevation: Stucco

ii) 	Side Elevations: Metal panels/ Stucco

iii) 	Rear Elevation:  Metal panels

See approved exterior materials at Appendix B, Part 3. Sec. 13.304.

COMMENTS:  CANNOT DETERMINE METAL PANEL TYPE ON ELEVATION -MUST BE 26 GAUGE OR BETTER, AND OF A LIKE COLOR TO MATCH OR COMPLIMENT STUCCO.

2) 	§13-305: Building Form and Design (does not apply to buildings <12,000 ft sq; applies to buildings of > 12,000 < 30,000, OR > 30,000 < 60,000, OR > 60,000)

a)	Roof Design:

1.	<30,000 – Moderate Change

2.	<60,000 – Major Change

3.	>60,000 – Major changes with significant architectural features at principal entrance with towers or significant projections.

COMMENTS:  N/A

b)	Architectural Detail – to include details that promote and are in harmony with design vernacular, including but not limited to:

Arcades, Awnings, Balconies, Canopies, Columns, Cornices, Oriel Windows, Porches, Quoining (cornerstone), Recessed Entries, Stringcourse finish, Window Molding.

COMMENTS:  N/A

c)	Multi-story –

1.	Building Base – Thicker walls, ledges, or sills; textured, colored, or patterned materials; raised planters integral to façade

2.	Building Top – Cornice treatments with integrally textured materials; sloping roofs with eaves and brackets.

COMMENTS:  N/A

d)	Building Dimensions –

1.	Max. Horizontal:

>16,000 s.f. – 80’ uninterrupted

<16,000 s.f. – 40’ uninterrupted

2.	Horizontal Offset:

>80’ – clear pronounced projections or recesses at 6’

<80’ – clear pronounced projections of recesses at 2’

Offset measured at least 20’ or 20% of façade, whichever is less.

3.	Entryways >30,000 s.f. project 6’ from façade (change in wall of building, not only covered entry).

COMMENTS:  N/A

e)	Roofing –

1.	Styles – Flat (hidden by parapet) or pitched – no mansard

2.	Materials – Architectural Shingles, Concrete Tile, Slate, Architectural Standing Seam Metal; Building Integrated Photovoltaics (solar panels that double as roofing material), Green Roof Systems (encouraged), or as approved by Director.

3.	Prohibited – Corrugated metal and any not listed.

COMMENTS:  N/A

f)	Elevations – Architectural Features:

1.	Front:

a.	Animate entire length spaced at intervals not more than 12’ in horizontal distance.  Features include:  Sconces, display windows, faux windows, brick patterning, entry areas, towers, pilasters, columns, horizontal and vertical offsets, and/or other elements that create pattern of light and shadow.

b.	Awnings, Arcades, Overhangs (project at least 6’ along 80% of building >12,000 s.f. connecting to entrance)

2.	Side and Rear –

a.	Animate no less than 60% façade nearest front elevation

i.	Awnings, arcades, or overhangs projecting 6’ and connect to feature at front elevation,

ii.	Spaced at 12’ intervals in horizontal distance,

iii.	Include wall sconces, display windows, entry areas, towers, pilasters, columns, horizontal and vertical offsets, and elements that create pattern of light and shadow.

b.	NO BLANK WALL of more than 30’ horizontal or 15’ vertical.

c.	Horizontal and vertical architectural interest through design and use of:  Arcades, Arches, Awnings, Decorative Cornices, Entryways, Patios, Pilasters or Columns, Porticos, Recesses/Projections No Less Than 2’ Horizontal or Vertical Offset in Building Wall, Towers, Windows, Finish Material Changes, Color Changes (used along will not meet intent), Comparable Techniques Creating Pattern of Light and Shadow.

COMMENTS:  N/A

g)	Transparency –

Front:	Glazed and transparent 50% of pedestrian view plane.  (Glazing transparent if 100% transparent from both interior and exterior).  Director’s discretion to required transparency whether inconsistent with operational requirements of the building.

COMMENTS:  N/A

h)	Awnings and Canopies –

1.	Attached and integral to structure

2.	Shall not completely obstruct window

3.	Support by columns, beams, and/or brackets

4.	Made of durable, protective, and water repellant materials – VINYL AND FIBERGLASS AWNINGS ARE PROHIBITED.

5.	Recessed lighting fixtures with lens cover recessed or flush with bottom of canopy; indirect lighting beamed upward then reflected down.  NO BACKLIT OR INTERNAL ILLUMINATED AWNINGS OR CANOPIES.

6.	Cannot be longer than storefront.

7.	Compatible color of façade attached (solid or subtle stripe preferred)

8.	Allowed to project within 2’ of curb line.

9.	Minimum clearance of 8’ from finished grade; drive-under not to exceed 16’ in height.

10.	Freestanding or semi-freestanding (pump islands and porte-cocheres) similar style, material, color and lighting of principal building.

COMMENTS:  N/A

i)	Building entrances clearly defined, highly visible and include three (3) of the following features:

1.	Canopies, porticos, arcades, or overhangs

2.	Recesses or projections

3.	Raised corniced parapets

4.	Over the door or peaked roof forms

5.	Arches

6.	Outdoor patios or plazas

7.	Display Windows

8.	Obvious different architectural details – moldings integrated into building structure and design, and/or

9.	Integral planters or wing walls for landscaped areas and/or places for sitting.

COMMENTS:  N/A

3)  	§13-306: Mechanical Equipment Screening:

a) 	Ground Level Equipment – Landscape (1’ higher than equipment); wall extensions, opaque fences, structural enclosures, each of materials and color that match or are consistent with principal building.

b) 	Roof Mounted:

1)	Parapet walls with cornice treatments of adequate height to fully screen equipment (one (1’) foot rise per 25’ run to determine adequate height);

2)	Screening walls high enough to fully screen of materials and colors to match or are consistent with design of principal building; or

3)	Sloped roof systems or other architectural elements of height to fully screen view from all adjacent properties and rights-of-way.

c) 	Building Mounted – enclosed, screened by opaque fencing and landscaping, or painted to match building façade.

COMMENTS:  CONFORMS

4)	§13-307: Service Bay with Overhead Doors

a)	Provided size, shape and orientation of the site allows, a building wall with overhead doors shall be perpendicular to public or private streets;

b)	If unavoidable, overhead doors that front or most directly face public or private streets shall be painted to match wall façade and shall include windows.

COMMENTS:  NO DETAILS PROVIDED ON OVERHEAD DOOR; MUST MATCH OR COMPLIMENT STUCCO.

B.	All buildings in the Olde Towne Preservation District and Fremaux Avenue Corridor are required to meet the design guidelines and have approval from the review committee.  (Ordinance No. 3486 & Z.O. Sec. 2.2217)

COMMENTS:  N/A

VII.	BUFFER ZONE REVIEW:

A.	Buffer zone required – The Zoning Ordinance requires that a buffer zone be established between differing land uses as follows:  (Z.O. Sec. 2.2209)

|  | LDR | HDR | C | LI | HI |
| --- | --- | --- | --- | --- | --- |
| Low Density Residential (LDR – Single Family Detached Dwelling) |  | X | X | X | X |
| High Density Residential (HDR – Attached Dwellings) |  |  | X | X | X |
| Commercial (C) |  |  |  | X | X |
| Light Industrial (LI) |  |  |  |  | X |
| Heavy Industrial (HI |  |  |  |  |  |
| X – Buffer Zone Required |  |  |  |  |  |

Note:  If subdivided, burden of buffer fence falls on the commercial developer.

COMMENTS:  N/A

B.	Buffer fence – An opaque fence at least six feet in height above the highest point of ground elevation must be located on the property line between the two abutting land uses.  (Z.O. Sec. 2.2210)

1.	The fence must be installed prior to issuance of a building permit.

2.	In situations where fill is placed on a development site, the required fence height shall be computed as follows:

Fence height = 6’ + final grade of site requiring fill – grade of property to be buffered).

COMMENTS:  N/A

C.	Ten-foot buffer area established – A ten foot landscaped area is required between the abutting land uses.  Trees must be located in the ten-foot area with a 20-25 foot on center spacing (maximum).  Each tree must have at least 10 shrubs of varying heights to support buffer requirement.  (Z.O. Sec. 2.2211)

COMMENTS:  N/A

D.	C-6 Regional Shopping Center District:

1.	The maximum buffer zone required between this zoning and any adjacent district is ten (10) feet.  (Z.O. Sec. 2.18A04(1)(b))

COMMENTS:  N/A

2.	The maximum setback required between the parking area of this zoning district and any adjacent district is ten (10) feet.  (Z.O. Sec. 2.18A04(1)(c))

COMMENTS:  N/A

VIII.	FENCES (Non-Buffer):  In all commercially zoned districts, fences shall be constructed of wood, iron, vinyl, aluminum, brick, or chain link.  Barbed wire, razor-ribbon, corrugated sheet metal, may not be used.  Chain link may not be used between the building or building setback and a street right-of-way. (Z.O. Sec. 2.2203)  Please state the building material for any fence being added that is not a required buffer fence.

COMMENTS:  CONFORMS

IX.	LAND CLEARING REVIEW:  In order to preserve existing trees on development sites, the Zoning Ordinance requires that a land clearing permit be approved prior to receiving a building permit.  Canopies of protected trees that overhang the property lines of the lot to be developed must be brought to our attention.  The City will give consideration to the following site conditions in its review of land clearing permit applications. (Z.O. Sec. 2.2502)

(1) 	Necessity to remove trees which pose a safety hazard to pedestrian or vehicular traffic or threaten to cause disruption of public services.

(2) 	Necessity to remove trees which pose a safety hazard to buildings.

(3) 	Necessity to remove trees weakened by disease or insects.

(4) 	Necessity to remove trees weakened by age, storm, fire or other injury.

(5) 	Necessity to remove trees in order to construct proposed improvements as a result of need for access to the building site for construction equipment, essential grade changes, surface water drainage and utility installations, or location of the proposed structure so as to avoid unreasonable economic hardships.

(6) 	Necessity to remove trees to observe good tree maintenance practices which will strengthen and protect existing trees as is determined necessary by the department of planning.

COMMENTS:  N/A

XI.	LANDSCAPE REQUIREMENTS REVIEW:  The Zoning Ordinance establishes landscape requirements for commercial developments.

NOTE:  LOUISIANA STATE LAW REQUIRES A LANDSCAPE PLAN MUST BE STAMPED BY A LICENSED LOUISIANA ARCHITECT OR A LICENSED LOUISIANA LANDSCAPE CONTRACTOR.  (Z.O. Sec. 2.2502, La. R. S. 3:3808 (E) & (F), and L.A.C. 7:XXIX.117 (B) & (F))

A.	Perimeter Planting Area – A perimeter planting area ten (10) feet in width must be provided off the public rights-of-way between the rights-of-way and parking area or structures.  Planting materials should achieve a balance between low-lying vertical and horizontal shrubbery and trees (3-tier effect). (Z.O. Sec. 2.2513)  Trees shall be placed 20-25 foot on center spacing with a compliment of no less than 10 shrubs per tree (National Arbor Society – good forestry practices).

COMMENTS:  MORE SHRUBBERY NEEDED ON LANDSCAPE PLANS; MUST HAVE TREE: SHRUB RATIO OF 1:10

B.	Parking Lot Planting – Parking lot interiors should be designed to provide at least one (1) tree for every twelve (12) parking spaces.  Each tree should be located in a landscaped island of at least two hundred (200) square feet and tree trunks shall be placed at a minimum of five (5) feet from all curbs or wheel guards.  Trees must be distributed uniformly throughout the parking lot so as to provide a canopy effect.  (Z.O. Sec. 2.2514)

COMMENTS:  N/A

C.	Pedestrian Access Planting – Landscaped areas must be provided between the building faces having access and the parking lot.  Such areas should provide 200 sq. ft. of landscape open space for each 50’ of linear building face.  No planting area should be less than five (5) feet wide and planting materials should achieve a balance between low-lying and vertical shrubbery and trees (3-tier effect). (Z.O. Sec. 2.2515)

COMMENTS:  CONFORMS

D.	Tree Height – Trees selected shall have a minimum maturity height of at least 	twenty (20) feet.  (Z.O. Sec. 2.2516)

COMMENTS:  UNKNOWN; PLEASE PROVIDE DETAILS ON MATURE TREE HEIGHT.

E.	Camellia species requirement - Camellia Bush Requirement. Installation of minimum of three camellia bushes for the first five acres and minimum of two camellia bushes for each acre over five acres. Any genus of the Camellia is acceptable. The camellia bush can be planted in the perimeter, parking, or pedestrian planting zones, but shall be planted in a prominent location visible from the public right-of-way or near a main entrance. (Z.O. Sec. 2.2519)

COMMENTS:  NO CAMELLIAS LISTED ON LANDSCAPE PLAN; MINIMUM OF 3 REQUIRED.

F.	Required Inspection – The landscaping will be inspected and approved prior to 	the issuance of a certificate of occupancy.  (Z.O. Sec. 2.2517)

NOTE:  Please schedule a final inspection once all landscaping is complete with the Planning Department.

XII.	PARKING AND ACCESS REVIEW:

A.	Demand Computation – The applicable demand computation for indicated use is calculated as follows:  (Z.O. Sec. 4.201-4.225)

B.	Access Analysis.  NOTE:  You must use one of the following qualified traffic study firms for a letter or a study:  Urban Systems, Inc. (N.O. 504-523-5511); Burk-Kleinpeter, Inc. (N.O. 504-486-5901); Neel-Schaffer, Inc. (B.R. 225-924-0235); Evans-Graves Engineering (Met. 504-836-8190); Sain Associates, Inc. (Birm., AL 205-940-6420); Krebs, LaSalle, LeMiex Consultants, Inc. (Met. 504-837-9470); Kelly J. McHugh & Associates, Inc. (Mandeville 985-626-5611); Richard C. Lambert Consultants (Mandeville 985-727-4440); Dean Tekell Consulting (Laf., LA 3370988-5211); or J.V. Burkes & Associates, Inc. (Slidell 985-649-0075):

COMMENTS:  CONFORMS

C.	Off-Street Loading Requirements:

1.  	Every hospital, institution, hotel, commercial or industrial building or use having a gross floor area in excess of 7,500 square feet of space and requiring the receipt of distribution by vehicle of material and merchandise shall have at least one permanently maintained off-street loading space 10 feet X 45 feet for each 7,500 square feet of gross floor area or fraction thereof and so located as not to hinder the free movement of pedestrian and vehicles over sidewalks, streets, and alleys.  (Z.O. Sec. 4.301)

COMMENTS:  N/A

2.	Retail operations, wholesale operations, and industrial operations with a gross floor area of less than 7,500 square feet shall provide sufficient off-street loading space (not necessarily full berth if shared by an adjacent establishment) so as not to hinder the free movement of pedestrians and vehicles over a sidewalk, street, or alley.  (Z.O. Sec. 4.302)

3.	Loading zone requirements in the C-1 district are the same as the C-2 zoning district (Z.O. Sec. 2.1606).  If a lot or parcel is contiguous to a residentially zoned parcel, no loading areas shall be constructed within twenty-five (25) feet of the rear or side lot line which is contiguous to the residential property.  (Z.O. Sec. 2.16A09)

COMMENTS:  CONFORMS

4.	Loading zone requirements in the C-6 district are as provided for under Part 4 (see 1. and 2. above) shall apply except that only (1) off-street loading space ten (10) feet by forty-five (45) feet is required for each one hundred thousand (100,000) square feet of building area.  (Z.O. Sec. 2.18A06)

COMMENTS:  N/A

XIII.	PEDESTRIAN ACCESS:  Sidewalks shall be provided within the public right-of-way adjacent to new residential and commercial development as follows (COO App. B, Sec. 4.2 & COO App. B, Part 4-Sec. 4.501 as revised).  (See COO App. B, Division II, Sec. 3. – Concrete Driveways and Sidewalks, Sub-Sec. 5.00 for minimum thickness.):

Sidewalks generally located one (1’) foot from property line within the right-of-way.  When location would cause the loss of specimen trees, the sidewalk should be designed to maintain as many trees as possible.

Sidewalks are not required in residential subdivisions where the majority of the lots have previously been built without public sidewalks, and where there are no public sidewalks within 200 feet of a lot or lots being improved on the same side of the street.

COMMENTS:  CONFORMS

XIV.	REQUIRED YARD AREA SETBACKS:  In addition to the setbacks previously identified, the Zoning Ordinance requires other applicable setbacks.

A.	Commercial Development – Whenever a commercial development abuts the rear of a residential development, the rear yard setback for the residential zoning district applies as the rear yard setback for the commercial property.  (Z.O. Sec. 2.1604(b) & 2.1804(b))  For C-6 district, the maximum setback required between parking area and any adjacent district is ten feet. (Z.O. Sec. 2.18A04(1)(c))

COMMENTS:  N/A

B.	Accessory Uses – The Zoning Ordinance regulates the location of accessory buildings and uses.  (Z.O. Sec. 10.4).

COMMENTS:  N/A

XV.	LOCATION OF DUMPSTERS:

A.  	Section 23-13 of the City of Slidell Solid Waste Ordinance requires every commercial site to provide containers capable of handling a week’s accumulation of trash in a sanitary manner.  The location of commercial dumpsters shall be located on the site plan and should be incorporated into the design for ease of access.  The dumpster shall be screened with a solid wood fence and gate or other suitable materials approved by the Planning Department. (COO Sec. 23-17)

COMMENTS:  NO DUMPSTERS SHOWN ON SITE

B.	Section 23-16 of the City of Slidell Solid Waste Ordinance states that the location of the construction dumpsters shall not be located within 10 feet of an adjacent dwelling, blocks the sidewalk, or placed on any public street or public alley.  (COO Sec. 23-16)

COMMENTS:  N/A

XVI.	LOCATION OF LITTER RECEPTACLES:  The receptacles shall be located on the site plan and should be incorporated into the design for ease of access.  A “litter receptacle” means a container of not less than fifteen gallons constructed, appropriately marked, and placed for use as a temporary depository for litter.

A.	The state’s litter law requires every property which is held out to the public as a place for parking consisting of fifteen or more parking spaces shall be responsible for the procurement, placement, and maintenance of litter receptacles. (LA RS 30: §2535(A))  Please provide one trash receptacle.

B.	Litter receptacles shall be required at any parking lot consisting of thirty or more parking spaces operated for public use.  (LA RS 30: §2535(C))  Total number of receptacles required to be determined by the Planning Department considering the total number of parking stalls provided.

COMMENTS:  NO LITTER RECEPTACLES SHOWN ON PLANS

XVII.	LOCATION OF FIRE HYDRANTS:  Location of fire hydrants shall be indicated on the site plan.  Water mains shall be designed of sufficient size to provide at least one (1) fire hydrant to within three hundred (300) feet of all points of a structure per “hose lay”, within a commercial and industrial area and comply with all rules and standards of the state rating bureau. (Subdivision Regulations: Sec. 4.702 (2))   Structures requiring sprinkler systems shall have a hydrant "dedicated" to the sprinkler system. An additional hydrant shall be required. The hydrants shall not be fed by the same service line/main, when possible. Location and connection of the hydrant units shall be subject to the approval of the fire prevention bureau, the building official, and city engineer. (Subdivision Regulations: Sec. 4.702 (3))

NOTE:  The Planning Department no longer reviews the location of fire hydrants.  Please check with the Engineering Department and Fire Prevention for approval.

XVIII.	SIGNS:  All types of signage require permits.  Experience has indicated that in all too many cases, signage is an afterthought.  Signage should be part of the design process and considered an integral part of the site planning process.  A sign review and approval will be made by the Planning Department before a permit is issued.  Submit the sign packet as a separate packet for the types of signage listed below for review and approval. Reminder: Every sign will require a building permit.  (Z.O. Sec. 503 & 504)

Freestanding Signs	Permit required ($30).

Fascia Signs (Building or Wall Signs)	Permit required for max. 2 per qualifying façades ($15/each).

Directional Signs	Directional signs to be reviewed with building permit submittal.  PLEASE PROVIDE DETAIL OF ANY ONSITE DIRECTIONAL SIGNAGE.

Interstate Signs (if applicable)	Contact State DOTD

Banners (Temporary Signs)	Permit required ($10).  Max. 90 days per year.  Max. 2 Banners per event.

COMMENTS:  NO SIGNAGE INDICATED

XIX.	EXTERIOR LIGHTING:  Section 2.19(J) in the Zoning Ordinance regulates glare.

- Commercial and Industrial Light Standards – No use in any district shall be operated so as to produce direct sky-reflected glare, or direct illumination, across the adjacent property line from a visible source of illumination of such intensity as to create a nuisance, or traffic hazard, or detract from the use or employment of adjacent property.  Residential lighting fixtures shall be directed or shaded to prohibit the intensity of light to exceed one-half foot-candle as measured at any adjacent residential property line.  Commercial lighting fixtures shall be directed and shaded to prohibit the intensity of light to exceed one (1) foot-candle as measured at any adjacent property line.  (Z.O. Sec. 2.19 (J(4)))

Please submit exterior lighting details that show number of foot-candles measured at property lines, and/or submit a letter stating that Section 2.19(J) of the City of Slidell Zoning Ordinance will not be violated.

COMMENTS:  UNKNOWN; NO LIGHTING SHOWN ON ELEVATIONS

B.	Hours of Operation – Lighting of parking or loading areas shall, except for emergency or safety lighting, cease at or before the hour of midnight when these areas face a residential district where the distance from the nearest light to the nearest residence is less than one thousand (1,000) feet, except in those instances in which a business establishment is open and business is being conducted after midnight.  (Z.O. Sec. 2.19 (J(4(a)))

COMMENTS:  N/A

C.	Blinking or Intermittent Lights – No exterior lights that blink or shine with an intermittent phase are permitted in any district, except as part of Christmas decorations.  (Z.O. Sec. 2.19 (J(5))  This includes signs.

COMMENTS:  N/A

XX.	CONSTRUCTION LAYDOWN AND TIME RESTRICTIONS:

A.	Construction Laydown Plan:  Provide plans showing

  - Location and composition of Construction entrance

  - Location of Construction Dumpster

  - Equipment and Material Staging Areas

  - Port-a-Johns

B.	Time Restrictions.

  - Construction is limited to Monday to Saturday between Beginning Morning and End Evening Civil Twilight. Contact Planning Division for Civil Twilight Times for City of Slidell.

  - Construction outside of above hours is approved on case by case basis by City Chief of Staff.  Submit request to Planning Division with Date, Times, Activities, Reason, and whether neighbors have been contacted.  Planning will submit to Chief of Staff and notify applicant of approval and if Chief of Staff has conditions that must be met.

C.	Blocking of Streets or Public ROW.  If you require to block/use Public ROW for offloading or other construction activities you must provide date, time, and temporary traffic control measures (cones, flagmen, signs) to Planning for approval for City Streets.  For State Highways you must get approval from LADOTD District 62.

COMMENTS:  NO INDICATION FOR CONSTRUCTION SCHEDULES AND TIME RESTRICTIONS- PROVIDE ACKNOWLEDGEMENT LETTERS FROM OWNER AND CONTRACTOR; PROVIDE INORMATION ON LAYDOWN YARD.