Factory fabricated Metal Specification

For:

Slidell Junior High School

Paraguard
Roof Perimeter Systems

Prepared by:

Siplast

1000 East Rochelle Blvd.

Irving, Texas 75062

(800) 922-8800

This specification is provided as a general guide for use of Siplast factory fabricated metal roof perimeter systems.  Siplast is strictly a manufacturer and supplier of roofing and waterproofing materials and factory fabricated metal accessory systems.  Siplast has no experience, training or expertise in the areas of architecture or engineering or in the area of consulting with respect to matters related to such areas.  We recommend that the Owner's representative independently verify the accuracy and appropriateness of a specification provided for a specific project.

January 29, 2014
SECTION 07710 MANUFACTURED ROOF SPECIALTIES (03/2010)
PART 1  GENERAL

1.01  SECTION INCLUDES:

A.
Preparation of surfaces to receive factory fabricated metal perimeter systems.
B.
Installation of factory fabricated and finished metal perimeter systems.
1.02  RELATED SECTIONS

A.
Section [----] - Rough Carpentry

B.
Section 07550 - Modified Bitumen Membrane Roofing

1.03  REFERENCE STANDARDS

NRCA
National Roofing Contractors Association


Rosemont, IL

OSHA
Occupational Safety and Health Administration


Washington, DC

SMACNA
Sheet Metal and Air Conditioning Contractors National Association


Chantilly, VA

FM
Factory Mutual Engineering and Research


Norwood, MA

ANSI
American National Standards Institute

Washington, DC

SPRI
Single Ply Roofing Industry


Waltham, MA

1.04  SUBMITTALS

A.
Submittals Prior to Contract Award:

1.
Submit a letter from the roofing membrane manufacturer confirming that the factory fabricated metal accessory systems furnished for the project are supplied or manufactured by the roofing membrane manufacturer and that each component section is embossed with the roofing membrane manufacturer’s logo.
2.
Latest edition of factory fabricated metal component manufacturer/supplier’s installer’s guide for factory fabricated metal perimeter systems.

3.
Samples from the manufacturer/supplier sized to represent metal components.

4.
Copies of the manufacturer/supplier's color selection chart showing the manufacturer/supplier's full range of standard colors as well as physical samples of each standard color.

5.
Sample copy of the roofing system manufacturer's inclusion addendum offering coverage of the factory fabricated metal perimeter systems.

1.05  QUALITY ASSURANCE

A.
Agency Approvals:  The proposed factory fabricated metal component shall conform to the following requirements.  No other testing agency approvals will be accepted.

1. The roof perimeter fascia systems shall be certified through third party verification by the manufacturer/supplier to meet performance design criteria according to the most recent edition of ANSI/SPRI/FM 4435/ES-1: Wind Design Standard for Edge Systems Used with Low Slope Roofing Systems.

2. SPRI Test Method RE-2 maximum 10-inch wide face, 0.050 aluminum fascia cover, 22 guage G-90 galvanized steel cleat with a rating of 159 psf. 

B.
Scope of Work:  The work to be performed under this specification shall include but is not limited to the following:  Attend necessary job meetings and furnish competent and full time supervision, experienced mechanics, all materials, tools, and equipment necessary to complete, in an acceptable manner, the factory fabricated metal installation in accordance with this specification.  Comply with the latest written application instructions of the manufacturer/supplier of the factory fabricated metal components. 

C.
Local Regulations:  Conform to regulations of public agencies, including any specific requirements of the city and/or state of jurisdiction.

D. Manufacturer Requirements:

1.
Ensure that the factory fabricated metal components are embossed with the roofing membrane manufacturer’s logo.

2.
Ensure that the factory fabricated metal component manufacturer/supplier provides direct trained company personnel to attend necessary job meetings, perform periodic inspections as necessary, and conducts a final inspection upon successful completion of the project.

1.06  PRODUCT DELIVERY STORAGE AND HANDLING

A.
Delivery:  Deliver materials in the manufacturer's original packaging. 

B.
Storage:  Store materials out of direct exposure to the elements.

C.
Strippable Film Masking:  Do not remove the strippable film masking on the metal component until immediately following installation.  Do not allow extended UV or heat exposure to metal components covered with strippable film masking.
D.
Damaged Material:  Any materials that are found to be damaged will be automatically rejected, removed and replaced at the Contractor's expense.

1.07  PROJECT/SITE CONDITIONS

A.
Requirements Prior to Job Start

1.
Related Work:  Verify that all related work performed by other trades is complete prior to installing the factory fabricated metal components.

2.
Component Substrate Condition:  Mounting surfaces shall be straight and secure and provide adequate widths to properly support the factory fabricated metal components.

3.
Safety:  Familiarize every member of the application crew with all safety regulations recommended by OSHA, SMACNA and other industry or local governmental groups.

B.
Protection Requirements

1.
Component Protection:  Protect newly applied factory fabricated metal component surfaces against mechanical damage.

2.
Limited Access:  Prevent access by the public to materials, tools and equipment during the course of the project.

3.
Debris Removal:  Remove all debris daily from the project site.

4.
Site Condition:  Complete, to the owner's satisfaction, all job site clean-up including building interior, exterior and landscaping where affected by construction.

1.08  GUARANTEE/ ADDENDUM

A.
Roof Membrane Guarantee Addendum:  In addition to the specified guarantee under section 07550, furnish the Owner with the roofing manufacturer's inclusion addendum to the guarantee offering coverage of the factory fabricated gravel stop and coping systems under the standard terms of the roofing membrane/system guarantee.

>
Siplast Paraguard Roof Perimeter System Inclusion Addendum

PART 2  PRODUCTS

2.01  DESCRIPTION OF FACTORY FABRICATED METAL SYSTEMS
A.
Factory fabricated Gravel Stop:  Factory fabricated gravel stop components shall be factory formed according to the requirements of the membrane manufacturer and embossed with the roofing manufacturer’s logo.  The gravel stop system shall consist of the following components:

-
A factory formed retainer cleat with pre-punched nail holes fabricated from 22 gauge, G90 galvanized steel, secured using galvanized roofing nails.

· A factory formed gravel stop with pre-punched nailing holes, secured using galvanized roofing nails. fabricated from minimum 0.050 inch aluminum having a spray coated Kynar™ finish of SP-19 Turquoise.
-
Factory formed concealed splice plates.

-
Factory formed welded miters.

>
Proform Gravel Stop, by Siplast, Inc., Irving, TX (800) 922-8800

PART 3 EXECUTION

3.01  SUBSTRATE PREPARATION
A.
Perimeter Nailers:  Perimeter nailers shall be flat and level to the building perimeter edge.  The front edge of the nailer must be flush with the outside face or wall of the building.  Anchor all perimeter nailers in strict accordance with the guidelines set forth in FM Global Property Loss Prevention Data Sheet 1-49.

B.
Curbs for Expansion Joint Components:  Curbs must be straight, level, and properly anchored to the building structural deck.  Any curbs, which are improperly installed or anchored, must be corrected prior to installation of the expansion joint systems.
C.
Flashing Membrane Installation:  Ensure that all roofing flashing treatments used in conjunction with factory fabricated metal components are installed according to the roofing membrane manufacturer's specifications, current technical guide, and details prior to installation of the factory fabricated metal component.
D.
Surface Cleaning:  Sweep or vacuum all surfaces to receive the metal components, removing all loose aggregate, soil, and foreign substances prior to installation of the factory fabricated metal components.

3.02  FACTORY FABRICATED METAL COMPONENT INSTALLATION 

A.
Install metal components in accordance with the roofing/waterproofing manufacturer's instructions and the following requirements.
B.
Factory fabricated Gravel Stop
1.
Place the continuous retainer cleat to the roofing surface firmly against the perimeter nailer.  The retainer cleat should be level and the nailing slots should align centered with the nailer underneath.  Fasten the retaining cleat in accordance with the gravel stop system manufacturer's installation instructions.

2.
Starting at the corners, trowel a bead of the roofing manufacturer's specified mastic over the base ply of membrane where the flange of the exterior fascia is to be set.  Hook the drip edge of the exterior fascia over the retainer cleat and fasten the flange through the pre-punched holes in accordance with the gravel stop system manufacturer's installation instructions.  Slide a concealed joint splice plate halfway into the fascia to allow the next section to fit halfway over the joint splice plate as well.  Allow a 1/8 inch gap between gravel stop sections for thermal movement.  Increase the gap to 1/4 inch when installing in temperature below 40(F.
3.
After installation of the factory fabricated gravel stop is complete, ensure that the roofing stripping and finish plies are installed in accordance with the roofing membrane manufacturer's specifications and details.

3.03  FIELD QUALITY CONTROL AND INSPECTIONS

A.
Site Condition:  Leave all areas around the job site free of debris, construction materials, equipment and related items after completion of job.

B.
Issuance Of The Addendum to the Roofing Membrane/System Guarantee:  Complete all post installation procedures and meet the factory fabricated metal manufacturer/supplier's final endorsement for issuance of the addendum to the specified roofing/waterproofing guarantee.

PG-28-50
07710 - 4
Manufactured Roof Specialties


