April 17, 2001

To:
Invited Contractors:

1. McDonald Construction

2. Miramon Construction

3. McMath Construction

RE:
Construct Food Store at Barkers Corner, Lee Road, north of Covington.

Gentlemen:

Mr. Sarbjit Singh, owner of the discount food store located at Barkers Corner, intends to demolish the existing food store, gas pumps and tanks, and an existing small residence on the site, and construct a new food store with additional lease space on the site.  The lease space (restaurant) will not be completed at this time, but may be negotiated with the successful low bidder, if a lease is negotiated prior to completion of the food store.  It is also possible that another type of business will be located in the portion presently designated as a restaurant.

With reference to the drawings, please delete/add the following work:

1. The concrete driveway will not be extended toward the rear of the building further than the front end of the building on the south side.  The concrete driveway on the north side will extend to the rear and then south to a point ten (10) feet beyond the rear door to the grocery warehouse/storage area.

2. Delete the gas pumps, gas island and all reference to gasoline sales from the new facility.

3. The dividing wall that separates the restaurant section from the store shall be completely closed off with framing and one layer of 5/8” gypsum board to the underside of the roof, on the grocery side.  The interior of the restaurant area will be framed only as required for the overall building, with all underground and venting of plumbing completed, including grease trap.  No permanent electrical services will be installed to the lease area, but two 4-lamp F40  T12 fluorescent fixtures, with switch and circuit and two 20amp receptacles will be installed in the area.  The exterior of the building will be completed according to the drawings, and the interior of the restaurant windows will be covered with brown paper.

4. The electrical service will be resized to a 400amp main fuse box, a 400amp three-phase panel P1, 42 pole, with short circuit capacity to exceed the local electric short circuit demand at that point.  Lighting panel #1 and power panel #2 will be installed.

5. Provide in your bid for removal of the gasoline sales equipment, tanks, etc.  Any remediation beyond normal testing will be an extra.

6. Supply and install a new chlorinated sewerage treatment facility to meet the requirements of the health department.  The treatment plant will service both the store and lease area.

7. The metal building shall be supplied and erected by Bayou Steel Corporation.  The building will be of the tubular steel type of construction, with R panel roofing.  Roof insulation will be 3” thick vinyl covered fiberglass.

Bids will be received by Dammon Engineering until Wednesday, May 9th, at which time the owner will review all bids and make his selection.  Please provide your bid in a closed envelope, marked with the job name and addressed to Mr. Singh, c/o Dammon Engineering.

The contractor will be expected to complete the work within 120 days, with a $50.00 a day liquidated damages fee after that.  The contractor shall provide owner with proof of insurance for Workmen’s Compensation, autos, general liability and builder’s risk.

Any changes to this request for proposals shall be by written addendum only.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

cc:   Mr. Sarbjit Singh

EGD:clh

