December 10, 2007

Mr. Stiller

Melida, Inc.

3684 Riviera Dr.

Slidell, LA  70458

RE:
Zito Residence

Dear Mr. Stiller,

Please note that the following is included as part of the piling plan for the above residence.

PILING SPECIFICATION:
1. PILES ARE TO BE CLASS 5 MODIFIED, 30’ PILES.
2. DESIGN LOAD – 5 TONS PER PILE.

3. NO FIELD SUPERVISION OR INSPECTION PROVIDED UNDER THIS SEAL UNLESS NOTED OTHERWISE.

4. PILE LAYOUT MAY BE MODIFIED DUE TO ACTUAL DRIVING CONDITIONS, ENGINEER TO BE NOTIFIED ON ANY MODIFICATION.

5. THIS PILE SUPPORTED FOUNDATION IS DESIGNED TO MEET THE GENERAL SOIL CONDITIONS OF THE AREA OF WORK.  THE CONTRACTOR OR OWNER IS ADVISED THAT A SOILS ANALYSIS SHOULD BE MADE, TO CONFIRM THE DESIGN.

6. A PILE BLOW COUNT LOG OF ALL PILES IS TO BE SUMITTED TO THE ENGINEER OF RECORD, FAILURE TO SUBMIT SAID LOG WILL RELEASE THE ENGINEER OF ALL RESPONSIBILITY.

7. CONTRACTOR IS RESPONSIBLE FOR THE COMPARISON AND VERIFICATION OF PILE LAYOUT DIMENSIONS WITH MOST RECENT ARCHITECTURAL DRAWINGS, ASSURING THAT PILES DO FALL WITHIN LIMITS OF THE DESIGN.

Sincerely,
Chuck Dammon

