Description:
The “SEA SPRAY” is a 47 foot length over all, 37 foot stem to rudder post, manufactured by Egg Harbor Yachts of all wood constructed with a all fiberglass coated hull, forward deck and cabin, having an electric anchor winch on the boy, and four water tight port lights on the port and starboard sides of the hull.  There is a forepeak rope locker for the anchor.  Aft of the rope locker is the forward cabin with two bunks one on the port and one on the starboard with stowage under each bunk.  Aft of the bunk on the port side is a stowage locker water closet and shower.  On the starboard side is a water closer and a double bunk cabin.  Continuing aft and up four steps to the main salon cabin having a galley with three burner stove/oven, microwave, refrigerator double sink, sofa sleeper and dining table, there are windows all around the cabin above the main deck covered with screens.  The engine compartment is under the main salon cabin,  Aft of the salon cabin is the aft deck with the stowage below the deck.  Above the main salon cabin is the flying bridge with steering, engine controls and communications, entered by a vertical ladder.

The hull is all wood constructed, covered with fiberglass, having a Raked Vee bow, molded sides and vertical transom.

The Engines are located to the port and starboard of centerline with the generator located on the centerline aft of the main engines.  The engines are General Motors V8-71 diesels, with Racor filters, rated at 350 hp each.  The Generator is an Onan three cyl. 9 kw diesel with Racor filter and battery start.  (Note the generator set was changed out and is in new conditions with 35 hours of use.)

The on board power is supplied through the generator and battery back up.  The system is circuit breaker protected.  There is a ashore power panel with converted and battery charger system.

Dimensions:

Length at deck:  47 ft.

Breadth: 13 ft. 02 in.

Depth: 8 ft.

Draft: 3 ft.

Height of eye at steer controls: 14 ft.

Keels Laid:  1965

Built: 1965

Hull:

Each hull is subdivide into three (3) watertight compartments, one forward of the engine room and one after of the engine room separated by watertight bulkheads.

All through hull fittings have been checked and reworked as needed.  Both shafts and bearings are new and the props have been reworked.  The shafts are 1-3/4 inch diameter and the props are 30” X 27”

The hull had been cleaned and was being coated at eth time of the survey while hauled.

The owner reported that the aluminum Fly bridge cover and canvas cover were recently installed.

Electronics:

One(1) Com ICM 402 WHF marine radio with remote

One (1) Standard Horizon digital depth finder

One (1) Ray Marine Radar/chart plotter

One (1) GPS Mod 12

One (1) VHF hand held

One (1) Raytheon VHS radio

One (1) 12V D/C charger system and inverter

Fire Extinguishers:

One (1) 5 pound CO2

One (1) 2-1/2 pound CO2

Two (2) B-I dry chemical

One (1) B-II dry chemical

Life Saving Equipment:

All personal flotation devices are in good condition.

Three (3) Flair signal kit.

Two (2) Throw able devices. 24 inch

One (1) type I PFD to fit each person on board

Two (2) CO2 detectors

Electrical Supply:

Shore power is supplied by 50-amp cable.  The 12 V D/C power supply is powered by marine batteries, for starting and for 12 V D/C on board power.  The generator supplied onboard power. All electrical services are circuit breaker protected.

Misc. Equipment
One (1) water pressure pump

One (1) 6 gal. hot water heater

Two (2) engine compartment vent blowers

Two (2) marine heads holding tank

One (1) 4 Ritchie inch magnetic compass

One (1) electric air horn

One (1) 6 inch bell

Six (6) bilge pumps (main and backup in each compartment)

Complete set of navigation lights

Complete set of deck cleats and mooring fixtures

One (1) approx. 35 pound Danforth type anchor with stock chain

One (1) approx. 3.5 cuft. Freezer

Two (2) window type air conditioning units

Condition:

A. Deck and hull coating are approximately 99.9% in tact.

B. Stern transom is in very good condition with no indications of structural damage.

C. Sides and bows are in very good condition with no indications of structural damage.

D. The bottom externals were sighted at this survey and found to be in very good condition.

E. The internals and bilges are clean and well kept.

F. Hand rails, steps and ladders all in good condition.

G. The through hull fittings are all in good condition.

H. All hoses and piping to the through hull fitting are in good condition.

I. There is no indication of damage or leaking in the hull.

Tankage

Fuel 700 gallons

Water 200 gallons

