# CRANE WHEEL LOAD DATA

W H E E L    L O A D S    A R E    B A S E D    O N    P R O P O S E D    G I R D E R

( A N D    S E R V I C E    P L A T F O R M )    W E I G H T

## 1	Wheel load drawing

## 2	Crane information

| Crane type | QXSk10-TON I+II x 44ft Hol:20.42ft |  | Buffer type | D2240 |
| --- | --- | --- | --- | --- |
| Span (Spa) | 44'-0" |  | Wheel base (Wb) | 6'-6 3/4" |
| Load (SWL) | 20 000 lbs |  | Crane rail in calculation | ASCE40 |
| Crane group | CMAA C |  | Wheel groove | 2 1/2" |
| Crane speed | 100 ft/min |  |  |  |
| Crane weight | 9 690 lbs |  | Crane travel limit switch | 2-step |

## 3	Hoist Information

| Hoist | Hoist type | Hoist group | Hoisting speed |
| --- | --- | --- | --- |

| Hoist 1 + 2 Main | SX40410050P | ASME H4 | 19/3.3 ft/min |
| --- | --- | --- | --- |
| Hoist 1 + 2 Aux |  |  |  |

## 4	Vertical wheel loads

| Wheel | NR1 | NR2 | NR3 | NR4 |
| --- | --- | --- | --- | --- |
| Rmax Stc | 12.5 kips | 11.5 kips | - | - |
| Rmin Stc | - | - | 3.0 kips | 2.7 kips |
| Rmax Dyn | 14.2 kips | 13.2 kips | - | - |
| Rmin Dyn | - | - | 3.3 kips | 3.0 kips |

## 5	Horizontal wheel loads (according to DIN 4132 + 15018 and FEM)

| 5.1 Inertia forces (from driving mechanisms) | HM1 = 0.4 kips |  | HM2 = 1.9 kips |
| --- | --- | --- | --- |
| 5.2 Wheel loads along crane runway |  |  | 0.9 kips |
| 5.3 Buffer force for dimensioning the crane runway end stop |  |  | Bf = 6 kips |
| 5.4 Forces coming from skewing |  |  |  |
| 5.4.1 Guiding (contact) force (S= HS2 + HS4) |  |  | S = 4.4 kips |
| 5.4.2 Friction forces due to oblique travel | HS2 = 3.6 kips |  | HS4 = 0.9 kips |