554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

Fax: 985-641-5950

dammonengineering.com

info@dammonengineering.com

March 7, 2019

Merlin DeCorte, PLA

Specialty Pools & Outdoor Living, LLC

1701 Belle Chasse Hwy.

Gretna, Louisiana 70056

Office: (504) 596 - 6343

Cell: (504) 874 - 2198

RE: Stone Bridge Manor Pool

We have reviewed the piling capacity test report from Stratum Engineering, dated Feb 25, 2019.  No additional piles will be required.

Some adjustments to the plan have been made:

- The maximum pool depth has been revised from 6ft depth to 5 ft maximum depth.

- The pool sidewall, near the piling that was tested, must be constructed directly over the piles as shown on detail #3 of the attached drawing   An additional #4 rebar has been added along this same sidewall to be placed directly on top of each pile for the entire length of the sidewall.

- An additional #4 rebar has been added in the middle of the pool floor between the two piles that have the greatest distance between them.  See attached.

- All of the pool sidewalls have been revised to show a single mat of re-bars, when the backfill on the sides of the pool is filled with River Sand for a thickness of 2ft.

- A general note has been added that all piles shall have a minimum of one #4 bars directly on top of the pile in each direction connecting with another pile.

If you have any questions please contact me.

Thank You,

David Dammon

President

Dammon Engineering, Inc.

(985) 649-5832

Bill Kingsmill’s contact information is as follows:

William Kingsmill (kingsmillprop@bellsouth.net)

Cell: (504) 528 3838

1200 Business Highway 190, Suite 13

Covington, LA 70433

Lawrence Wm. Haik, Sr

Republic Commercial Properties, LLC

1539 Royal Palm Dr.

Slidell, LA 70458

(985) 960-7033