April 14, 2010

Mr. Mark Davies

British American Design Group

29124 Grand Ave.

Lacombe, LA  70445

RE:	Hallway Floor Trusses

LaQuinta Inn/Hammond, Louisiana

Dear Mark,

I was the engineer who approved the trusses for use in the halls when they were submitted for approval and I have viewed the photos of the holes drilled in the floor trusses for the electrical work.

Since those trusses only support the hallway floor dead and live loads, with no walls bearing on the trusses, since the closest that an edge of the hole is to the end of the truss is six inches, and since the trusses are 16” o.c., I have determined that their safety factor and ability to support the dead and live loads of hallway traffic will not be compromised.

Sincerely,

Emmett G. Dammon, P.E.

La. License no. 8796