Estimated loads from :

Dead Loads

Roofing

Asphalt Shingles								4 psf

Plywood 5/8”									2.5 psf

Wood Framing 2x6 24” O.C.							5 psf

Total Roofing = 11.5 x 1.6 = 19 psf

Ceilings

Gypsum Board 5/8” 								2.75 psf

Mechanical Duct								4 psf

Insulation 6”									1.8

Wood Ceiling Joist 2x8 @ 16” O.C.						7 psf

Total Ceiling = 15.55 x 1.6 = 25 psf

Walls

Exterior 2x6 @ 16” O.C. with gypsum and wood exterior	12 psf x 1.6 =  	20 psf

Interior 2x4 @ 16” O.C. with gypsum each side			11 psf x 1.6 = 	18 psf

Floors

Floor joists 2x12 @ 16” O.C.							7 psf

Floor decking 1” OSB 								3.2 psf

Floor Covering 	 Tile								16 psf

Floor Covering hardwood							4psf

Insulation 10” 									3 psf

Total Floor = 	29.2 x 1.6 = 47 psf

25. Residential

One- and two-family dwellings

Uninhabitable attics without storage			10 psf

Uninhabitable attics with storage 			20 psf

Habitable attics and sleeping areas 			30 psf

All other areas 						40 psf

26. Roof

All other construction 				20 psf

Add the weight of vehicles for garage and parking on slabs.