Design  Pool  Shell Steel

Weight of Water 						62.4 #/ft3   x 5 ft depth = 312 psf

Pool bottom thickness = 					8” (.75 ft) x 150 pcf = 112.5 psf

Total unfactored  Dead Load 								424.5 psf

Occupancy = 111 person with an average weight of 200#/person = 22,200# / 2237 s.f. = 9.9 psf

Total unfactored Loads								434.4 psf

Factored Loads  =  1.2(D+F+T) + 1.6(L+H) + 0.5(Lr or S or R)

=  1.2 (424.5) + 1.6 (9.9) = 525.24 psf

To estimate gravity loads at Pool Wall the thickness =	10” (.83 ft) x 150 pcf x 5.5 ft tall = 684.75 plf

Pool Shell Floor uses unfactored loads for soil pressure, the allowable gravity load pressure is estimated at 3,000 psf with a Factor of Safety of 3, therefore both bottom and sides are well within the allowable soil pressure.

ShotCrete estimated to have a compressive strength of 5,000 psi @ 28 days

ML= WL x L28= 9.9 x 128=1.2 ft-#

MD= WD x L28= 424.5 x 128=53.1 ft-#

MU= 1.2 MD+  1.6 ML=1.2x 53.1+ (1.6x1.2) = 65.6 ft-#

AsRequired=  MU Φfyjd= 65.60.960,000(0.9)(5.3)=2.54 x 10-4 sq inches

#3 bars @ 10” o.c. has 0.132 sq. inches   			therefore OK