554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

November 27, 2018

For: 	Joel J. Bruno

120 Village Street

Slidell La. 70458

Ref:    61004 Timber Bend Dr.

Lacombe, La.

Construction:

Two-story, wood frame, wood veneer, ceramic and carpet flooring with a metal roof on a CMU pier and beam foundation. The home is approximately 28 years old.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due floors being uneven.

Findings:

Upon inspection, a zip level was used to verify the floor height conditions.  The level was set up at the front door at zero inches and measured throughout the home, there is a -1.8 inch difference noted in the kitchen of the home.  It is important to note that all floor height conditions are within acceptable tolerances. The center of the home appears to have the most notable slope.

The interior of the home was noted to have some cracks in the ceramic flooring but none where noted in the sheet rock walls. An overall visual inspection of the exterior of the home was conducted and no items were noted.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

It is my opinion that the structural integrity of this home is currently sound, but the CMU pier and beam foundation will expand and contract with the ground movement and any standing water. The home can be leveled if desired. The yard should always be graded to allow for proper drainage of rainwater away from the foundation.

Sincerely,

Brian Mistich, P.E.