Renovation Project Specifications

740 Oak Harbor Blvd, Slidell, LA, 70458

EXTERIOR

- Repair roof flashing – replace fasteners with marine grade stainless steel

- Driveway expansion – Obtain feasibility and estimate for widening driveway and turn on the northwest and southwest corners of the property.

- Remove all parking bumpers along newly established delivery driveway on both sides of the building.

- Relocate dumpster – Move to southwest corner of the property.

- Eliminate small garden on the southwest corner of the building as to expand the exit driveway.

- Stucco – conduct test to assess feasibility of removing building stains (via chemical wash), thereby possibly avoiding repainting the building via chemical wash.

- Loading ramp – Demo concrete at shipping receiving door and add concrete ramp for forklift use.

INTERIOR

NOTE 1: Refer to hand written notes on hardcopy plan.

NOTE 2: We will reuse/repurpose doors, cabinetry, and other material as much as feasible.

Ground Level – Remove all walls as depicted on plan.

NOTE 3: The instructions below correspond to the handwritten numbers on the plan, which are circled and highlighted.

- Remove Shampoo bowls, patch drywall.

- Modify “Storage Room” adjacent to Stairwell #2.

- Offices 1, 2, & 3 are pre-existing and do not appear as is on the original plan.  No work needed in this office suite.

- Replace carpet in Office #4.

- Install solid core door.

- Add hallway to connect Showroom to Stairwell #1.

- Install solid core door.

- Relocate select base cabinet sections with counter tops to designated areas to second floor classrooms 2 and 3.

- Remove all counters, patch drywall.

- Relocate LED track lights to Showroom.

- Replace rear entry door with loading bay capacity doors which are compliant with Oak Harbor Property Owners Association.

- Relocated base cabinets, countertops, and wall mounted cabinets to designated area in new Break Room on second floor.

Second Floor

- Remove all counters in “Instructors Office”.

- Install durable, low maintenance floor tile.

- Designated area from ground floor cabinets/countertops. (See #8 on ground floor plan).

- Replace new white boards in Classrooms 2 and 3.

- Designated area for cabinets, countertop, and sink from ground floor Break Room.

General

- Repaint interior

- Patch concrete flooring where needed.

To Be Determined

- Electrical – Manifest Station, electric forklift charging station.