Prefabricated Metal Specification

For:

6th Ward Junior High School
72360 Highway 41

Pearl River, Louisiana

Prepared by:

Siplast

1000 East Rochelle Blvd.

Irving, Texas 75062

(800) 922-8800

This specification is provided as a general guide for use of Siplast roof perimeter systems.  Siplast is strictly a supplier of roof perimeter systems and has no experience, training or expertise in the areas of architecture or engineering or in the area of consulting with respect to matters related to such areas.  Siplast recommends that the Owner's representative independently verify the accuracy and appropriateness of a specification provided for a specific project.

March 22, 2010

SECTION 07710 PREFABRICATED FASCIAS COPINGS AND EXPANSION JOINTS 

(Rev 10/2007)at
PART 1  GENERAL

1.01  SUMMARY:

A.
Work included: Furnishing and installing factory fabricated and finished raised edge, gravel stop, coping, and expansion joint systems.

1.02  RELATED SECTIONS

A.
Section [----] - Rough Carpentry

B.
Section [----] - Roof Decks

D.
Section [----] – Rigid Roof Insulation

E.
Section 07550 – Modified Bitumen Membrane Roofing

1.03  REFERENCE STANDARDS

References in these specifications to standards, test methods, codes etc., are implied to mean the latest edition of each such standard adopted.  The following is an abbreviated list of associations, institutions, and societies which may be used as references throughout these specifications.

FM
Factory Mutual Engineering and Research


Norwood, MA

ANSI
American National Standards Institute


Washington, DC

SPRI
Single Ply Roofing Industry


Waltham, MA

1.04  SUBMITTALS

A.
Submittals Prior to Contract Award:

1.
Latest edition of the prefabricated metal component supplier’s specification.

2.
Latest edition of prefabricated metal component supplier’s Installer’s Guide for gravel stop system.

3.
Samples:  Available on request; sized to represent metal components adequately.

4.
Copy of the roofing system manufacturer's inclusion addendum offering coverage of the prefabricated systems under the standard terms of the roofing guarantee.

1.05  QUALITY ASSURANCE

A.
Agency Approvals:  The proposed prefabricated metal component shall conform to the following requirements.  No other testing agency approvals will be accepted.

1. FM Approval for Class 1-___ Windstorm Classification for Roof Perimeter Fascia Systems.

*NOTE: Verify the FM Approvals Windstorm Classification for the appropriate metal component dimension and gauge per FM Global RoofNav.

2.
The roof perimeter fascia system shall be certified through third party verification by the manufacturer/supplier to meet performance design criteria according to the most recent edition of ANSI/SPRI ES-1: Wind Design Standard for Edge Systems Used with Low Slope Roofing Systems.

B.
Scope of Work:  The work to be performed under this specification shall include but is not limited to the following:  Attend necessary job meetings and furnish competent and full time supervision, experienced mechanics, all materials, tools, and equipment necessary to complete, in an acceptable manner, the prefabricated metal installation in accordance with this specification.  Comply with the latest written application instructions of the supplier of the prefabricated metal components. 

C.
Local Regulations:  Conform to regulations of public agencies, including any specific requirements of the city and/or state of jurisdiction.

D.
Manufacturer Requirements:  Ensure that the prefabricated metal component supplier provides direct trained company personnel to attend necessary job meetings, perform periodic inspections as necessary, and conducts a final inspection upon successful completion of the project.

1.06  PRODUCT DELIVERY STORAGE AND HANDLING

A.
Delivery:  Deliver materials in the manufacturer's original packaging. 

B.
Storage:  Store materials out of direct exposure to the elements.

C.
Plastic Surface Film:  Do not remove the plastic surface film on the metal component until after it is installed.

D.
Damaged Material:  Any materials that are found to be damaged will be automatically rejected, removed and replaced at the Contractor's expense.

1.07  PROJECT/SITE CONDITIONS

A.
Requirements Prior to Job Start

1.
Verify that all other trades responsible for related work are complete prior to installing the prefabricated metal components.

2.
Mounting surfaces shall be straight and secure and provide adequate widths to properly support the prefabricated metal component.

3.
Safety:  Familiarize every member of the application crew with all safety regulations recommended by OSHA, SMACNA and other industry or local governmental groups.

B.
Protection Requirements

1.
Prefabricated Metal Component Protection:  Provide protection against mechanical damage for newly applied prefabricated metal component surfaces throughout the project.

2.
Limited Access:  Prevent access by the public to materials, tools and equipment during the course of the project.

3.
Debris Removal:  Remove all debris daily from the project site.

4.
Site Condition:  Complete, to the owner's satisfaction, all job site clean-up including building interior, exterior and landscaping where affected by the construction.

1.08  GUARANTEE/ ADDENDUM

A. Roof Membrane Guarantee Addendum:  In addition to the specified guarantee, furnish the Owner with the roofing manufacturer's inclusion addendum to the guarantee offering coverage of the prefabricated raised edge, gravel stop, coping, and expansion joint systems under the standard terms of the roof membrane/roof system guarantee.

>
Siplast Paraguard Roof Perimeter System Inclusion Addendum

PART 2  PRODUCTS

2.01  PREFABRICATED METAL COMPONENT SUPPLIER

A. Siplast

1000 East Rochelle Blvd.

Irving, TX  75062

B.
Prefabricated Gravel Stop.  Prefabricated gravel stop components shall be factory formed according to the requirements of the membrane manufacturer.  The gravel stop system shall consist of the following components:

-
A factory formed retainer cleat with pre-punched nail holes fabricated from 24 gauge, G90 galvanized steel,, secured using galvanized roofing nails.

· A factory formed gravel stop with pre-punched nailing holes, secured using galvanized roofing nails. fabricated from minimum 24 gauge galvanized steel having a spray coated Kynar™ finish.

-
Factory formed concealed splice plates.

-
Factory formed welded miters.

>
Proform Gravel Stop, by Siplast, Inc., Irving, TX
(800) 922-8800

PART 3 EXECUTION

3.01  PREFABRICATED METAL COMPONENT INSTALLATION 

A.
Reference Paraguard Roof Perimeter Systems Installer’s Guide for installation procedures

3.02  FIELD QUALITY CONTROL AND INSPECTIONS

A.
Site Condition:  Leave all areas around job site free of debris, construction materials, equipment and related items after completion of job.

B.
Issuance Of The Addendum to the Roof Membrane/System Guarantee:  Complete all post installation procedures and meet the prefabricated metal suppliers final endorsement for issuance of the addendum to the specified roofing guarantee.

PG-28-35
07710 - 1
Paraguard Roof Perimeter Systems


