SECTION 01582

PROJECT IDENTIFICATION SIGN

- GENERAL

  - SUMMARY

    - Section Includes:  Project Identification Sign

    - Related Sections include the following:

      - Not Used.

- PRODUCTS

  - SIGN MATERIAL

    - Graphic Design, Content, Colors, Lettering Style:  See attached sketch.

    - Sign Size:  nominal 4 feet by 8 feet.

    - Quantity:  Provide 2 Project identification signs.

    - Support Structure, Framing, and Trim: All pressure treated, sizes as specified on sign detail at end of this Section.

    - Paint and Primers:  Exterior quality, gloss alkyd enamel, two coats, paints shall be approved by Architect.

      - One coat primer:  Exterior Alkyd Wood Primer:

        - Sherwin Williams: Exterior Oil Wood Primer, A-100.

      - Two topcoats:  Exterior Alkyd Enamel (Gloss): (Gloss Level 6).

        - Sherwin Williams; Seaguard 1000 Marine Enamel, Industrial & Marine.

        - Color shall be Sherwin Williams SW 6524 “Commodor” or approved equal.

    - Concrete:  Concrete for sign post footings shall be minimum 3,000 psi.  Comply with ASTM C 94; ACI 301, “Specification for Structural Concrete”; ACI 117, “Specifications for Tolerances for Concrete Construction and Materials”; and CRSI’s “Manual of Standard Practice.”

    - Fasteners: Fasten sign with stainless steel screws and stainless steel cup washers.  All other fasteners and accessories shall be hot dip galvanized.

- EXECUTION

  - INSTALLATION

    - Install Project identification signs within 30 days after date fixed by Notice to Proceed.

    - Erect Project identification signs in locations as directed by Architect.

    - Install signs plumb and level.  Anchor securely.

    - Paint all exposed surfaces of sign, supports, and framing according to manufacturer’s and MPI’s recommendations.

  - MAINTENANCE

    - Maintain signs and supports and keep clean.  Repair damage.  Replace deteriorated signs during course of project as directed by Architect.

  - REMOVAL

    - Remove signs, framing, supports and foundation at completion of Project and restore the area.

END OF DOCUMENT