Gutters and Downspouts

SMACNA:

Bldg B

Area = 115 ft x 30 ft = 3450 s.f. (Each Side)

Minimum size gutter for a square gutter is recommended as 6”.

Maximum length of gutter per downspout is 50ft  The minimum number of downspouts to service 115 linear ft is 3 downspouts.  Our design shows 7 downspouts.  If we reduce the number of downspouts to 4, which is what is currently installed; we exceed the minimum number of downspouts.