ST. TAMMANY PARISH SCHOOL BOARD

SALMEN HIGH SCHOOL

NEW PRESS BOX

SLIDELL LA 



Copyright 2011 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

SECTION 133416.03 – MODULAR PRESS BOX
Revise this Section by deleting and inserting text to meet Project-specific requirements.

This Section uses the term "Architect." Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 SUMMARY

A. Provide engineering, material, freight, installation, and supervision to provide new 8' – 0” x 48'-0” wide prefabricated, Type VB, modular press box.
1.2 REFERENCES

A. NFPA 70 National Electric Code.

B. International Building Code, (IBC)

C. Act 364 of the Louisiana 2007 Regular Session, requires that Industrialized Buildings have the plans for each model reviewed by the Louisiana State Fire Marshal. 
1.3 ACTION SUBMITTALS

A. Complete shop drawings showing plans, sections, elevations, and wall sections showing complete detail of layout, connection, and trim detail.
1. Include all attachments to grandstands.

2. Plans shall have Louisiana State Fire Marshal stamp indicating their review and approval.

B. The building shall have the proper Louisiana State Fire Marshal insignia installed indicating that this building meets the Louisiana State requirements.

1.4 CLOSEOUT SUBMITTALS

A. Maintenance data.

1.5 DESIGN CRITERIA

A. General : Provide proper temporary bracing of the structure for wind and construction loads until the installation of all permanent structural elements is complete.

B. Code Compliance : Units shall comply with NFPA 101, FBC, and ASCE 7
C. All materials shall be new and shall comply with the requirements of the ASTM specifications.
1.6 QUALITY ASSURANCE

A. Manufacturer shall have a minimum of 5-years experience in fabrication of grandstand structures.

B. Engineering Qualifications :

1. A licensed professional engineer registered in the State of Louisiana shall approve the Press Box design and all submittals shall bear said professional's seal.
2. Calculations are required, must show all vertical and lateral loads, and must show positive and negative biaxial stress ratios.
3. Submit the calculations with the approval drawings.
C. Act 364 of the Louisiana 2007 Regular Session, requires that Industrialized Buildings have the plans for each model reviewed by the Louisiana State Fire Marshal. 
D. Product Liability : Provide details Certificate of Insurance, including products/complete operations insurance.
1.7 WARRANTY

A. Provide product guaranteed for a period of one year against defective materials and workmanship for the manufactured modular press box and installed contents.
B. Provide product guaranteed for a minimum period of 15 years against defective materials and workmanship for the roof of the manufactured modular press box.
Verify available warranties and warranty periods for units and components with manufacturers listed in Part 2. 20-year period is common for fluoropolymer finish; 25- and 30-year periods are available from some manufacturers. 10-year period is usually available for siliconized polyester.

C. Damage resulting from abnormal use, vandalism, or incorrect installation (if installed by other than authorized installer of the manufacturer) is not applicable.

Verify available warranties and warranty periods for units and components with manufacturers listed in Part 2.

Insert requirements for extra materials such as metal roof and wall panels, trim, and accessories if required. Verify with Owner because these materials can present storage difficulties.

PART 2 -  PRODUCTS

2.1 MANUFACTURERS

A. Basis of Design: Subject to compliance with requirements, provide product indicated on drawings by “National Recreation Systems” or comparable elevated press box product by one of the following:
1. American Press Box

2. Panel Built Incorporated

3. Whitley Manufacturing
B. Prior approval before bid is required for products by one of the alternates listed above.
2.2 SIZE
See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers. Retain one of two paragraphs and list of manufacturers in this article. See Section 016000 "Product Requirements."

A. The overall size of the press box shall be 8'-0” x 48'-0” width and entrance platforms on both ends.
Because there are significant differences between manufacturers' systems, metal building system specifications are usually developed to suit design requirements of a single manufacturer. Evaluate other manufacturers' systems to verify suitability for Project.

B. MATERIALS : 
1.  FLOOR CONSTRUCTION

a. Bottom Board: 1/2” CDX plywood sheathing (painted black).

b. Insulation : Min. R-19 fiberglass batts, with vapor barrier.

c. Joists : 2” x 6” #2 SPF, on 12” centers, transverse framing.

d. Decking : ¾”, underlayment grade, tongue and groove fir plywood, (Index 24” o.c.)

e. Covering : 1/8” vinyl composition tile, color by owner.

f. Molding : 4” vinyl base molding.

2. WALL CONSTRUCTION

a. Studs : 2” x 6” dimensional lumber, grade #2 or better SPF, installed 16” o.c., IBC framing.

b. Bottom Plate: 2” x 6” dimensional lumber, grade #2 or better SPF.

c. Top Plates : (2) 2” x 6” dimensional lumber, grade #2 or better SPF

d. Headers : As span and design load requires.
e. Ceiling Height : 8’-0” in front sloped to back

f. Covering : 5/8 vinyl-faced gypsum panels, Class A, F.S.R.

g. Insulation : Minimum R-21 fiberglass batts with vapor barrier.

h. Sheathing : ½” CDX Plywood and House wrap air infiltration barrier.

i. Siding : MBCI “PBU-Panel” 26 gauge ribbed steel panels with Kynar 500 finish.

3. ROOF CONSTRUCTION
a. Joists : 2” x 10” dimensional lumber, grade #2 installed 16” o.c or #1 SYP as required.

b. Overhang : 15-1/2” over front wall; 6” over rear wall.  .019 aluminum fascia with perforated aluminum soffit panels.
c. Ceiling : 5/8” gypsum board, taped and bedded with smooth finish, Class A F.S.R.

d. Insulation : Minimum R-30 fiberglass batts with vapor barrier.

e. Decking : ¾” Sturdifloor, underlayment grade, tongue and groove plywood (Index 24” o.c.)
f. Covering : .060 polyester reinforced skid and spike resistant pvc membrane, fully adhered.  Membrane shall be reinforced skid & spike resistant
4. WINDOWS

a. Interior windows to be ¼” tempered safety glass fixed pane with stained jambs and casing.
b. Exterior windows shall be 62-1/4"x48" WINGUARD #HR7710A series, or equal, aluminum horizontal roller window w/ 13/16" LIG (1/8" ANN-3/8" AIR-5/16" ANN/ANN laminated) impact-resistant glass, gray tint and screens (mulled to create 125-1/2" clear span)
5. DOORS : (Exterior) : “Oakcraft” wood-gran textured insulated fiberglass entry door with solid vinyl jabs; 16” insulated/tempered lite, aluminum threshold, vinyl weather stops, stainless steel knuckle hinges and heavy-duty retention chain.  Doors equipped with commercial lever-handled keyed locksets.  Min. U-Value = .22
6. DOORS : (Interior) : 1-3/8” solid-core stained Birch door with stained Fruitwood Birch wood jambs and casing.  Passage lever-handled hardware.
7. ELECTRICAL

a.  Service Entrance Panel : Square D QO112M100 with Main Disconnect; rated at 120/240v, single phase, 100 amp capacity.
b. Switches/Receptacles : 
1) Pass & Seymour #TM870 125 volt/15 amp duplex, spec-grade, switches.

2) Intermatic #EI500 programmable astronomical timer switches as required.
3) Lutron #MSOPS5MLA occupancy sensor switches as required.

4) Pass & Seymour #3232 125 volt/15 amp duplex, spec-grade,

receptacles.

5) Wiremold 5400 Series two-piece multi-channel, dual voltage, non-          metallic surface raceway along front wall below scorer’s counter, outlets on 48” centers.

6) Conduit prep and circuity for customer’s PA and DATA systems.

c. Lighting 

1) Interior: SATCO #45/LED/1X4/FLUSH/3K/WH 45 watt, 30K LED 1X4 surface mounted LED light.

2) Exterior: SATCO #S9014 4” (7 watt) 4000K LED recessed light
3) Emergency/Exit: Lithonia ECR-REM-LED emergency combination exit/flood light with 90 min. battery back-up and ERE-SLG-WP LED remote emergency flood light.
d. Circuits: All branch circuit wiring is minimum #12 THHN copper wire           encased in EMT thin-wall conduit or MC cable.

e. HVAC:  GE Zoneline 4100 series packaged terminal HVAC units with integral thermostats if required

8. SCORER’S COUNTER : 18” deep x 3/4” lauan grade plywood with 1-1/2” x 2” edge, surfaced with .060 plastic laminate by Nevamar or equal.
9. CAMERA DECKS 

a. Hatch:          Bilco Model NB50, or equal, 2’6” x 4’6” aluminum roof hatch.

b. Ladder (Aluminum):          Alaco Model H70, or equal, 70-degree ships ladder.

c. Roof Surface:          Dec-K-ing, or equal, .060 polyester reinforced skid and spike resistant PVC membrane, fully adhered.

d. Railing mounts: 1/2” galvanized threaded bolts & nuts through roof fascia on 48” centers along perimeter edge of roof.
10. MISCELLANEOUS 

     10 LB. dry chemical fire extinguisher, tagged locally for Fire Marshal Inspection.  Rated 4-A: 20-B:C

2.3 FABRICATION

A. General: Design components and field connections required for erection to permit easy assembly.

1. Mark each piece and part of the assembly to correspond with previously prepared erection drawings, diagrams, and instruction manuals.

2. Fabricate structural framing to produce clean, smooth cuts and bends. Punch holes of proper size, shape, and location. Members shall be free of cracks, tears, and ruptures.

PART 3 -  EXECUTION

3.1 ERECTION OF STRUCTURAL FRAMING

A. Erect building system according to manufacturer's written erection instructions and erection drawings.

B. Do not field cut, drill, or alter structural members without written approval from building system manufacturer's professional engineer.

END OF SECTION 133416.03
	MODULAR PRESS BOX
	133416.03  - 1



