554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

Fax: 985-641-5950

dammonengineering.com

dammoneng@bellsouth.net

# Structural Inspection

June 6, 2014

For: 	Mr. Mark Stielper

253 Grand Ave.

Lafayette, LA 70503

Re: 	46 Wyndham Ct.

Slidell, La. 70458

Construction:

Single-story, wood frame, brick veneer, composition shingle roof on a conventional foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the residence. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers

And is not an explanation of cause, effect, or engineering.

History:

Mark Stielper contacted Dammon Engineering to request a structural inspection of the referenced home due to concerns from a separation in the exterior rear laundry room door and brick veneer that developed over time. This is a 30+ year old home. A zip level was used to verify existing floor height conditions.

Findings:

Upon inspection, it was noted that the rear exterior laundry room door has separated from the brick veneer.  The separation starts at the bottom of the jamb and widens as it nears the top jamb.  Upon inspecting the inside door jamb, a hairline crack was noted in the sheet rock located near the top wood trim.  The crack runs up to the ceiling and ends at that point.  No visible cracks were noted in this same general area of the flooring or foundation.

The exterior door was found to be a hollow core door and is not suitable for exterior application.  Furthermore, although there are gutters along the rear of the home, rainwater appears to be overflowing the gutters and saturating the door.  This exposure of an interior door to exterior elements has exaggerated the normally expected expansion and contraction that would occur on an exterior door.  While still intact, the door is sticking at the bottom and is difficult to open.  The above scenario has caused the door to come out of alignment.

A zip level was used to verify the floor height conditions, please note that all floor height conditions are within acceptable tolerances.  The differences noted are from different types of flooring material (carpet, tile and wood flooring).

An overall visual inspection of the exterior of the residence did not reveal any cracks in the brick veneer or foundation footing, nor were there signs of repair.

Conclusion:

It is important to note that all house foundations settle.  Usually everything settles together, and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

The separation in the door jamb does not appear to have compromised the structural integrity of the slab.

It is my opinion that the structural integrity of this slab is currently sound.

Sincerely,

Brian Mistich, P.E.

Attachment:  Floor Plan Noting Elevation Differences