554 Old Spanish Trail

Slidell, LA 70458

Phone: 985-649-5832

dammonengineering.com

info@dammonengineering.com

# Structural Inspection

November 30, 2016

For: Dale Herzog

105 Charlie Drive

Slidell La. 70461

Ref: 628 Poupee Lane

Slidell, La

Construction:

Two-story, wood frame, brick veneer, ceramic/carpet and wood flooring with a composition shingle roof on a pier and beam foundation.

Scope:

This inspection is limited to a visual inspection of the shell of the home, including the interior and exterior foundation of the home. No Inspection of the mechanical or electrical systems was performed.  This report is as outlined by the National Academy of Building Inspection Engineers and is not an explanation of cause, effect, or engineering.

History:

Dammon Engineering was contacted to request a structural inspection of the referenced home due to a home inspection that noted some cracks in the brick veneer.

Findings:

An overall visual inspection of the exterior of the home was conducted and several items were noted. There are areas where the brick veneer was noted to have cracks in them. One on the corner of the home at the front left porch, one below the utility room window, one on the right side of the home near the ridge and one below the ridge near the porch along with some of the flood vents.  All of them expect for one has been re pointed and one was caulked.

The right side of the home was noted to have some earth erosion near the foundation due to roof water runoff.

An overall visual inspection of the interior walls and attic of the home was noted to be fine.

Conclusion:

It is important to note that all foundations settle.  Usually, everything settles together and the fastest rate of settlement occurs in the first year after construction.  By the fifth or sixth year the settlement has exponentially slowed to where there is little danger that something will happen unless caused by weather, expansion & contraction, tree roots, improper drainage, etc.

Recommendation:

The cracks in the brick veneer do not appear to have compromised the structural integrity of the home.  It is my opinion that the structural integrity of this home is currently sound, but the foundation will expand and contract with the ground movement and any standing water.

Items to be addressed and keep an eye on are as follows:

1)  Reseal the cracks in the brick veneer with a non-shrinking grout, to prevent insects from entering the home when cracks are visible.

2) The yard should also be re-graded to allow for proper drainage of rainwater away from the foundation as not to cause erosion.

Sincerely,

Brian Mistich, P.E.