Ashford Place

West Monroe, LA

Date of inspection: Wednesday, August 15, 2007

Unit 9, a 3BR complex

Met with Mr. Billy Manshack, Project Superintendent to inspect two buildings.

This unit is ready for sheetrock.

In each unit the plumbing appeared to be correctly installed.  Straps were in the proper locations.  Although the hot water heaters were not installed the plumbing was ready to be connected.  Drains were correctly fire calked in all areas.

Outlet boxes on the opposite sides of a wall are installed so as not to be between the same two studs.

Bathroom exhaust fans are installed so as not to require a penetration through a plate or stud and are vented to an outside wall.

A fire sprinkler is now installed on each porch area.

All firewalls are taped and sealed.

Straps properly support flex hoses for HVAC.

Phone and TV cables have been installed.

3rd floor recessed lighting has been installed with proper smoke partition.

I understand that there are weekly meeting with all subs where any conflicts are discussed and decisions are made as to how to correct them before the meeting is finished.  These subs appear to be working together.  The work is clean and neat.  Floors are swept clean and trash removed. When a sub completes working a given area he must leave that area broom clean with no trash remaining.

Jack Stroupe

