
SECTION 15440PRIVATE 


PLUMBING FIXTURES

PART 1 GENERAL


1.01
SECTION INCLUDES



A.
Water closets.



B.
Hot water heaters.



C.
Lavatories.



D.
Sinks.



E.
Bath tubs.


1.02
REFERENCES



A.
ASME A112.18.1 ‑ Finished and Rough Brass Plumbing Fixture Fittings.



B.
ANSI/ASME A112.19.1 ‑ Enameled Cast Iron Plumbing Fixtures.



C.
ANSI/ASME A112.19.2 ‑ Vitreous China Plumbing Fixtures.



D.
ANSI/ASME A112.19.4 ‑ Porcelain Enameled Formed Steel Plumbing Fixtures.



E.
ANSI/ASME A112.19.5 ‑ Trim for Water‑Closet Bowls, Tanks, and Urinals (Dimensional Standards).

F. ANSI/ARI 1010 ‑ Drinking‑Fountains and self-contained, Mechanically‑Refrigerated Drinking‑Water Coolers.



G.
Reference to General Conditions.



1.03
SUBMITTALS



A.
Submit under provisions of Section 01300.



B.
Product Data: Provide catalogue illustrations of fixtures, sizes, rough‑in dimensions, utility sizes, trim, and finishes.



C.
Manufacturer's Installation Instructions.


1.04
OPERATION AND MAINTENANCE DATA



A.
Submit under provisions of Section 01700.



B.
Maintenance Data: Include fixture trim exploded view and replacement parts lists.


1.05 
DELIVERY, STORAGE, AND HANDLING



A.
Deliver, store, protect and handle products to site under provisions of Section 01600.



B.
Accept fixtures on site in factory packaging.  Inspect for damage.



C.
Protect installed fixtures from damage by securing areas and by leaving factory packaging in place to protect fixtures and prevent use.


1.06
FIELD MEASUREMENTS



A.
Verify that field measurements are as instructed by the manufacturer.



B.
Confirm that millwork is constructed with adequate provision for the installation of countertop lavatories and sinks.


1.07
WARRANTY

A. Provide five-year warranty under provisions of Section 01700.



B.
Warranty:  Include coverage of electric water cooler compressor.


1.08
EXTRA MATERIALS



A.
Furnish under provisions of Section 01700.



B.
Provide two faucet washers flush valve service kits

PART 2 PRODUCTS


2.01
FIXTURES: Plumbing contractor shall furnish and install all plumbing fixtures shown on accompanying drawings.  Refer to both plumbing and architectural and provide all fixtures shown on either.  Fixtures shall be complete with all necessary brass and accessories required for a complete installation including traps, escutcheons, angle supplies, basin cocks, etc.  All fixtures shall be new and shall be delivered to the building properly crated in perfect condition.




All brass must be of the best quality, lightweight goods will not be accepted.  All brass pipes shall be seamless brass tubing and nipples shall be extra heavy.  All fittings and trim shall be chromium plated heavy brass unless otherwise specified.  “P” traps shall be cast brass with clean-outs.  All exposed piping shall be chromium plated.  Provide cut-off valves at each fixture in both hot and cold water piping.




For the purpose of establishing type and class of fixtures required, the following plate numbers have been taken from the Kohler catalog unless otherwise indicated.  Fixtures as manufactured by Crane, Eljer, or American Standard, of equivalent quality and manufacturer will be accepted.




All fixtures shall be new.  All wall-hung fixtures shall have factory type mounting brackets in wall.

A. Water Closet: (15” high) – 
Manufacturers:

1. American Standard

2. Colony
3. Afton

round front toilet. White, vitreous china, siphon jet, elongated water closet.  Solid plastic, white seamless cover and with hinge. 

B. Handicap Water Closet:
(18” high) – 
Manufacturers:

1. American Standard
2. Kohler

3. Crane

white, vitreous china, siphon jet, elongated water closet, solid plastic, white,  toilet seat and cover and with check hinges.
C. Lavatory:
Manufacturers:

1. American Standard
2. Kohler

3. Colony

15” round white enameled steel mounted on cabinet top.  Mixing faucet.
D. Bath tubs:

Bath #1: 
Manufacturers:

1.
“Sterling” Advantage
2.
Kohler

3.
Crane

5 ft. tub/shower, 60”L x 30”W x 73 ¼” H, with white swirl glass finish.
Bath #2:
Manufacturers:

1.
“Sterling” Ensemble
2.
Kohler

3.
Crane

5 ft. tub/shower Model 712201, 60”L x 43 ½” W x 73 ¼” H, white solid vikrell material, oval bath with tile wall surround.

E. Sinks:
Manufacturers:

1. Dayton “Kingsford” Model K-50-23322
2. Kohler

3. Crane

Double bowl, stainless steel, 33x22 with faucet openings for four faucets.

F. Hot Water Heaters:
Manufacturers:

1. A.O. Smith

2. G.E.

3. Rheem

40 Gallon, 52 Gallon: Energy Saver Residential Electric Water Heater with two 4500 Watt, 240 Volt single phase elements and water-tight plastic water heater pan.
PART 3 EXECUTION


3.01

EXAMINATION



A.
Verify that walls and floor finishes are prepared and ready for installation of fixtures.



B.
Verify that electric power is available and of the correct characteristics.


3.02
PREPARATION



A.
Rough‑in fixture piping connections in accordance with minimum sizes indicated in fixture rough‑in schedule for particular fixtures.


3.03
INSTALLATION



A.
Install in accordance with manufacturer's instructions.



B.
Install each fixture with trap, easily removable for servicing and cleaning.



C.
Provide chrome plated rigid or flexible supplies to fixtures with screwdriver stops, reducers, and escutcheons.



D.
Install components level and plumb.



E.
Install and secure fixtures in place with wall supports and bolts.



F.
Seal fixtures to wall and floor surfaces with sealant as specified in Section 07900, color to match fixture.



G.
Solidly attach water closets to floor with lag screws.   Lead flashing is not intended hold fixture in place.


3.04
INTERFACE WITH OTHER PRODUCTS



A.
Review millwork shop drawings.  Confirm location and size of fixtures and openings before rough in and installation.


3.05
ADJUSTING



A.
Adjust work under provisions of Section 01700.



B.
Adjust stops or valves for intended water flow rate to fixtures without splashing, noise, or overflow.


3.06
CLEANING



A.
Clean work under provisions of 01700.



B.
At completion clean plumbing fixtures and equipment.


3.07
PROTECTION OF FINISHED WORK



A.
Protect finished Work under provisions of Section 01500. 



B.
Do not permit use of fixtures.


END OF SECTION

PAGE  
Crosswinds Apartments
15440 - 4
PLUMBING FIXTURES

