Coalition for Citizens

Gwen Smith

Fund Raising Consultant

We were asked by Gwen Smith to inspect a two story brick building with basements Approx. 30,000.00 sq.ft. The building is approximately 40+ years old and was used as a hospital.

This building was constructed in three phases. All are brick with the exception of one small addition in the rear of the building. Parts of the building are wood frame construction with a brick veneer. Most parts of the building are cmu blocks with a brick veneer. The floors are concrete with either carpet or vinyl coverings. Through out the building the interior walls vary from Sheetrock to cmu blocks. The ceilings also vary from drop ceilings/sheetrock to concrete ceilings. There are a number of bathrooms and rooms with bathrooms that don’t meet the current ADA code. These buildings have central air, window units and wall units for cooling and heating.

Upon our visual inspection it was noted that:

1) In a small area of wall near an entrance door in the courtyard some brick facade are coming loose form the wall. The other sides of the brick bldg. looked fine.

2) The facia and soffit was noted in some places to be rotten, Also some gutters need replacing.

3) The roof appears to be fairly new, some places were noted to have been repaired. 

4) The interior of the bldg. shows signs of wear. The ceilings in some places are coming down due to water damage from the roof that appears to have been repaired.

5) All walls need to be repainted.

6) All floor coverings need to be replaced.

7) The are numerous air conditioners in this building. All of them need to be serviced.

8) There is also an elevator in one section of the building that needs to be serviced and brought up to code.

The overall structure of the building appears to be fine.
