Friday, April 27, 2007

Inspected the doors, ceiling tiles and recessed overhead lights.

1. Doors are solid core wood.  Several doors are partially de laminated on lower edge probably from flooding.  Others have damage with sections broken out.  Others appear to be in acceptable condition.

2. Recessed florescent lights are in very good condition and should be reusable after being cleaned.  One exception in the maintenance room.  All lights here (12 double florescent) are surface mounted and should be replaced with matching recessed fixtures.

3. Ceiling tiles are not in good condition.  Most have what appears to be water damage that is probably from roof leaks, or condensation from AC ducts.  They will all need to be replaced.  They are 2x2 ft in size.

4. Dropped ceiling frame appears to be in good condition and reusable. 

Jack

