Summary Report for Roof Damage at MECO Building # 2 Port Of New Orleans

MECO Bldg 2 is a metal-framed structure located on the Industrial Canal side of New Orleans LA at the Port of New Orleans. The facility has metal wall panels and an R-panel type of roof system. There was also a small portable building located on the north side of the building. The roof is metal R-panel roof system with an area of approximately 13,240 SF.  The portable building is a R-panel covering with an area of approximately 102 SF.

The initial investigation reveled that there was damage to the roof system due to projectile damage and wind lifted panels. There was damage to the wind turbines, eve and gable trim, and metal ridge cap. The small portable building was not in the initial investigation. The profile of this type r-panel is no longer available. Roof to be replaced.

A subsequent investigation reveled that there has been temporary repairs in the form of metal panels covering holes and a new panel at the rake on the west end. Elastomeric coating has been applied to large areas of the roof. There continues to be large areas of projectile damage and damage to the gutter and downspouts as well as damage to the rake trim. There has been new metal ridge cap installed as well. The portable building appears to have suffered no damage.

