Summary Report for Roof Damage at MECO Building # 5 Port Of New Orleans

MECO Bldg 5 is a metal-framed structure located on the Industrial Canal side of New Orleans LA at the Port of New Orleans. The office area has brick walls in front. The facility has metal wall panels and an R-panel type of roof system, and a metal corrugated type roof system. The roof has an area of approximately 25,584 SF. The initial investigation reveled that there was damage to multiple areas of the roof. There was projectile damage from flying debris throughout the roof system. There was damage to the ridge at various locations throughout the roof system. The corrugated roof area was severely damaged as panels were blown off the roof and pealed back. The R-panel type roof system had holes in the roof from projectile damage and a large hole in the center of the area with vinyl backed laminated insulation. The roof appears to have lifted through out the system. The laminated backed insulation beneath the R-panels was damaged by wind and water and should be removed and replaced. Due to the widespread damage throughout the roof the entire roof system and the vinyl-backed insulation should be replaced.

A subsequent investigation reveled that very few if any temporary repairs have been made sense the initial repairs after the storm. The roof and insulation continue to deteriorate and should be replaced.

