GENERAL CONTRACTOR'S RESPONSIBILITY:

The General Contractor shall:

Provide all new material, Labor, tools, equipment, insurance, temporary utilities, transportation, permits, fees, tax, superintendant, temporary construction and all other services or facilities necessary to complete and deliver the finished project to the Owner.

Take out permits and arrange for all inspections required by all governmental agencies.

Verify all grades, lines, levels and dimensions as shown on the drawings.

Take necessary precautions for the safety of employees on the job, to prevent accidents or injury to personnel performing the work, and prevent other unauthorized personnel from being on the job site.

Employ and keep on the job at all times a competent general superintendent as his representative: in the absence of the Contractor, instructions given to the superintendent shall be binding upon the Contractor.

Maintain adequate protection of all work from damage and protect the Owner's property.

CONTRACT DOCUMENTS:

The contract documents consist of the executed contract agreement general conditions, the drawings, the specifications and all addenda incorporated in the document prior to execution.  The contract documents shall be signed in duplicate by the Owner and the Contractor, which shall constitute an agreement between them to all of the contract documents, and what is called for by any one Document shall be binding as if called for by all of them.

5. CONTRACTOR'S UNDERSTANDING:

It is understood and agreed that the Contractor and each sub-contractor, by careful examination, satisfied himself as to the nature and location of the work, the character, quality and quantity of the material to be encountered, the character of equipment and facilities needed, the general and local conditions, and all ether matters which can in any way affect the work under this contract.

6. ERRORS & OMISSIONS:

If the Contractor finds any discrepancy between the drawings and the physical conditions at the site or any error or omissions in the drawings or specifications it shall be his responsibility to inform the Engineer immediately.  Any work done to correct any discrepancy, until authorized by change order, will be done at the Contractor's risk.

8.  ENGINEER'S AUTHORITY:

All work shall be subject to inspection and approval by the Engineer or his representative to insure strict compliance with the plans and specifications and terms of the contract. The Engineer shall have authority to stop work whenever such stoppage may be necessary to insure the proper execution of the Contract.  He shall also have authority to reject all work and materials which does not conform to the Contract. He shall decide questions which arise in the execution of the work. The Engineer shall, within a reasonable time after their presentation to him, make decisions in writing on all questions of the Contractor and on all other matters relating to the execution and progress of the work or the interpretation of the Contract Documents.  All disputes concerning questions of fact which may arise under this Contract, and which are not disposed of by mutual agreement, shall be decided by the Engineer subject to written appeal by the Contractor or the Owner.

GUARANTY:

All work under this Contract shall be guaranteed for one year from date of final payment. Whenever guaranteed work is found faulty, the Contractor must repair or replace it in a satisfactory condition. Upon the Contractor's failure to proceed promptly to comply with the terms of any guaranty under this contract, the Owner may have such work performed economically and the Contractor shall promptly reimburse the owner for the cost of the labor, materials, or equipment used in the repair.  Wear and tear and the result of accidents, which are not the fault of the Contractor or his agents, are excluded from the guarantee.

15.  “OR EQUAL” CLAUSE:

If an item is specified by using the trade name of a product or a particular manufacturer, or type of material or equipment, and the Contractor wishes to make an “or equal” substitution, he must obtain prior written approval of the substitution , by  the Engineer.

5. RELATION TO WATER MAINS:

Horizontal Separation: Whenever possible, sewers should be laid at least 10 feet, horizontally, from any existing or proposed water main. Should local conditions prevent a lateral separation of 10 feet, a sewer may be laid closer than 10 feet to a water main if:

a. It is laid in a separate trench.

b. It is laid in the same trench with the water mains located at one side on a bench of undisturbed earths.

c. In either case the elevation of the crown of the sewer is at least 18 inches below the invert of the water main.

Vertical Separation: Whenever sewers must cross under water mains, sewer shall be laid at such an elevation that the top of the sewer is at least 18 inches below the bottom of the water main. When the elevation of the sewer cannot be buried to meet the above requirement, the water main shall be relocated to provide this separation or reconstructed with slip-on or mechanical-joint cast iron pipe, asbestos or cement pressure pipe or prestressed concrete cylinder pipe for a distance of 10 feet on each side of the sewer.  One full length of water main should be centered over the sewer so that both joints will be as far from the sewer as possible.

9. GOVERNMENTAL AUTHORITIES:

This contractor shall secure and pay for all permits required.  He shall arrange for all field tests required and secure all certificates of approval.  Perform all work in accordance with the rules and regulations of local and national codes, Public Health Department, Air and Water Pollution Control Commission:, and any other applicable governing Authorities.

10. GUARANTEES:

Except where longer warranty periods are specified or are provided by product manufacturers, all materials and workmanship shall be guaranteed free from defects for the period of one year from date of acceptance of the work by the Engineer.

The Contractor, when notified in writing by the Engineer or Owner, shall promptly remedy, without cost to the Owner, any defects which develop within the guarantee period. |

CLEANING UP:

Upon completion, remove all excess materials, equipment, temporary structures and debris, leaving the completed project in a neat and clean condition throughout.