May 1, 2013
REQUEST FOR WRITTEN QUOTES

The City of Slidell is soliciting written quotes for City Hall Parking Lot & Courtyard  project.  The project is located at Bouscaren Street.  Quotes will be received until 10:00 am Friday, May 10, 2013 and shall be hand delivered to the Purchasing Department located at 1329 Bayou Lane or mailed to P.O. Box 828, Slidell, LA. 70459, Attn: Purchasing Agent.

Quotes must be submitted in a sealed envelope and clearly marked:

Quote: City Hall Parking Lot & Courtyard
            Quote #13-Q016

File Number 600-129
Due:    May 10, 2013, 10:00 AM CST


Specifications are attached.  

For the successful quote exceeding $25,000, the contractor will be required to provide a payment and performance bond in the amount of not less than 100% of the awarded contract amount. 

*Quotes exceeding $50,000 – A Louisiana State Contractor License is required to perform the work and your license number shall be included in your quote.  The Contractor shall hold a Louisiana Contractor’s license in Highway, Street, and Bridge Construction.
An executed affidavit will be required prior to award of contract.  Certificate of Liability Insurance and Statutory Limits Worker’s Compensation must be submitted prior to award of contract.  

Contractor shall have 21 calendar days to complete this project from the date of Notice to Proceed.  The contractor will be charged liquidated damages of One Hundred Fifty Dollars ($150.00) per calendar day in excess of the stated time of completion.
Quote submissions:  Quotes shall be submitted on the forms provided and signed by an authorized company representative.   

Any questions concerning this project shall be directed to Ms. Donna O’Dell, City Engineer, at (985) 646-4270.

QUOTE FORM for CITY HALL PARKING LOT & COURTYARD
City of Slidell, LA

Quote  # 13-Q016
	ITEM NO.
	ITEM DESCRIPTION
	QUAN.
	UNIT
	UNIT 
PRICE
	EXTENSION

	A-1
	Mobilization
	1
	L.S.
	
	

	A-2
	Excavation
	1,450
	C.Y.
	
	

	A-3
	Backfill (Gran. Material)
	1,050
	C.Y.
	
	

	A-4
	Remove CB & Pipe/Signs
	1
	L.S.
	
	

	A-5
	Remove Existing Concrete, Sidewalks & Curbs
	1
	L.S.
	
	

	A-6
	Install CB’s
	2
	EA
	
	

	A-7
	Install Drain Pipe
	133
	L.F.
	
	

	A-8
	Handicap Ramps
	3
	EA
	
	

	A-9
	Exposed Agg Paving & Brick Pavers
	770
	S.Y.
	
	

	A-10
	Concrete Parking Lot Paving
	850
	S.Y.
	
	

	A-11
	Curb
	430
	L.F.
	
	

	A-12
	Striping and Signs
	1
	L.S.
	
	

	A-13
	Landscaping & Irrigation
	1
	L.S.
	
	

	A-14
	Grass Pavers
	375
	S.Y.
	
	

	A-15
	Tie in to Existing CB
	1
	L.S.
	
	

	A-16
	Remove shrubs
	6
	EA
	
	

	
	
	
	
	
	


NOTE:  The above quantities are approximate and will be adjusted to fit the budget. 
                                    



TOTAL QUOTE PRICE IN FIGURES OF ASPHALT ROADWAY
                                    



TOTAL QUOTE PRICE IN WORDS OF ASPHALT ROADWAY
*LA. LICENSE NUMBER 






SIGNATURE 







 

COMPANY NAME 







SIGNER’S NAME (PRINTED) 





CITY HALL PARKING LOT & COURTYARD
City of Slidell, LA

Quote # 13-Q016
Slidell File No. 600-129
SPECIFICATIONS

This project consists of the installation of a new parking lot & courtyard for Slidell City Hall to be located at Bouscaren Street in Slidell, Louisiana.  The project consists of removing the existing concrete parking lot, excavating the base a minimum of 24” and replacing with a minimum of 24” of granular fill, placement of 6” thick concrete parking & courtyard, pond, landscaping, irrigation system and drainage.  All work shall take place on Bouscaren Drive.  A description of pay items, including measurement and payment is given below:

Mobilization

This work consists of preparatory work, and operations, including those necessary for the movement of personnel, equipment, supplies, and incidentals to the project site, the cost of bonds and any other required insurance, and other pre-construction expenses necessary for the start of the work.
Removal and Disposal of Concrete Pavement
This work consists of the removal and disposal of existing concrete roadway materials.  The accepted quantities of pavement removed shall be paid for at the contract unit price per square yard.  Bid prices shall include all costs for labor, equipment, and materials necessary to remove existing pavement.  Proper disposal of excavated material shall be included in this pay item.
General Excavation
The contractor shall furnish all materials, labor, and equipment necessary to remove all earth, rock, water, debris, tree stumps and other materials to the extent required by the Engineer in preparation for concrete pavement and courtyard area base.  Excavation will be paid per cubic yard by net section.  Disposal of excavated material shall be included in this pay item.  Responsibility for disposal of all excavated material to satisfy all local, state, and federal regulation rests with the Contractor.

Granular Fill
Specifications for bedding and backfill shall be non-plastic and siliceous material.
Granular material shall conform to the following requirements:

Liquid limit (maximum)

25% (percent)


Plasticity index (max)


6% (percent)
U.S. Sieve


Metric Sieve


Percent Passing
½ inch



12.5 mm


100

No. 10



2.0 mm


75-100

No. 200


75 um



0-10
Granular material will be paid for per cubic yard by net section.  The bid price shall include all costs for material, labor, equipment, hauling, placing, spreading, compacting and grading for this item.  Granular fill shall be uniformly compacted to at least 95% modified proctor of maximum dry weight density.
See Stratum Engineering Geotechnical Engineering Report Dated 01/17/2013 for compaction requirements.

Parking Concrete Mix Design
Mix for one cubic yard of fiber-reinforced concrete.


28 DAY STRENGTH




4000 PSI
CEMENT (ASTM C-150, TYPE I/II)

4.64 SACKS (436 LBS.)

FLY ASH (ASTM C-618)



1.16 SACKS (109 LBS)

GRAVEL (ASTM C-33, GRADE A)


1775 LBS.

SAND (ASTM C-33)




1226 LBS.

WATER (POTABLE)




30 GALLONS (250 LBS.)

TYPE A WATER REDUCER (ASTM C-494)
16.35 LBS.

AIR ENTRAINMENT
5% BY VOLUME, USE PER MANUFACTURERS SPECIFICATIONS

FIBER REINFORCEMENT
1.5 LBS/CY MICROFIBERS, AS SPECIFIED BELOW

THICKNESS
SIX (6) INCHES THICK

Handicap Ramps
Contractor shall furnish all materials, labor, and equipment needed to construct handicap ramps as shown on the plans.  The concrete used to construct the ramps shall be four (4”) inches thick 4000-PSI at 28 days.  Truncated dome tile shall be minimum ¼” thick with embedded truncated domes, 3-inches on center through entire length of tile.  Tile surface shall be covered with protective plastic sheeting.  Each handicap ramp shall consist of only one tile, which shall be anchored into the concrete.  Color of tile shall be brick red, unless otherwise specified.  Removal of existing pavement, disposal of excavated material, curb, sub-base, base material, and sidewalk associated with the handicap ramp, as required, shall be paid under the appropriate other pay items.

Payment for handicap ramp shall be measured per each and shall include truncated dome tile with anchoring system.
Sidewalk Notes

1.  Sidewalks shall be located as noted and verified by the engineer in the field.

2. All sidewalks shall be constructed of Portland Cement concrete and have a minimum thickness of 4”.

3. Sand base course (typ.) shall have a minimum of 6” thick compacted to 95% standard proctor.

4. All exposed aggregate sidewalks shall be reinforced with 6x6 welded wire fabric, supported such that it is mid depth in the finish slab.  Solid brick pavers shall be St. joe or approved qual.  Color of brick pavers and exposed aggregate surface shall match existing sidewalks.

5. Elevations shown are M.S.L.

6. Field verify all elevations and drainage system placement prior to start of work.

Traffic Control
The section of roadway shown to be replaced on the site map may be closed to through traffic during construction.  A traffic control plan shall be submitted and approved prior to notice to proceed.  At least two weeks prior to closure, the Contractor shall submit a request to the Slidell Engineering office and the City will issue a press release.  The request must include the date of and length of closure.  Also, signage giving clear notification will be required at each entrance off Highway 433, at minimum.  Measurement and payment for traffic control, including this signage, will be lump sum.
The Slidell Engineering office must be contacted at least 24 hours prior to required inspections.

[image: image1.emf]
[image: image2.emf]
Page 1 of 8
Page 7 of 8

