September 27, 2006

TO:
All Bidders

RE:
Roof Repairs

Slidell City Court

501 Bouscaren St.

Slidell, LA  70458

The office building for Slidell City Court has approximately 800 s.f. of flat roof between the south courthouse wall and the north side of the administrative building roof, above the administration offices and along the rear of the court house. Rainwater is entering through the flat roof and into the rear hallway along the east wall.  Two roof coverings exist on the flat roof.  (1) a top torch down roof and (2) below that a granular covered asphalt based roofing.  The torch down roof is loose and water enters the building via the area between the two coatings.

This project consists of removing the old flat roof and insulation and installing a new flat roof and new insulation with white single ply reinforced thermoplastic polyolefin based roofing (TPO) with a minimum thickness of sixty mills. 

The project will entail removal of the shingle roof up three layers from the drip edge on the north side of the administrative building, removal of the metal siding on the south wall of the courthouse, removal of the parapet metal cap along the north and west sides of the flat roof of the court house, and removal of the gutter and down spouts along the east side of the flat roof.  It will also require temporarily raising the A/C unit while installing the new work.

The new single ply roof system will wrap up and on to the roof of the administrative building, with the new shingles on the administrative building, over the single ply flat roof.  The roof system will then wrap up the south wall of the court house two feet above and behind the bottom of the metal siding and over the parapet wall.  Reinstall the metal siding on the south wall of the courthouse and reinstall the metal cap along the north and west parapet wall.  A second layer of reversed TPO, gray side up, is to be installed under the A/C unit and its supports.  A thirty-six inch wide walkway leading to the rear of the building and over to the area above the rear door of the court house shall be installed using reversed TPO. The A/C supports shall be replaced with new treated 4X4 lumber glued to the roof.

The roofing system will be installed according to all governing local, state and national codes, and the manufacturer’s installation specifications.

The roof system should be warranted for a minimum of fifteen years.

Use the manufacturer’s approved adhesive, cleaners and sealants.

Protection of the work area during inclement weather is required.

Daily site clean up and removal of construction debris and new materials is required.

To accommodate scheduling of the court, weekends may be required.

Contractor shall have proof of workman’s comp. and general liability insurance.

All work shall be completed within thirty (30) days of Notice to Proceed.

Please submit all proposals to Dammon Engineering by Monday, October 30, 2006 at 1:00 p.m.
Sincerely,

Emmett G. (Pete) Dammon, PE

