August 7, 2006

TO:
All Potential Bidders
RE:
Roof Repairs

Slidell City Court

501 Bouscaren St.

Slidell, LA  70458
Ladies and Gentlemen:
The office building for Slidell City Court has an equipment deck on top of the roof, surrounded by a parapet wall, that has one scupper to drain the deck.  Water now builds up on the equipment deck, forcing water up the wall of the parapet to a point that it leaks into the building.

This project consists of   fabricating two new sheet metal scuppers to match the existing  & installing one each in the north & south parapet walls, patching  and caulking the new work as required.

The scupper body shall be a minimum of 18 gauge galvanized steel.
Sealants shall be marine rated.

Roof patching material must be equal to the existing roofing material.

Contractor shall have workman’s comp. insurance for employees working on roof.

Please submit all proposals to Dammon Engineering by August 15, 2006.

Sincerely,

Emmett G. (Pete) Dammon, PE

