COUNTRY GARDENS


SECTION 15410
PLUMBING FIXTURES
Copyright 2003 and 2005 by The American Institute of Architects (AIA)

Exclusively published and distributed by Architectural Computer Services, Inc. (ARCOM) for the AIA

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

PART 1 -  GENERAL

1.1 RELATED DOCUMENTS

A. Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification Sections, apply to this Section.

1.2 SUMMARY

A. This Section includes the following conventional plumbing fixtures and related components:

Adjust list below to suit Project.

1. Faucets for lavatories and sinks.

2. Laminar-flow faucet-spout outlets.

3. Flushometers.

4. Toilet seats.

5. Protective shielding guards.

6. Fixture supports.

7. Interceptors.

8. Water closets.

9. Lavatories.

10. Commercial sinks.

11. Owner-furnished fixtures.

1.3 DEFINITIONS

Retain abbreviations and terms that remain after this Section has been edited.

A. ABS:  Acrylonitrile-butadiene-styrene plastic.

B. Accessible Fixture:  Plumbing fixture that can be approached, entered, and used by people with disabilities.

C. Cast Polymer:  Cast-filled-polymer-plastic material.  This material includes cultured-marble and solid-surface materials.

D. Cultured Marble:  Cast-filled-polymer-plastic material with surface coating.

E. Fitting:  Device that controls the flow of water into or out of the plumbing fixture.  Fittings specified in this Section include supplies and stops, faucets and spouts, shower heads and tub spouts, drains and tailpieces, and traps and waste pipes.  Piping and general-duty valves are included where indicated.

F. FRP:  Fiberglass-reinforced plastic.

G. PMMA:  Polymethyl methacrylate (acrylic) plastic.

H. PVC:  Polyvinyl chloride plastic.

I. Solid Surface:  Nonporous, homogeneous, cast-polymer-plastic material with heat-, impact-, scratch-, and stain-resistance qualities.

1.4 SUBMITTALS

A. Product Data:  For each type of plumbing fixture indicated.  Include selected fixture and trim, fittings, accessories, appliances, appurtenances, equipment, and supports.  Indicate materials and finishes, dimensions, construction details, and flow-control rates.

B. Operation and Maintenance Data:  For plumbing fixtures to include in emergency, operation, and maintenance manuals.

C. Warranty:  Special warranty specified in this Section.

1.5 QUALITY ASSURANCE

Revise first paragraph and subparagraph below to suit manufacturers and types of fixtures specified.  No manufacturer makes all types of fixtures, faucets, etc.

A. Source Limitations:  Obtain plumbing fixtures, faucets, and other components of each category through one source from a single manufacturer.

1. Exception:  If fixtures, faucets, or other components are not available from a single manufacturer, obtain similar products from other manufacturers specified for that category.

B. Electrical Components, Devices, and Accessories:  Listed and labeled as defined in NFPA 70, Article 100, by a testing agency acceptable to authorities having jurisdiction, and marked for intended use.

Retain option in first paragraph below if required by authorities having jurisdiction.

C. Regulatory Requirements:  Comply with requirements in ICC A117.1, "Accessible and Usable Buildings and Facilities"; Public Law 90-480, "Architectural Barriers Act"; and Public Law 101-336, "Americans with Disabilities Act"; for plumbing fixtures for people with disabilities.

D. Regulatory Requirements:  Comply with requirements in Public Law 102-486, "Energy Policy Act," about water flow and consumption rates for plumbing fixtures.

E. NSF Standard:  Comply with NSF 61, "Drinking Water System Components--Health Effects," for fixture materials that will be in contact with potable water.

F. Select combinations of fixtures and trim, faucets, fittings, and other components that are compatible.

G. Comply with the following applicable standards and other requirements specified for plumbing fixtures:

Retain only subparagraphs below that match fixtures, components, and features specified.

1. Enameled, Cast-Iron Fixtures:  ASME A112.19.1M.

2. Porcelain-Enameled, Formed-Steel Fixtures:  ASME A112.19.4M.

3. Slip-Resistant Bathing Surfaces:  ASTM F 462.

4. Solid-Surface-Material Lavatories and Sinks:  ANSI/ICPA SS-1.

5. Stainless-Steel Commercial, Handwash Sinks:  NSF 2 construction.

6. Vitreous-China Fixtures:  ASME A112.19.2M.

7. Water-Closet, Flush Valve, Tank Trim:  ASME A112.19.5.

8. Water-Closet, Flushometer Tank Trim:  ASSE 1037.

H. Comply with the following applicable standards and other requirements specified for lavatory sink faucets:

1. Faucets:  ASME A112.18.1.

2. Hose-Connection Vacuum Breakers:  ASSE 1011.

3. Hose-Coupling Threads:  ASME B1.20.7.

4. Integral, Atmospheric Vacuum Breakers:  ASSE 1001.

5. NSF Potable-Water Materials:  NSF 61.

6. Pipe Threads:  ASME B1.20.1.

7. Sensor-Actuated Faucets and Electrical Devices:  UL 1951.

8. Supply Fittings:  ASME A112.18.1.

9. Brass Waste Fittings:  ASME A112.18.2.

I. Comply with the following applicable standards and other requirements specified for miscellaneous fittings:

Retain only subparagraphs below that match fittings and features specified.

1. Atmospheric Vacuum Breakers:  ASSE 1001.

2. Brass and Copper Supplies:  ASME A112.18.1.

3. Dishwasher Air-Gap Fittings:  ASSE 1021.

4. Manual-Operation Flushometers:  ASSE 1037.

5. Plastic Tubular Fittings:  ASTM F 409.

6. Brass Waste Fittings:  ASME A112.18.2.

7. Sensor-Operation Flushometers:  ASSE 1037 and UL 1951.

J. Comply with the following applicable standards and other requirements specified for miscellaneous components:

1. Flexible Water Connectors:  ASME A112.18.6.

2. Floor Drains:  ASME A112.6.3.

3. Grab Bars:  ASTM F 446.

4. Hose-Coupling Threads:  ASME B1.20.7.

5. Off-Floor Fixture Supports:  ASME A112.6.1M.

6. Pipe Threads:  ASME B1.20.1.

7. Plastic Toilet Seats:  ANSI Z124.5.

8. Supply and Drain Protective Shielding Guards:  ICC A117.1.

1.6 WARRANTY

When warranties are required, verify with Owner's counsel that special warranties stated in this Article are not less than remedies available to Owner under prevailing local laws.  Coordinate with Division 01 Section "Product Requirements."

A. Special Warranties:  Manufacturer's standard form in which manufacturer agrees to repair or replace components of whirlpools that fail in materials or workmanship within specified warranty period.

1. Failures include, but are not limited to, the following:

a. Structural failures of unit shell.

b. Faulty operation of controls, blowers, pumps, heaters, and timers.

c. Deterioration of metals, metal finishes, and other materials beyond normal use.

Verify available warranties for units and components and insert numbers below.

2. Warranty Period for Commercial Applications: Three year(s) from date of Substantial Completion.

PART 2 -  PRODUCTS

Part 2 articles specify typical plumbing fixtures and components.  Retain and edit only those articles applicable to Project.

If applying for LEED certification, retain options in articles below that limit flow rates and usage to those used in the calculation to show water use savings.

2.1 LAVATORY/SINK FAUCETS

Copy and edit paragraph and subparagraphs in this Article for each type lavatory faucet required.

A. Lavatory Faucets: Just JWF-200-W4; Bradley S53-315, or equal
See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of first three subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

1. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain one of five lists below.

Select from list below for commercial, solid-brass faucets.

a. American Standard Companies, Inc.

b. Bradley Corporation.

c. Chicago Faucets.

d. Delta Faucet Company.

e. Eljer.

f. Elkay Manufacturing Co.

g. Fisher Manufacturing Co.

h. Grohe America, Inc.

i. Just Manufacturing Company.

j. Kohler Co.

k. Moen, Inc.

l. Royal Brass Mfg. Co.

m. Sayco; a Briggs Plumbing Products, Inc. Company.

n. Speakman Company.

o. T & S Brass and Bronze Works, Inc.

p. Zurn Plumbing Products Group; Commercial Brass Operation.

Select from list below for general-duty, solid-brass faucets.

q. American Standard Companies, Inc.

r. Brasstech Inc.; Newport Brass Div.

s. Broadway Collection.

t. Central Brass Manufacturing Company.

u. Franke Consumer Products, Inc.; Kitchen Systems Div.

v. Gerber Plumbing Fixtures LLC.

w. Geberit Manufacturing, Inc.

x. Hansgrohe Inc.

y. Hydrotek International, Inc.

z. Intersan Manufacturing Company.

aa. Pegler, Ltd.

ab. Price Pfister, Inc.

ac. Rohl LLC.

ad. Water Management, Inc.

ae. Wolverine Brass, Inc.

af. Zurn Plumbing Products Group; Commercial Brass Operation.

2. Description:  See Plans and attached Cut Sheets. 
2.2 TOILET SEATS

Copy and edit paragraph and subparagraphs in this Article for each type toilet seat required.

A. Toilet Seats:

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of first three subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

1. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain one of two lists below.

Select from list below for residential and light-commercial toilet seats.

a. American Standard Companies, Inc.

b. Bemis Manufacturing Company.

c. Centoco Manufacturing Corp.

d. Church Seats.

e. Eljer.

f. Kohler Co.

g. Olsonite Corp.

h. Sanderson Plumbing Products, Inc.; Beneke Div.

i. Sperzel.

Select from list below for standard and heavy-duty, commercial toilet seats.

j. Bemis Manufacturing Company.

k. Centoco Manufacturing Corp.

l. Church Seats.

m. Kohler Co.

n. Olsonite Corp.

o. Pressalit A/S.

p. Sanderson Plumbing Products, Inc.; Beneke Div.

q. Sperzel.

2. Description:  Toilet seat for water-closet-type fixture.

If basis-of-design product manufacturer's name, product name or designation, and description above are explicit enough, requirements in subparagraphs below may be reduced or omitted.

a. Material:  Molded, solid plastic with antimicrobial agent.

b. Configuration:  Open front with cover.

c. Size:  Elongated.

d. Hinge Type:  SC, self-sustaining, check.

e. Class:  Heavy-duty commercial.

f. Color:  White.

2.3 PROTECTIVE SHIELDING GUARDS

A. Protective Shielding Pipe Covers:

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of first two subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

1. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

a. Engineered Brass Co.

b. Insul-Tect Products Co.; a Subsidiary of MVG Molded Products.

c. McGuire Manufacturing Co., Inc.

d. Plumberex Specialty Products Inc.

e. TCI Products.

f. TRUEBRO, Inc.

g. Zurn Plumbing Products Group; Tubular Brass Plumbing Products Operation.

2. Description:  Manufactured plastic wraps for covering plumbing fixture hot- and cold-water supplies and trap and drain piping.  Comply with Americans with Disabilities Act (ADA) requirements.

B. Protective Shielding Piping Enclosures:

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of first two subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

1. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

a. TRUEBRO, Inc.

2. Description:  Manufactured plastic enclosure for covering plumbing fixture hot- and cold-water supplies and trap and drain piping.  Comply with ADA requirements.

2.4 FIXTURE SUPPORTS

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of two paragraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

A. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

1. Josam Company.

2. MIFAB Manufacturing Inc.

3. Smith, Jay R. Mfg. Co.

4. Tyler Pipe; Wade Div.

5. Watts Drainage Products Inc.; a div. of Watts Industries, Inc.

6. Zurn Plumbing Products Group; Specification Drainage Operation.

Copy and edit paragraph and subparagraph below for each type water-closet support required.

B. Water-Closet Supports:

1. Description:  Combination carrier designed for accessible mounting height of wall-mounting, water-closet-type fixture.  Include single or double, vertical or horizontal, hub-and-spigot or hubless waste fitting as required for piping arrangement; faceplates; couplings with gaskets; feet; and fixture bolts and hardware matching fixture.  Include additional extension coupling, faceplate, and feet for installation in wide pipe space.

Copy and edit paragraph and subparagraphs below for each type lavatory support required.

C. Lavatory Supports,:

1. Description:  Type III, lavatory carrier with hanger plate and tie rod for wall-mounting, lavatory-type fixture.  Include steel uprights with feet.

2. Accessible-Fixture Support:  Include rectangular steel uprights.

Copy and edit paragraph and subparagraph below for each type sink support required.

2.5 INTERCEPTORS

See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of first two paragraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

A. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

1. Josam Company.

2. MIFAB Manufacturing Inc.

3. Smith, Jay R. Mfg. Co.

4. Tyler Pipe; Wade Div.

5. Watts Drainage Products Inc.; a div. of Watts Industries, Inc.

6. Zurn Plumbing Products Group; Specification Drainage Operation.

2.6 WATER CLOSETS

Copy and edit paragraph and subparagraphs below for each type wall-mounting, back-outlet water closet required.

A. Water Closets: Kohler Cimarron K-3589, or equal.
See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of first three subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

1. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain one of four lists below.

Select from list below for gravity-tank-type, 1.6-gal./flush (6-L/flush) water closets.

a. Crane Plumbing, L.L.C./Fiat Products.

Select from list below for Sloan's "Flushmate" flushometer-tank-type, 1.6-gal./flush (6-L/flush) water closets.

b. American Standard Companies, Inc.

Select from list below for flushometer-valve-type, 1.6-gal./flush (6-L/flush) water closets.

c. American Standard Companies, Inc.

d. Briggs Plumbing Products, Inc.

e. Capizzi.

f. Crane Plumbing, L.L.C./Fiat Products.

g. Eljer.

h. Kohler Co.

i. St. Thomas Creations.

j. TOTO USA, Inc.

Select from list below for flushometer-valve-type, 3.5-gal./flush (13.3-L/flush) water closets.

k. American Standard Companies, Inc.

l. Crane Plumbing, L.L.C./Fiat Products.

m. Eljer.

n. Kohler Co.

2. Description-  See Plans and Attached Cut Sheets.

If basis-of-design product manufacturer's name, product name or designation, and description above are explicit enough, requirements in six subparagraphs and associated subparagraphs below may be reduced or omitted.

2.7 LAVATORIES/SINKS
Copy and edit paragraph and subparagraphs below for each type wall-mounting lavatory required.

A. Lavatories: Just SL-Series-A-18 GA; Zurn Z5360-PED, or equal.
See Editing Instruction No. 1 in the Evaluations for cautions about naming manufacturers and products.  Retain one of first three subparagraphs and list of manufacturers below.  See Division 01 Section "Product Requirements."

1. Available Manufacturers:  Subject to compliance with requirements, manufacturers offering products that may be incorporated into the Work include, but are not limited to, the following:

Retain one of two lists below.

Select from list below for enameled, cast-iron lavatories.

a. American Standard Companies, Inc.

b. Commercial Enameling Company.

c. Eljer.

d. Kohler Co.

Select from list below for vitreous-china lavatories.

e. American Standard Companies, Inc.

f. Barclay Products, Ltd.

g. Briggs Plumbing Products, Inc.

h. Crane Plumbing, L.L.C./Fiat Products.

i. Eljer.

j. Gerber Plumbing Fixtures LLC.

k. Kohler Co.

l. Mansfield Plumbing Products, Inc.

m. Peerless Pottery, Inc.

n. Sterling Plumbing Group, Inc.

o. St. Thomas Creations.

p. TOTO USA, Inc.

2. Description:  See Plans and Attached Cut Sheets.

If basis-of-design product manufacturer's name, product name or designation, and description above are explicit enough, requirements in 14 subparagraphs and associated subparagraphs below may be reduced or omitted.

Copy and edit paragraph and subparagraphs below for each type counter-mounting lavatory required.

PART 3 -  EXECUTION

3.1 EXAMINATION

A. Examine roughing-in of water supply and sanitary drainage and vent piping systems to verify actual locations of piping connections before plumbing fixture installation.

B. Examine cabinets, counters, floors, and walls for suitable conditions where fixtures will be installed.

C. Proceed with installation only after unsatisfactory conditions have been corrected.

3.2 INSTALLATION

A. Assemble plumbing fixtures, trim, fittings, and other components according to manufacturers' written instructions.

B. Install off-floor supports, affixed to building substrate, for wall-mounting fixtures.

1. Use carrier supports with waste fitting and seal for back-outlet fixtures.

2. Use carrier supports without waste fitting for fixtures with tubular waste piping.

3. Use chair-type carrier supports with rectangular steel uprights for accessible fixtures.

First paragraph below applies to wall-mounting fixtures such as water closets and urinals.

C. Install back-outlet, wall-mounting fixtures onto waste fitting seals and attach to supports.

D. Install floor-mounting fixtures on closet flanges or other attachments to piping or building substrate.

First paragraph below applies to wall-mounting fixtures such as lavatories and most sinks.

E. Install wall-mounting fixtures with tubular waste piping attached to supports.

F. Install floor-mounting, back-outlet water closets attached to building floor substrate and wall bracket and onto waste fitting seals.

G. Install counter-mounting fixtures in and attached to casework.

H. Install fixtures level and plumb according to roughing-in drawings.

I. Install water-supply piping with stop on each supply to each fixture to be connected to water distribution piping.  Attach supplies to supports or substrate within pipe spaces behind fixtures.  Install stops in locations where they can be easily reached for operation.

1. Exception:  Use ball, gate, or globe valves if supply stops are not specified with fixture.  

J. Install trap and tubular waste piping on drain outlet of each fixture to be directly connected to sanitary drainage system.

K. Install tubular waste piping on drain outlet of each fixture to be indirectly connected to drainage system.

L. Install flushometer valves for accessible water closets and urinals with handle mounted on wide side of compartment.  Install other actuators in locations that are easy for people with disabilities to reach.

M. Install tanks for accessible, tank-type water closets with lever handle mounted on wide side of compartment.

N. Install toilet seats on water closets.

O. Install water-supply flow-control fittings with specified flow rates in fixture supplies at stop valves.

P. Install faucet flow-control fittings with specified flow rates and patterns in faucet spouts if faucets are not available with required rates and patterns.  Include adapters if required.

Q. Install traps on fixture outlets.

1. Exception:  Omit trap on fixtures with integral traps.

2. Exception:  Omit trap on indirect wastes, unless otherwise indicated.

R. Install escutcheons at piping wall ceiling penetrations in exposed, finished locations and within cabinets and millwork.  Use deep-pattern escutcheons if required to conceal protruding fittings.  

Delete paragraph below if sealants are specified in Division 07 Section "Joint Sealants."

3.3 Seal joints between fixtures and walls, floors, and countertops using sanitary-type, one-part, mildew-resistant silicone sealant.  Match sealant color to fixture color.  

3.4 CONNECTIONS
Coordinate piping installations and specialty arrangements with schematics on Drawings and with requirements specified in piping systems.  If Drawings are explicit enough, these requirements may be reduced or omitted.

A. Drawings indicate general arrangement of piping, fittings, and specialties.

B. Connect fixtures with water supplies, stops, and risers, and with traps, soil, waste, and vent piping.  Use size fittings required to match fixtures.

3.5 FIELD QUALITY CONTROL

A. Verify that installed plumbing fixtures are categories and types specified for locations where installed.

B. Check that plumbing fixtures are complete with trim, faucets, fittings, and other specified components.

C. Inspect installed plumbing fixtures for damage.  Replace damaged fixtures and components.

D. Test installed fixtures after water systems are pressurized for proper operation.  Replace malfunctioning fixtures and components, then retest.  Repeat procedure until units operate properly.

E. Install fresh batteries in sensor-operated mechanisms.

3.6 ADJUSTING

A. Operate and adjust faucets and controls.  Replace damaged and malfunctioning fixtures, fittings, and controls.

Delete fixture types in first paragraph below not in Project.

B. Adjust water pressure at faucets and flushometer valves to produce proper flow and stream.

C. Replace washers and seals of leaking and dripping faucets and stops.

D. Install fresh batteries in sensor-operated mechanisms.

3.7 CLEANING

A. Clean fixtures, faucets, and other fittings with manufacturers' recommended cleaning methods and materials.  Do the following:

1. Remove faucet spouts and strainers, remove sediment and debris, and reinstall strainers and spouts.

2. Remove sediment and debris from drains.

B. After completing installation of exposed, factory-finished fixtures, faucets, and fittings, inspect exposed finishes and repair damaged finishes.

3.8 PROTECTION

A. Provide protective covering for installed fixtures and fittings.

B. Do not allow use of plumbing fixtures for temporary facilities unless approved in writing by Owner.

END OF DOCUMENT
PLUMBING FIXTURES

15410 - 10

