Revised   9-27-2013

SLIDELL MEMORIAL GENERAL RELOCATION SCOPE OF WORK

- Alterations to existing HVAC as needed to accommodate new walls,

Replace/install all diffusers with new.

- Modify existing Fire Alarm.

- Modify existing phone system.

- Modify sprinkler head locations as needed to accommodate new layout.

- Demo scope identified in demolition plan.

- New walls shown hatched solid.

- Half walls shown hatched with diagonal lines.

- Install Hospital Corner guards at corridor corners.

- Replace all light fixtures with new T-8 fixtures.

- New Signage for each department.

- Install new 120/208V 225Amp 42 pole power panel and 75KVA transformer to support Cardio.

- Remove all existing flooring for new flooring scheme.

- Relocate all furniture, equipment and fixtures per SMH direction.

- Add power & Data outlets as noted on plans.

- Reuse all existing doors, prior to installation inspect and refinish.

Note: This project requires phasing with occupancy at the completion of each phase. There will be a 1 week delay between the completion of phase 1 and the beginning of phase 2 to allow for movement of furniture and personnel.  There will be a 1 week delay between the completion of phase 2 and the beginning of phase 3 to allow for movement of furniture and personnel.

101 EXIST. UNISEX TOILET

- Relocate existing door and infill existing door opening, wall to 12” above ceiling.

- Replace existing ceiling grid with 2’x2’ grid.

- Replace ceiling tiles.

- Install new light fixtures.

- Install new air diffusers.

- Demo existing shower.

- Install new sheet vinyl flooring.

- Paint walls to SMH color scheme.

- Install new ADA grab bars.

102 New Closet  (LOCATED AT OLD SHOWER)

- Construct new wall to 12” above ceiling.

- Install new door

- Install new ceiling grid and tiles

- Install new sheet vinyl flooring.

- Paint walls to SMH color scheme.

- Demo Existing Shower.

103 EXIST. UNISEX TOILET

- Replace existing ceiling grid with 2’x2’ grid.

- Replace ceiling tiles.

- Install new light fixtures.

- Install new air diffusers.

- Demo existing shower.

- Install new sheet vinyl flooring.

- Paint walls to SMH color scheme.

- Install new ADA grab bars.

104 New Closet

- Construct new walls to 12” above ceiling.

- Install new door.

- Install new ceiling grid and tiles.

- Paint walls to SMH color scheme.

- Install new sheet vinyl flooring.

105 CARDIO REHAB OPEN AREA

- Replace existing ceiling grid with 2’x2’ grid to accommodate new walls/layout.

- Replace ceiling tiles.

- Relocate existing door with offset to Main Corridor per plans. Same locking hardware to remain.

- New Cubbies millwork and new counter top w/closed base cabinet.

- New dedicated outlets.

- Paint walls to SMH color scheme.

- New NORA flooring as identified on dwg.

- Relocate existing fixtures/furniture to new area per SMH direction.

- Add electrical outlets at ceiling height for fans.

- Relocate existing equipment to new area. (Lots of exercise equipment)

- Add electrical outlets for workout equipment.

- Add water line for drinking fountain.

- Install new light fixtures.

- Install new air diffusers.

106 CARDIO WAITING AREA

- Reduce full height wall in waiting area to 48” with wood cap.

- Install new glass door w/ sidelite, enlarge opening to accommodate new door.

- Relocate existing door, enlarge opening to match plans.

- Replace existing ceiling grid with 2’x2’ grid to accommodate new walls/layout.

- Replace ceiling tiles.

- Install new exit signs.

- Install new air diffusers.

- Paint walls to SMH color scheme.

- New carpet to match SMH standard.

107 CARDIO NURSE’S STATION

- New base cabinet with countertop and upper cabinet with storage.

- New Bar Height Counter.

- Paint walls to SMH color scheme.

- New NORA flooring.

108 CONSULTATION

- Relocate existing door to accommodate offset entrance to Main Corridor per plans.

- Replace existing ceiling grid with 2’x2’ grid to accommodate new walls/layout.

- Replace ceiling tiles.

- Demo existing wall per demo plan.

- Demo existing pre-action sprinkler system for MRI room (Dry System).

- Install new air diffusers.

- Install new lights.

- Paint walls to SMH color scheme.

- New carpet to match SMH standard.

- Relocate all furniture and fixtures per SMH direction.

109 EDUCATION CONFERENCE ROOM

- Demo existing shielding from MRI room.

- Repair/patch existing void in slab.

- Demo existing shielded door.

- Demo all existing electrical/mechanical piping from MRI equipment and dedicated HVAC system.

- Demo all existing hot & cold water plumbing from Science Room.

- Demo all existing walls per demo plans.

- Paint new walls to SMH color scheme.

- New carpet to match SMH standard.

- Replace existing ceiling grid with 2’x2’ grid to accommodate new walls/layout.

- Replace ceiling tiles.

- Install new lights.

- New TV monitor area: with full base cabinets and cabinets on each side of the 40” TV.

- New kitchen cabinet with Corian counter top with built-in sink and upper cabinets. Dedicated electrical outlets for a coffee pot and microwave.

- Current dry sprinkler system will be converted to wet system.

110 EMPLOYEE LOCKERS

- Demo existing door, shelving and non associated equipment.

- Replace existing ceiling grid with 2’x2’ grid to accommodate new walls/layout.

- Replace ceiling tiles.

- Install new lights.

- Paint new walls to SMH color scheme.

- New carpet to match SMH standard.

111 CPR OFFICE

- Demo existing shielding on wall and door to restroom.

- Infill opening of existing door to restroom w/ wall to 12” above ceiling.

- Replace existing ceiling grid with 2’x2’ grid to accommodate new walls/layout.

- Replace ceiling tiles.

- Install new lights.

- Paint walls to match SMH color scheme.

- New carpet to match SMH standard.

112 EXIST. EMPLOYEE TOILET

- Replace existing ceiling grid with 2’x2’ grid to accommodate new walls/layout.

- Replace ceiling tiles.

- Infill opening of existing door w/ wall to 12” above ceiling.

- New ADA grab bars.

- Paint walls to match SMH color scheme.

- Install new sheet vinyl flooring.

- Install new light fixtures.

- Install new air diffusers.

113 CPR ORIENTATION ROOM

- Replace existing ceiling grid with 2’x2’ grid to accommodate new walls/layout.

- Replace ceiling tiles.

- Install new light fixtures.

- Install new air diffusers.

- Paint walls to match SMH color scheme.

- New carpet to match SMH standard.

114 – 118 INTERVIEW ROOM  (5)

- Replace existing ceiling grid with 2’x2’ grid to accommodate new walls/layout.

- Replace ceiling tiles.

- New walls to 12” above ceiling.

- (5) New door and hardware.

- Infill opening of existing door w/ wall to 12” above ceiling.

- Install new light fixtures.

- Install new air diffusers.

- Paint walls to match SMH color scheme.

- New carpet to match SMH standard.

119 ARCHIVE ROOM

- Demo two doors and install one new door and hardware to corridor.

- Replace existing ceiling grid with 2’x2’ grid to accommodate new walls/layout.

- Replace ceiling tiles.

- New walls to 12” above ceiling.

- Install new light fixtures.

- Install new air diffusers.

- Paint walls to match SMH color scheme.

- New carpet to match SMH standard.

120 – 122 EXISTING INFUSION ROOM (3)

NOT IN CONTRACT

123- 124 ADA UNISEX TOILET NEAR INFUSION ROOM (2)

- Replace existing ceiling grid with 2’x2’ grid to accommodate new walls/layout.

- Replace ceiling tiles.

- Install new light fixtures.

- Install new air diffusers.

- Paint walls to match SMH color scheme.

- New sheet vinyl to match SMH standard.

125 EXIST. PATIENT WAITING AREA

- New glass door and locking hardware.

- Replace existing ceiling grid with 2’x2’ grid to accommodate new walls/layout.

- Replace ceiling tiles.

- Install new light fixtures and exit signs.

- Install new air diffusers.

- Paint walls to match SMH color scheme.

- New carpet to match SMH standard.

- Keep existing door bell & speaker system.

126 EXIST. RECEPTION

- Replace existing ceiling grid with 2’x2’ grid to accommodate new walls/layout.

- Replace ceiling tiles.

- Install new light fixtures.

- Install new air diffusers.

- Paint walls to match SMH color scheme.

- New carpet to match SMH standard.

127 CORRIDOR NEAR INFUSION ROOMS TO NEW CARDIO REHAB

- Relocate existing door to Main Corridor with offset per plans. Same locking hardware to remain.

- Install new glass door at end of corridor entering Cardio room.

- Replace existing ceiling grid with 2’x2’ grid to accommodate new walls/layout.

- Replace ceiling tiles.

- Install new light fixtures and exit signs.

- Install new air diffusers.

- Paint walls to match SMH color scheme.

- New sheet vinyl to match SMH standard.

128 MAIN CORRIDOR AT NEW OFFSET DOORS

- Repair ceiling grid to accommodate new layout.

- Install new ceiling tiles to match existing.

- New paint in offset to match SMH color scheme.

- New Sheet vinyl to match SMH standard.