La Quinta 
Design and Construction Standards


[image: image1.jpg]LaQuinta

Inns - Inn & Suites





Design and Construction Standards

Date: July 28, 2003







THESE MATERIALS MAY NOT BE COPIED

OR DUPLICATED IN ANY FORMAT FOR ANY PURPOSE.


This manual is the proprietary property of La Quinta and may not be copied or distributed without its consent.

You may not lend, lease, sell, convey or otherwise transfer or share this manual with any other person, corporation or entity.

La Quinta is a registered trademark of La Quinta Corporation or its affiliates.  All rights reserved
TABLE of CONTENTS

201 Introduction

201.1 Project Design Process- New Construction

201.2 New Construction

201.02.1 Design of New Facilities
201.02.2 Submittals
201.02.3 Construction Phase
201.3 Project Design Process- Conversions

201.4 Assistance

202 General Design Criteria

202.1 Building Codes 

202.2 Americans with Disabilities Act

202.3 Life Safety Requirements

202.4 Fire Detection, Alarm and Suppression

203 Site Design and Construction

203.1 General 

203.2 Entrance Drives

203.3 Parking Areas

203.4 Parking Garages and Covered Parking

203.5 Sidewalks

203.6 Connections Between Buildings

203.7 Site and Building Lighting

203.8 Site Signage

203.9 Service Area

203.10 Fences

203.11 Landscaping

204 Swimming Pool and Spa

204.1 Design

204.2 Signage

204.3 Deck and Coping

204.4 Enclosure

204.5 Equipment

204.6 Carbon Monoxide Detection

204.7 Lighting

204.8 Indoor Pools and Spas

205 Building Exterior

205.1 Porte Cochere

205.2 Branding Elements

205.3 Building Decor

205.4 Building Signage

205.5 Windows and Sliding Glass Doors

205.6 Railings

206 Interior Public Spaces

206.1 General

206.2 La Quinta Inns

206.3 La Quinta Inn and Suites 

206.4 Entrance Doors

206.5 Lobby

206.6 Public Area Floors

206.7 Public Area Walls

206.8 Public Area Ceilings

206.9  Public Area Lighting

206.10 Public Area Artwork

206.11 Public Area Furnishings

206.12 Public Area Window Treatment

206.13 Front Desk

206.14 Continental Breakfast Room

206.15 Elevators

206.16 Elevator Lobbies

206.17 Amenities 

206.017.1 Meeting Rooms

206.017.2 Business Center

206.017.3 Exercise Room

206.017.4 Telephones 

206.017.5 Public Toilets

206.017.6 Vending 

206.18 Guest Corridors 

206.19 Room Numbers

207 Guest Rooms

207.1 La Quinta Inns

207.2 La Quinta Inn and Suites 

207.3 Closets

207.4 Décor

207.5 Minimum Size and Configuration

207.6 Acoustics

207.7 Entrance Doors

207.8 Connecting Room Doors 

207.9 Floor Covering

207.10 Wall Finish

207.11 Ceiling Finish

207.12 Electrical

207.13 Environmental Controls

207.14 Drapery

207.15 Artwork
207.16 Lighting
207.17 Microwave / Refrigerators
207.18  Telephones and Data Terminals

208 Guest Baths

208.1 Size and Configuration

208.2 Floor

208.3 Walls

208.4 Ceiling

208.5 Vanity

208.6 Plumbing Fixtures

208.7 Tub Surrounds

208.8 Lighting

208.9 Electrical

208.10 Exhaust System

208.11 Bath Door

208.12 Mirrors

208.13 Bath Accessories

209 Support Areas and Systems

209.1 Electrical Panel Boxes/ Circuits

209.2 Emergency Lighting

209.3 Property Management System

209.03.1 Manager’s Office

209.03.2 Mainframe System

209.03.3 Front and Back Office Requirements

209.03.4 Telephone Requirements

209.4 Employee Break Room

209.5 Environmental Controls

209.6 Hot Water Supply

209.7 Laundry/Housekeeping Area

209.8 Mechanical Rooms/ Electrical Rooms/ Elevator Equipment Rooms

209.9 Storage Rooms

209.10 Trash Collection

209.11 TV/ Video Equipment Rooms

210 Appendix

210.1 Approved Décor Packages

210.01.1 
210.01.2 
210.01.3 
210.01.4 
210.01.5 
210.2 Finish Materials

210.3 Paint Specifications
210.4 Carbon Monoxide Project
210.5 Selected Room Floor Plans

210.05.1 Inn Standard Double/ Double Room

210.05.2 Inn Standard Single King Room

210.05.3 Inn & Suites Standard Double/ Double Room

210.05.4 Inn & Suites Standard Single King Room

210.05.5 Inn & Suites Sofa Single King Room

210.05.6 Inn & Suites Extended King Suite

210.05.7 Inn & Suites Two Room King Suite

210.05.8 Fixtures, Furniture and Equipment Schedule

210.6 Design Standard Drawings

210.06.1 Front Desk and Breakfast Buffet Elevation

210.06.2 Front Desk Sections

210.06.3 Breakfast Buffet Section

210.06.4 Closet Details

210.06.5 Six Panel Door Molding Layout

210.06.6 Typical Tower Elevation

210.06.7 Tower Trim

210.06.8 Porte-Cochere Column Detail

210.06.9 Column Base Trim

210.06.10 Column Crown Detail
210.06.11 Elevator Detail

210.06.12 Elevator Detail
210.7 Preferred Vender List

210.8 Signage
201 Introduction

La Quinta Lodging Facilities are high quality, mid-priced, limited service lodging facilities, designed to appeal to both business and leisure travelers.  Each and every aspect of the interior and exterior appearance, layout and design is important to La Quinta. The Design and Construction Standards apply to all hotels entering the La Quinta system through new construction or as a conversion. Should these standards be in conflict with Federal, State, or Local requirements, then the more stringent shall apply.

201.01 Project Design Process - New Construction

La Quinta has developed four prototypes that offer flexibility and efficiency of design.  These prototype designs must be used for all newly constructed facilities, unless La Quinta approves otherwise.  The number of rooms can be adjusted to meet specific planning or economic needs.  We will consider modifications to public and back of house areas based on market demands or operational preferences.  Guest room layout and exterior building elevations can be adjusted to accommodate different room counts.  Typical room counts range from approximately 65 to 106 rooms. As part of the review process La Quinta will be assuring brand consistency, design integrity, and high quality standards of the brand.  La Quinta has established quality standards for furniture, fixtures and equipment.  We have five different soft good packages to choose from.  A copy will be provided for your use.  Our Design and Construction department at La Quinta is available to assist you with your design, planning and material procurement. 
201.02
New Construction
201.01.1 

· 
· 
· 
· 
· 
· 
· 
· 

201.02.1 Design of New Facilities

 In order to assist you with preliminary site planning, La Quinta will provide prototype floor plans and sample site plans.  After creating a preliminary site plan it should be forwarded to La Quinta for approval. Upon site plan approval and acceptance of the franchisee application, La Quinta will forward electronic CADD files or hard copy documents to your design team. The documents will consist of prototype drawings that best suit your project. The documents will include architectural drawings with interior and exterior detailing. Inclusive in the documents will be mechanical, electrical and plumbing (MEP) fixture layouts for typical guest rooms and public areas. Prototype interior finish and fixture schedules will be supplied.

We will provide the Franchisee with a prototypical set of our standard plans and specifications.  Such prototypical plans will not necessarily contain the requirements of any federal, state or local law, code or regulation, including the ADA or similar rules governing accommodations for persons with disabilities, nor will such plans contain the requirements of, or be used for, construction drawings or other documentation necessary to obtain permits or authorizations to build a specific La Quinta Lodging Facility.

It is the responsibility of the Franchisee’s Design Consultant to site adapt and complete the Construction Documents and to meet all building codes and other legal requirements and obtain governmental approvals.  The Franchisee must obtain, at its sole cost and expense, all governmental licenses and permits that are required for completion of the hotel facility in accordance with the final plans approved by La Quinta, including any required building, occupancy, sewer and utility permits.  La Quinta is not responsible for, and shall have no liability for, the architecture, design, engineering or construction of a Franchisee’s facility, for the facility’s compliance with any federal, state or local law (including the ADA and any other federal, state or local law or ordinance regulating standards for access to, use of the, or modification of buildings for and by persons whose disabilities are protected by law), for any errors, omissions or discrepancies of any nature in any drawings or specifications with respect to the facility, or for any other matter relating to the development, use or operation of the facility.

 





201.02.2 Submittals

1. Schematic Plans: Complete a Dimensional Site plan, floor plans and elevations.  Any deviation from the La Quinta prototype plans should be noted at this time.

2. 30% to 50 % Construction Documents: Architectural plans detailing building floor plans and exterior elevations, architectural, civil, MEP documents, including, 100% site plan, 100% floor plans, 100%  elevations, wall sections, enlarged public area and guestroom plans and elevations, room finish window and door schedules, structural framing plans and electrical plans showing all lighting including a complete schedule.
3. 100 % Construction Documents: Construction documents complete and ready to submit for building permit with all code compliance and city comments addressed.  The final plans shall include an interior and exterior signage plan.  Final document approval by La Quinta will be required prior to submission for building permit and start of construction.  
Documents may be submitted for review at any time the owner or architect deems necessary.
La Quinta will require a letter from the architect, engineer or licensed designer of record stating that the proposed construction:

a. Complies with all Federal, State, and Local codes and regulation. Should the proposed construction be located in an area not regulated by a building code, it shall be designed to meet the current addition of the International Building Code and stated in the letter. 

b. Complies with the Americans with Disabilities Act

c. Complies with La Quinta Design and Construction Standards
All review comments by La Quinta shall be incorporated into the construction documents prior to permit and start of construction.  No construction may commence until final plans have been approved by us, and the Franchisee has obtained building permits and otherwise complied with all applicable codes and regulations.  The Franchisee must construct the hotel facility in full compliance with the approved final plans, and no material changes may be made to the approved final plans without La Quinta’s prior consent.

201.02.3
Construction Phase

During construction, La Quinta design and construction personnel may conduct periodic on-site inspections to insure that architectural plans and quality standards are being met.  Site visits may be conducted during construction and at the final building punch-out.



1. 
2. 
a. 
b. 
c. 
d. 
e. 
f. 
g. 
h. 
i. 
j. 
k. 
l. 
m. 
3. 
4. 
5. 


2. 
3. 
4. 
5. 
6. 
a. 
b. 
c. 


201.03 Project Design Process- Conversions

If the property is a conversion of an existing hotel to a La Quinta Inn® or a La Quinta Inns & Suites®, La Quinta will prepare a Product Improvement Plan (PIP). The PIP will identify items that must be changed to meet current La Quinta Design and Construction Standards.  If changes are identified they must be remedied before opening as a La Quinta.  Where changes are required to the building, La Quinta will attempt to incorporate existing building elements so that minimal adaptations are needed.  When FF&E needs to be changed, La Quinta will require the use of one of the La Quinta décor packages.  Revisions to the existing facility may require the services of an architect, engineer or other licensed design professional.  Review of the design documents by La Quinta is required.  Additionally, any deviation from La Quinta Design and Construction Standards will require review by La Quinta prior to making revisions. Submit written request for review to:

La Quinta 



909 Hidden Ridge, Suite 600




Irving, Texas 75038




Attn: Design and Construction Department
201.04 Hotel Opening

Upon completion of construction, conversion, renovations or any expansion, the Franchisee must notify La Quinta that the facility is ready for a pre-opening inspection.  La Quinta will then conduct a pre-opening inspection to determine if the facility is completed and ready to provide lodging services to guests on a fully operational basis.  La Quinta will conduct one (1) inspection without additional charge.  If La Quinta determines that additional inspections are required, the Franchisee will be required to reimburse La Quinta for all costs and expenses thereof, including wages, travel, lodging and meals.  The Franchisee must take all measures necessary to promptly remedy any deficiencies indicated by such inspection.  Any conditional approval may be rescinded if the Franchisee fails to satisfactorily remedy any identified deficiencies.  La Quinta will issue an Opening Approval Notice, signifying that the facility may be opened for business as a La Quinta Lodging Facility if, upon completion of the pre-opening inspection, all of  La Quinta’s conditions have been met to La Quinta’s satisfaction, including:

(i) the facility (including all public areas, work areas and guest rooms) is completed (including the completion of work described in the Property Improvement Plan, if applicable); contains the number of required guest rooms and is ready to provide lodging services to guests on a fully operational basis;

(ii) the governmental authorities under whose jurisdiction the facility was constructed have issued a permanent certificate of occupancy, if required by applicable law;

(iii) all other licenses, permits and other consents required for the occupancy and operation of the facility have been obtained; and  

(iv) all deficiencies revealed in the pre-opening inspection have been corrected and the facility complies with all of our Standards.


In no event shall the facility open for business as a La Quinta Lodging Facility prior to La Quinta’s issuance of an Opening Approval Notice.
201.05 Assistance

La Quinta stands ready to assist you with your Lodging Facility needs.  Your Franchise Sales Director can put you in touch with some one that can help, or you may contact the following:

La Quinta Corporation


909 Hidden Ridge, Suite 600


Irving, Texas 75038

214-492-6600
Murry Cathlina
VP Design and Construction Services
214-492-6886

Gregory Ammon
Director of Architectural Design

214-492-6958

Rajiv Trivedi

VP Franchise Operations


214-492-6753

Jeff Palla

Director of Franchise Services

214-492-6646

Steve Williams 
Director of Franchise Services

360-666-0667

Rick Adlington
Director of Franchise Services

770-632-5878




202 General Design Criteria

202.02 Building Codes and Applicable Laws
All new Construction shall be constructed in accordance with a national model code, such as the Uniform Building Code, the Standard Building Code, the Basic/National Building, or the International Building Code.  All conversions must comply with the current building code in the jurisdiction having authority.  All properties must conform to the requirements of all laws, rules and building codes of all regulatory authorities having jurisdiction.  In the event a requirement of these Standards and any law or code conflict, the more stringent requirement shall apply.
Notwithstanding anything to the contrary contained herein, La Quinta’s exercise of its right to approve the site layout, to approve any plans, to inspect the construction of the facility and to issue the Opening Approval Notice shall be solely for the purpose of assuring compliance with La Quinta’s Standards and shall not be construed as any express or implied representation or warranty that the facility complies with any applicable laws, codes or regulations (including the ADA or any other federal, state, or local law or ordinance regulating standards for the access to, use of, or modification of buildings for and by persons whose disabilities are protected by law) or that the construction thereof is sound or free from defects.  La Quinta’s criteria for approval or disapproval do not encompass technical, architectural or engineering considerations.  La Quinta shall have no liability or obligation with respect to the design or construction of the Facility.

202.03 Americans with Disabilities Act

Analysis of the impact of Title III of the Americans with Disabilities Act of 1990 (as amended, the “ADA”) on a particular hotel is the owner's responsibility and is not contained within the scope of these standards; therefore, legal counsel, architects and/or other necessary advisors should be consulted.

In the case of new construction, the Franchisee’s architect will be required to certify to La Quinta the Franchisee’s construction documents will be or have been prepared in accordance with the ADA.  In addition, prior to opening, the Franchisee must certify to La Quinta that the Franchisee’s hotel fully complies with all ADA requirements.

In the case of a conversion property, following the completion of all conversion renovations, the Franchisee must certify to LA Quinta that the Franchisee’s hotel fully complies with all ADA requirements.
202.04 Life Safety Requirements

All properties shall comply with the National Fire Protection Association (NFPA) Standards.  Copies of NFPA Standards may be obtained from:




National Fire Protection Association




Battermarch Park




Quincy, MA  02269




1-800-344-3555




www.nfpa.org 

202.05 Fire Detection, Alarm, and Suppression Systems

A general alarm system activated by smoke and/or fire shall be provided which meets or exceeds local codes and ordinances. Fire extinguishers are required according to the current code.  In addition, fire suppression systems are recommended for all properties.  If required by code, a fire suppression system shall be installed.
	
	
	
	
	

	
	
	
	
	

	
	
	

	
	
	

	
	
	


203 Site Design and Construction

203.02 General 

Site must be attractive, with “curb appeal,” and well maintained. The new lodging facility should have good visibility with easy accessibility. It should be well positioned for business within the competitive market. A site may not be considered which would breach a predefined market radius of another La Quinta Lodging Facility.  La Quinta will review information regarding each proposed site to determine whether to accept or reject the site.  Factors that may be considered include, without limitation:

· General location and neighborhood
· Demographic information
· Traffic patterns
· Vehicular and pedestrian (if appropriate) access

· Visibility

· Location of other lodging facilities

· Size, configuration, topography, appearance and other physical characteristics of the site
· Local Covenants, Codes, and Restrictions (CCR), including zoning classification, setbacks, height restriction, and Floor Area Ratio (FAR)
It is recommended that the site be submitted for review prior to proceeding with the project design. 


203.03 Entrance Drives

Entrance drives must be constructed of concrete or asphalt with continuous concrete curbs.  Entrances must be easily recognizable and attractive in order to present a pleasant arrival point for the guest.  Each main entrance should have a La Quinta internally illuminated sign and additional landscaping. All drives must be maintained in good condition.  

203.04 Parking Areas

Parking lots must be constructed of concrete or asphalt with continuous concrete curbs.  All parking spaces must be clearly marked.  Parking spaces for the disabled must comply with ADA.  All paving must be maintained in good condition.  Drive aisle shall be a minimum of 22’-0”  (24’ is recommended) wide.  Provide a minimum of one 9’ x 18’ parking space per guest room plus adequate employee parking as required by codes.  At all locations adhere to local ordinances regarding off-street parking.  Parking lots must be adequately sloped for proper drainage; however, slopes shall not be excessive.
203.05 Parking Garages and Covered Parking

Requirements for surface parking apply except as modified below: 

1. Light level must be maintained at least 10 foot-candles, or as required by code.
2. A complete fire sprinkler system shall be provided to comply with all codes and authorities having jurisdiction.

3. Where multi-level garages are provided, exit stairwells must meet the same requirements as for the main hotel.

4. A fire alarm system is required with pull stations at all exit doors. Other devices must be provided to comply with NFPA codes.
5. Panic hardware (crash bar type) is required at exit doors
6. 

203.06 Sidewalks  

Sidewalks shall be constructed of concrete and finished with a uniform non-slip texture.  Provide walks from all areas of the parking lot and to the pool.  Sidewalks shall be a minimum of 5’ wide.  Any walks that are cracked or displaced must be replaced. 

203.07 Connections Between Buildings

Covered walkways are recommended between buildings, and must be a minimum of 5’-0" (1.52m) wide and be of non-slip design and texture. All floors must be sloped for positive draining.  
203.08 Site and Building Lighting

Provide adequate lighting to ensure guest safety at all vehicular entrances to the site, pedestrian entrances to the building, parking lots, sidewalks, steps, porte cochere and pool.  Provide a minimum of 2 -foot candles, or as required by codes, throughout all parking areas.  Utilize pole mounted area light fixtures as appropriate. Wall pack type fixtures are not allowed. 

Light the exterior of the building in order to provide visibility from adjacent thoroughfares.  Mount building wash lights on parking lot light poles.  Provide building wash lighting on all sides that that are visible to local traffic.

203.09 Site Signage

Site signage should be maximized as allowed by local codes and ordinances.  Provide a La Quinta internally illuminated pole sign manufactured by an approved vendor of appropriate size. Provide a La Quinta internally illuminated entrance sign manufactured by an approved vendor at each main entrance. Primary exterior signs shall be lighted from sunset to sunrise.  Provide site sign as required that are professionally designed and fully coordinated.  “No Vacancies” signs are not permitted.

203.010 Service Areas

Locate in inconspicuous areas.  Provide visual screening of dumpsters and other similar equipment with an architectural enclosure that coordinates with the building. Enclosures shall screen the equipment from the guest’s view and have full height screening gates.  The paved area in front of the dumpster shall be 5” thick concrete and be 20’-0" in length by the width of the dumpster enclosure opening.

203.011 Fences

Fences required by local codes or other reasons shall be of the ornamental type and constructed of metal, stained concrete block, stone, or a combination.  Wood fences will be allowed only by special permission. Chain link fences are not allowed anywhere on the property.

203.012 Landscaping

A minimum of fifteen (15) percent of the site shall be landscaped. Planting shall be mature, abundant, appropriate to the location and professionally designed.  Provide planting areas with seasonal color at the vehicular entrances, porte-cochere, and on either side of the front door.  All planted areas shall be irrigated.  Submit landscape plans for all new constructions and conversions to La Quinta for approval.

204 Swimming Pool and Spa

204.02 Design

Locate pool on south or southwest side of the building to take advantage of afternoon sun.  Outdoor pools shall be a minimum of 720 square feet, and indoor pools must be a minimum of 480 square feet.  Maximum depth of all pools must not be greater than five (5) feet. Spas separate from the pool are preferred.  All spas shall be a minimum of 50 square feet in area.  Pool and spas must comply with all current codes.

204.03 Signage

Professionally painted signs indicating pool rules, how to summon emergency help and hours of operation must be displayed.  The water depth must be marked by tiles permanently affixed to the top of the pool edge and at the pool rim at the water line. Tiles reading “No Diving” shall be installed between the depth markers.  Refer to section on signage for detailed requirements.

204.04 Deck and Coping

A pool deck at least 5’-0” in width must surround all pools and spas and sized appropriately for the following minimum furnishings:


4- tables with 4 chairs and an umbrella each.


8- lounge chairs

The deck and coping must have a non-slip finish with a coefficient of friction of 0.6 or greater when wet.  Drainage for the deck must be provided.
The deck finish should be Kool-Deck, Sundec, or equal.
204.05 Enclosure 

As dictated by site design and visibility, the pool area shall be screened by landscaping or a screen wall.  The pool and spa must be enclosed in a minimum 4’-0” high (or higher if required by code) decorative fence.  Gates into the enclosure must be self-closing and self-latching.  

204.06 Equipment

Filtration and pump must have the capacity to re-circulate the entire contents of the pool within a six- hour period, or as required by code.  Water purification such as chlorination, bromination, or ozone treatment is required. 

Diving boards and slides are prohibited.  Steps with a stainless steel handrail are required at the shallow end. A stainless steel ladder with a handrail is required at the deep end.

An underwater light is required in the pool and in the spa.  All electrical connections within the pool area must be of the ground fault (GFIC) type.

Only anti-vortex drains shall be installed in the pool and spa. Spas must be equipped with emergency shut-off devices. 

The following safety equipment shall be provided:

2- U.S. Coast Guard approved life safety devices (i.e. ring buoy with rope)

1- life hook 
A telephone directly accessible to guest shall be provided.  When the receiver is lifted the phone must automatically dial the front desk or, if required by local code, the local emergency medical response personnel. 
A pool heater must be provided.  Pool supplies are not to be stored in the same room as dehumidification or filter equipment.
204.07 Carbon Monoxide Detection

If the pool or spa heater is gas fired and connected to or within the hotel building, a CO detection and alarm system must be provided that meets La Quinta specifications, see Appendix 210.04.

204.08 Lighting

Provide adequate lighting at the pool deck.

204.09 Indoor Pools and Spas

Indoor pools and spas require a separate HVAC system that is capable of removing the excess moisture.  Proper ventilation must be provided to prevent accumulations of odor and humidity.  Chlorine is not recommended to treat the water due to its corrosive nature.  The pool area shall be designed to have negative pressure to reduce odors in the hotel.
When an enclosed swimming pool is provided, its exterior design must be consistent with the exterior design of the main building. The pool area must be separated from the rest of the hotel with a vapor barrier.

Acceptable floor finish is Sundec or equal and slip resistant ceramic tile. The floor must be non-slip with a minimum static coefficient of friction of 0.6 when wet per ASTM 1028.
The wall finish must be appropriate for an enclosed pool environment. Acceptable wall finishes include synthetic stucco, moisture resistant gypsum board with a knock-down textured finish and enamel paint and ceramic tile. If the textured gypsum board finish is used, it is recommended that a ceramic tile wainscot be used also.

A finished ceiling should be provided in the pool area.  If an exposed structure is desired, finishes shall be submitted for approval. Exception: painted, exposed spiral ductwork is acceptable. 
The finishes and materials in the pool area should provide an acceptable acoustical environment. The use of acoustical ceilings, banners, etc., should be considered. Sound ratings between pool and adjacent guestrooms shall be a minimum of 54 STC.


Provide a remote card reader that is operated by guestroom card keys at the indoor pool entrance door. Remote card readers are also required at doors from the indoor pool to the outdoor sun deck unless the sun deck is enclosed with a fence. The fence must be the same as required for outdoor pools.

Window frames must be aluminum. Doorframes should be aluminum where possible.

Where used, exposed metal accessories are to be chrome-plated brass/bronze or stainless steel.

205 Building Exterior

205.02 Porte Cochere

The porte cochere is the hotel’s front door.  As such it should identify the building as a La Quinta Lodging Facility, be visually welcoming and functional.  Where space allows, the canopy should be large enough for two lanes of traffic- two cars deep, with a covered area approximately 24’ x 32’.  It must be well lit. Provide a minimum of four (4) decorative light fixtures at the porte cochere/entry area. The recommended drive surface should have a decorative pavement of exposed aggregate concrete and pavers.  The area immediately in front of the entrance door shall have a maximum slope of two (2) percent.  All elements of the drive must be non-slip with a minimum static coefficient of friction of 0.6 (wet) per ASTM 1028 and without abrupt changes in elevation.  The clear height over the driveway shall be a minimum of 13’-6”.  If the clear height is less than 13’-6”, provide a warning sign. 
205.03 Branding Elements

Elements that identify the property as a La Quinta Lodging Facility must be incorporated into all new construction and into conversions as required by the PIP. These elements may include:


Signature Tower

Ornamental Railings and Awnings

Fascia and Window Trim

Large Windows with full width PTAC grilles

Red Barrel Tile Roof

Exterior Finish and Colors

These elements are important to establish brand identity and to establish consistency among properties.  The prototype drawings contain construction details for all branding elements which, when required to be included in a property, must be followed closely.

205.04 Exterior Building Facade
Generally, light colored buildings of appealing design are preferred.  All new construction shall adhere to La Quinta Inn or La Quinta Inns & Suites prototype drawings.  If a conversion is required to be repainted, the following colors shall be used:

Exterior Walls

Sherwin Williams
LQ-5006
Old Frosty White


Exterior Trim


Sherwin Williams
LQ-1079
Pueblo 

Inn & Suite Fascia Trim 
Sherwin Williams
LQ-1109
Trinidad
Inn Fascia Trim 

Sherwin Williams
LQ-77

Patina Green
The hotel buildings should have sufficient details and architectural enhancements to provide appealing appearance and curb appeal.
205.05 Building Signage

Provide a La Quinta internally illuminated building sign manufactured by an approved vendor on all building facades that face a major thoroughfare.  Signage should be maximized in accordance with local codes and ordinances. All building mounted signs must be semi-recessed per La Quinta “sleeved” sign details. See La Quinta Identification program 210.08.
205.06 Windows and Sliding Glass Doors

Each guestroom must have a window with a minimum of 20 square feet of double glazed glass area, or per code requirements.


All guestroom windows shall have a stop limiting the window opening to 4” unless prohibited by code.  Stops shall not be able to be removed by guest.

Louvers for thru-wall units shall be architectural style with finish to match the window sections or the adjacent wall, as appropriate. 

Sliding glass doors are not allowed in new construction except at beachfront property and only by special permission.  Sliding glass doors are required to have an auxiliary locking device above the reach of children.

205.07 Railings

Railings must conform to local and state codes and shall be fabricated from electro statically finished aluminum or painted steel with a 6” high top rail. Horizontal style railing is not permitted at any location of the hotel, including interior or exterior balconies, stairs, or changes in grade.

206 Interior Public Spaces

206.02 General

La Quinta has developed Interior Decor Packages for guest rooms, guest baths, and all public spaces that should be used whenever updating an existing property or constructing a new one.  These packages were developed by nationally recognized interior design firms and are suitable for any location. Three different Decor Packages are available with selections for floors, walls, lighting, furnishings, countertops, artwork, softgoods, casegoods, and many other interior items.  Color coppies of the décor package color boards are included in section 210.  Complete Decor Package specifications can be obtained from: 

La Quinta Corporation




909 Hidden Ridge, Suite 600




Irving, Texas 75038




Attn: Design and Construction Department
Any deviations to the following standards must be submitted to La Quinta for approval.
206.03 La Quinta Inns 

The hotel shall be a two story double loaded interior corridor facility with a minimum room count of 60.  Provide a mix of Double Doubles and Single Kings as appropriate to the market.  All La Quinta Inns shall include:

· Desk with high speed internet in all rooms and a second phone on the desk in single bed rooms and suites.

· A Continental Breakfast Area

· A Pool 

Suites included in La Quinta Inns shall meet the requirements for suites as listed below.

206.04 La Quinta Inns & Suites

The hotel shall be a three story double loaded interior corridor facility with a minimum room count of 60.  A minimum of twenty percent (20%) of the total room count must be suites in order for the property to be signed as a La Quinta Inn and Suites.  Guest rooms must be at least 396 square feet and have a separate soft seating area in order to classified as a suite.  In addition, La Quinta Inn and Suites must:

· Be an upscale interior corridor properties of three or more stories

· Include:

1. a microwave and refrigerator, 

2. a desk with data port phone and high speed internet,

3. and a second phone on the desk 
4. in all single bed rooms and suites.

· Provide a mix of Double Doubles and Single Kings and Suites as appropriate to the market

· Include a Meeting Room facility (min. 650 s.f.) as appropriate to the market
· Include a well appointed Lobby soft seating 
· Include a well appointed Continental Breakfast
· Include a Pool and Spa

· Include an Exercise Room

· Include a Business Center or fax, printing and copy service at the front desk.

· Guest Laundry

· Include public areas spacious enough to sufficiently accommodate the above stated requirements. Likewise, the physical facilities must convey a sense of value enhancement to contribute to the guest’s experience.

206.05 Entrance Doors

Provide a pair of 3’-0” wide full glass doors with wide stiles and high quality commercial hardware.  A vestibule should be used in cold climates. Automatic sliding doors are recommended in all facilities and required in facilities over 120 rooms.  Secondary entrances shall have electronic card readers.
206.06 Lobby

The lobby is often the guest first impression.  As such, it should be welcoming, attractive and functional. Finishes and furnishings must be stylish, well coordinated and in like new condition.  Two-story lobbies are acceptable, but may require additional fire protection measures. Architectural features such as cove lighting, cove molding, and chair rails are required. 

206.07 Public Area Floors

Provide a combination of carpet and ceramic tile flooring in the lobby with a coordinating wood, tile or stone base. An entry mat is required immediately in front of the entrance door.  Concrete, vinyl composition tile, and glue down carpet are not acceptable in any public area.  All floors shall have a minimum static coefficient of friction of 0.6 (wet). 

Carpet must be a 36-oz. commercial grade solution dyed nylon tufted cut pile with at least three (3) yarn colors as per La Quinta Décor Packages and specifications.

206.08 Public Area Walls

Walls textured with a knocked down finish are preferred throughout the hotel.  If vinyl wall covering is used, it shall be type II in all public areas.  Painted concrete block and painted vinyl wall covering are not acceptable. 

206.09 Public Area Ceilings 

Gypsum board ceilings with an acoustic orange peel textured are preferred.  If lay-in ceilings are existing, they shall be in combination with gypsum board accents and shall be 24” x 24’ with a tegular or reveal edge and a fine fissured, non-directional pattern. 

206.010 Public Area Lighting

Lighting in public spaces should be accomplished with cove lighting, recessed can lights, decorative wall sconces, decorative pendants and tabletop lighting. Light fixtures with prismatic lenses, egg crate diffusers or bare bulbs are not acceptable.  Provide well-lit spaces using fixtures per La Quinta décor packages.  Select lamps to provide a consistent warm light color (2700 Kelvin) throughout the facility, long lamp life, and low energy usage.

206.011 Public Area Artwork

Provide permanent artwork in decorative wood frames in the lobby, continental breakfast area, and other areas as appropriate.  

206.012 Public Area Furnishings

For Inn & Suite properties less than 100 rooms, provide one seating group for 5-6 guests in the lobby.  For Inn & Suite properties over 100 rooms, provide a minimum of 4 seating groups for 4-5 guests each. 
Provide seating in the continental breakfast area equal to twenty-five per cent (25%) of the total number of guest rooms, in a combination of two-tops and four tops.

206.013 Public Area Window Treatment

Provide a valance, full length sheers, and full length side panels per La Quinta décor package at all windows in public spaces.  All windows shall have a uniform appearance from the exterior. 

206.014 Front Desk 

The front desk must be convenient to the front door and, where possible, provide a view to the breakfast area.  Minimum length shall be 19 feet in new construction and 14 feet in conversions.  A minimum of two workstations must be provided.

The desk must be upscale and coordinated with the remainder of the lobby.  The top of the desk must be solid surfacing or granite.  The front shall be tile, solid surfacing, or high quality millwork.  The back work surfaces may be plastic laminate.  Use La Quinta Décor Package in all new construction and in conversions if the front desk is to be replaced or refaced.


The wall behind the desk must be free of counters and equipment.  Provide vinyl wall covering from La Quinta Décor Package and a La Quinta logo from an 
approved vender on this wall. 
206.015 Breakfast Bar

The breakfast bar must be of quality permanent construction.  Portable bars are not allowed except as supplemental areas for service.  The portable bar, if used, must be decorative and match the permanent bar.  A minimum bar length of 16’ is required for facilities with up to 90 rooms.  Additional length as needed for facilities over 90 rooms.  The maximum length of a portable bar is 5’.
The bar top must be constructed of solid surfacing material, with plastic laminate cabinets.  Provide a ceramic tile backsplash and a decorative tile nosing.  Upper cabinets are not preferred.

Provide space for the storage of all food out of the guest’s view when not in use.  A pantry with a refrigerator, dishwasher, sink and storage cabinets is preferred in conversions and is required in new construction.  Breakfast bar pantry and associated areas must comply with all local health requirements.

206.016 Elevators

Elevators shall be a minimum of 2,500-pound capacity with a minimum 3’- 6” wide door.  Minimum speed shall be 175 feet per second for buildings four stories 
or less.  Provide one (1) elevator for up to 90 rooms.  Provide finishes in the guest elevators per La Quinta décor packages (plastic laminate walls, floors carpet of ceramic tile, if carpet it shall match the corridors.
If a rear door opening into the laundry is provided, the door must be operable only by using a staff key to prevent guest access to the laundry.

206.017 Elevator Lobbies

Provide elevator lobbies with self-closing fire/smoke assemblies when required by code.  Elevator lobbies shall be a minimum of 6’-0” wide and attractively furnished to coordinate with the remainder of the hotel.  

206.018 Amenities

206.018.2 Meeting Rooms

Meeting rooms, if provided, shall be finished and furnished to coordinate with the remainder of the hotel. Provide an attractively decorated room with carpet, carpet base and textured and painted walls.  Carpet must be a 36-oz. commercial grade solution dyed nylon tufted cut pile with at least three (3) yarn colors as per La Quinta Décor Packages and specifications.
If meeting rooms are subdivided with folding partitions, the folding partitions must have a minimum STC rating of 50, and an STC rating of 54 is recommended. Accordion type partitions are not permitted.

206.018.3 Business Center

If a business center is provided, it must have at least two workstations, one that is accessible per ADA. Provide a computer that is connected to the internet and a printer.  Carpet must be a 36-oz. commercial grade solution dyed nylon tufted cut pile with at least three (3) yarn colors as per La Quinta Décor Packages and specifications. 

206.018.4 Exercise Room

If an exercise room is provided, provide an attractively decorated room with carpet, carpet base and textured painted walls.  Carpet must be a 26-oz. commercial grade solution dyed nylon tufted loop as per La Quinta Décor Packages and specifications.  The room must be well ventilated and large enough for the required equipment.  One wall must be fully mirrored from a minimum of 12” above the floor to 12” below the ceiling.  A telephone, mounted on the wall near the entry at 48” above the floor, is required.  Telephone must ring the front desk when the handset is lifted. Provide the following minimum equipment:

1- high quality commercial grade stationary bicycle


1- high quality commercial grade treadmill

1- high quality commercial grade multi station gym

1- drinking fountain or purified water dispenser



1- wall or ceiling mounted 27” color TV with remote control


1- wall clock


2- decorative robe hooks


1- soiled towel bin

Free weights, dumbbells, etc are not permitted. The ceiling is recommended to be 9’-0” high to accommodate exercise equipment such as a stair stepper.  The entry door must be protected by a lockset operable by the guest’s card key.  Additionally, it must be possible to see into the exercise room from the corridor.

206.018.5 Public Area Telephones

A house phone and a card swipe phone shall be located on a table in the lobby seating area.
206.018.6 Public Area Toilets

There shall be one restroom for men and one for women in the lobby area. The restrooms shall be accessed from a secondary corridor. They shall not open directly onto the lobby area. Direct line of visual sight into restrooms with multiple stalls is prohibited.  The number of lavatories, stalls and urinals shall not be less than dictated by code.


Finishes shall include 8x8 ceramic tile floors and knock-down textured and painted walls per La Quinta décor package. 
Toilet partitions and doors are to be plastic laminate or solid surface material and are recommended to be ceiling hung. Coat hooks are required on the back of the doors.

Provide a solid surface or granite vanity top with an under-counter mounted porcelain enamel lavatory and single lever faucet. 
A full-width, full-height or decorative framed mirror is required. 
Power exhaust fans, which comply with the local codes, must provide ventilation per local codes. If two or more toilet stalls are provided in the public restrooms, HVAC supply must also be provided in addition to continuous exhaust.

Men and women's restrooms must be clearly identified with international symbols.  
All restroom entrance doors must be a minimum of 3’-0” (.91m) wide. Door closures and kick plates are required on public restroom doors.

A smoke detector and emergency lighting must be provided.

The following accessories are required:

1. Reserve roll toilet tissue holder or surface-mounted dual dispenser.

2. Recessed combination paper towel dispenser and large waste receptacle.

3. Wall-mounted liquid soap dispenser.

4. 
5. 
6. Wall-mounted grab bars as required by ADA.

206.018.7 Vending

Vending with ice and soda is required on every floor.  Provide 6” x 6” or larger ceramic tile at vending areas per La Quinta décor package.

206.019 Guest Corridors

Primary corridors in the guestroom areas must be a minimum of 5’-0" (1.5m) wide.  Refer to Public Area Floors, Walls, Ceilings and Lighting above for general requirements.  
In Inn and Suites, a corridor wall trim is required.  The trim is a wood molding painted the color to the door frames and located 10”  from the ceiling to bottom of molding.
Floors must be carpeted, and double stick padding is required.  Carpet must be a 36-oz. commercial grade solution dyed nylon tufted cut pile with at least three (3) yarn colors as per La Quinta Décor Packages and specifications.  A 4” bound carpet base is required.  
Color coordinated vinyl corner guards are recommended at wall direction changes.

Lighting shall be accomplished by a combination of recessed can lights and decorative wall sconces.  Provide well-lit corridors, with emphasis at guest room entrances. 

206.020 Interior Signage
Signage must be purchased from a La Quinta approved vendor.  Room numbers must be mounted on the wall, on the latch side of the door. Mount all room signs 60” above the floor measured to the centerline of the sign.  
Required Directional Signs

Locations where directional signs are to be used:
· Transition of Lobby to corridors
· Each Elevator Lobby

· Each corridor intersection

· Each secondary building entrance

Directional signage shall indicate the location of guestrooms, vending lobby and other guest amenities.
206.20 Environmental Controls

Heating, ventilation (fresh air) and air conditioning must be provided for all occupied areas and shall be thermostatically controlled.  Where thru-wall air conditioning units (PTACs) are used:
1. Provisions must be made to allow positive drainage from the unit.

2. Thru-wall units must be complete with a metal sleeve.
3. Louvers for thru-wall units shall be architectural style with finish to match the window sections or the adjacent wall, as appropriate.      

Thru-wall wall sleeves shall be properly sealed on the exterior and interior.



207 Guest Rooms 

207.02 La Quinta Inns 

Provide a mix of Double Doubles and Single Kings as appropriate to the market.  Suites included in La Quinta Inns shall meet the requirements for suites as listed below.  Nominally the rooms shall be a minimum of 12’ wide and 288 square feet.
207.03 La Quinta Inns & Suites

A mix of extended suites (min. 12’x33’) and appropriately sized two-room suites equal to a minimum of twenty percent (20%) of the total room count is required in order for the property to be signed as a La Quinta Inn and Suites.  All guestrooms shall be nominally 12’ wide and a minimum of 312 square feet.  In order to classify as a suite, guest rooms must be nominally 12’ wide and at least 396 square feet and have a separate soft seating area.  In addition, all rooms must include a microwave and refrigerator, a desk with data port phone, and a second phone in the bedroom.

207.04 Closets

Closets are a requirement in all La Quinta lodging facilities.  Provide a closet with minimum inside dimensions of 2’-6” wide and 2’-0” deep in each guest room.  Closets shall have a pair of swinging three-panel doors with dummy hardware. Mount a hardwood or chrome plated hanging rod, shelf, iron and ironing board inside the closet.  Closet floor shall be carpet with carpet base, and walls are to be painted knockdown texture.  A folding luggage rack is required in all closets.
207.05 Décor

La Quinta has developed Interior Decor Packages for guest rooms and guest baths.  Use La Quinta Décor Packages in all new construction and conversions.  Selected portions of the packages are included in section 210.  Complete Decor Package specifications can be obtained from: 

La Quinta Corporation




909 Hidden Ridge, Suite 600




Irving, Texas 75038




Attn: Design and Construction Department

207.06 Minimum Size and Configuration

All rooms are required to be adequately sized to accommodate the La Quinta furnishings packages. 

The following furniture is required:


	Item
	
	La Quinta Inn 
	La Quinta Inn & Suites 
	

	X Number
	Description
	Standard Double
	Standard King
	Standard Double
	Standard King
	Standard King Sofa
	Extended King Suite

	X101AD   X101ADB   
	Double Bed     54" w  x 80" l       
	2
	
	2
	
	
	

	X101AK   X101AKB   
	King Bed         76" w  x 80" l
	
	1
	
	1
	1
	1

	X201D   X201K
	Straight Headboard
	2
	1
	
	
	
	

	X201DA   X201KA
	Arched Headboard
	
	
	2
	1
	1
	1

	X200
	2 Drwr Dresser 60" w x 22" d
	1
	1
	1
	1
	1
	1

	X202
	Night Table    24" w x 16" d
	1
	2
	1
	2
	2
	

	X203
	Night Table    16" w x 16" d
	
	
	
	
	
	2

	X204
	Desk-Rect 26" w x 48" d
	1
	
	1
	
	1
	

	X213
	Desk P-Top 42" dia x 66" l
	1
	
	1
	
	1

	X205
	Desk Chair  Ergonomic
	1
	
	1
	
	
	

	X209
	Desk Chair  High Back
	
	1
	
	1
	1
	1

	X212
	Desk Chair Straight Back
	1
	
	1
	
	

	X210
	Lounge Chair
	1
	
	1
	
	
	2

	X211
	Recliner Chair
	
	1
	
	1
	
	

	X222
	Barrel Chair   King 
	
	
	
	
	
	

	X214
	Love Seat Sleeper 60" l
	
	
	
	
	1
	1

	X215
	Coffee Table 34" w x 20" d
	
	
	
	1
	1

	X216
	End Table  27" w x 22" d
	
	
	
	
	
	1

	X217
	Round Table 24" dia x 23" h
	
	
	
	
	1

	X218   
	Micro-Fridge Cabinet
	
	
	1
	1
	1
	1




207.07 Acoustics

Demising walls between guest rooms and walls between guest rooms and the corridor shall provide an STC 50 or better.  Walls between guest rooms and noisy spaces such as elevators, maid’s closets and vending areas must provide an STC 54 or better.

Guest room floors/ceiling assemblies shall provide a minimum STC rating of 50. 

207.08 Entrance Doors

Guestroom entry doors for interior corridor properties shall be 1 ¾” (4.45cm) 20 ga 6 panel steel, tight fitting smoke- and draft- control assemblies having a fire protection rating of not less than 20 minutes or as otherwise required by code. Ratings shall be so indicated by Underwriters Laboratory labels attached permanently to the doors and frames.

Existing guestroom flush doors shall receive applied molding to simulate a six-panel door, refer to details.
Guestroom entry doors are to be a minimum of 3-0" x 6'-8" (.81m x 2.03m). Doors are to have two spring-loaded hinges or commercial grade door closers with covers.  Doors must be adjusted to be self-closing and self-latching.

Frames for interior corridor guestroom entry doors shall be 18 gauge hollow metal or knockdown type. Framed, non-masonry walls shall be reinforced on both sides of the jamb to prevent prying and flexing for security control.


Provide the following door hardware:

1. 1 ½ pair of butt hinges, 1 pair to be spring-loaded or a commercial grade door closer with cover.

2. Kick plate on the corridor side of the door.

3. Sound seals on interior doors and thresholds and weather-stripping on exterior doors. 

4. U-bolt type security latch located no higher than 48” above finished floor.

5. Tamper-proof, one-way viewers located at 60" above finished floor.  

6. An electronic lockset with lever handle for hotel usage as manufactured by TESA, VingCard, or Saflok or equal.  

a. 
b. 
c. 
d. 
e. 
f. 
g. 
h. 
i. 
Door hardware finish for all hotels must be US4 Satin Brass. The finish on all hardware must be the same.

In addition to the requirements above, exterior doors are to have thresholds and weather stripping in lieu of sound seals.

207.09 Connecting Room Doors

The use of connecting rooms is discouraged.  If used, connecting rooms are to be connected by two solid core wood or steel doors rated as required by code.  The two doors shall have a combined STC rating of not less than 50. Provide the following hardware for both connecting room doors:

1. Latch set equal to Sargent 10G15-3LL with operating lever on the room side only. 

2. 1 1/2 pair of butt hinges

3. Thumb turn deadbolt with 1" throw with a blank plate installed on the opposite side of the door.

4. Sound seals.

5. The finish of all connecting room hardware must match the entry door finish. 

207.010 Floor Covering

Guest rooms floors are to be carpeted with pad and a carpet base.  Carpet must be a 32-oz. commercial grade solution dyed nylon tufted cut pile with at least three (3) yarn colors.  Refer to La Quinta Décor Packages for complete specifications. 
207.011 Wall Finish

Walls are to be bright, appealing and well maintained.  Painted knock-down texture is preferred.  Paint all four walls:

Sherwin Williams
 City Loft
SW1032
Latex Eggshell Enamel

Contract quality, type I (15 oz.), Class A, vinyl wall covering is acceptable in conversions.  Painted vinyl wall covering and painted concrete block are not acceptable. 
Where ceiling trim is installed, the wall above the trim is to be Luminous White SW 1900 eggshell.
207.012 Ceiling Finish

Ceilings shall be sound absorbent, light reflecting and a minimum of 8’-0” high. Cover ceilings uniformly with spray acoustical (popcorn) texture.  Paint ceilings:

Sherwin Williams
 Luminous White
SW1900
Latex Flat

207.013 Electrical

Electrical outlets are to be placed to allow convenient connection of the individual fixtures/lamps. Electrical cords must be concealed from guests’ view.

1. Provisions must be made for electrical outlets to supply power to the television set, AM/FM clock radio, microwave, refrigerator, coffee maker and hair dryer.

2. At least one electrical outlet shall be conveniently located and accessible to guests for use with an iron.

3. A duplex outlet and telephone outlet mounted 15” above the floor are required at each desk.

4. A ceiling or wall mounted light fixture is required in the entry area. This light must be controlled by a switch mounted adjacent to the entry door.

5. Back-to-back power, telephone, television and other electrical outlets or junction boxes are not permitted between guestrooms.  Device boxes in adjacent rooms must not be set in the same stud cavity.

6. Ground fault interrupter circuit (GFIC) receptacle shall be installed at any location within six feet (6’) of a water source. 

207.014 Environmental Controls

Heating, ventilation (fresh air) and air conditioning must be provided for all guestrooms and shall be thermostatically controlled.  Where thru-wall air conditioning units (PTACs) are used in guestrooms:

1. Provisions must be made to allow positive drainage from the unit.

2. 
3. Thru-wall units must be complete with a metal sleeve.
4. Louvers for thru-wall units shall be architectural style with finish to match the window sections or the adjacent wall, as appropriate.      

5. Thru-wall wall sleeves shall be properly sealed on the exterior and interior.

207.015 Drapery

Provide the following at all guest room windows:

1. Side panels- Fixed full length Min. 15” wide panels on either side of window

2. Valance- Hardboard framed 15” deep or proportioned to the ceiling height.  Match side panel fabric

3. Sheers- Flame retardant polyester, 200% fullness

4. Black-out panels- White black-out fabric with batons and weights to prevent blowing

Refer to La Quinta décor package for complete details of draperies and drapery fabric.

207.016 Art Work

Inn: In a double/double a total of four pieces, one over each bed, one at the desk and one at the dresser.  In a king a total of three pieces, one over the bed, one at the desk and one at the dresser.  Refer to La Quinta décor package for artwork selections.
Inn and Suite:  A total of three pieces in each room, one at the desk, one at the dresser and one on the bath wall adjacent to the bed.  There is no art over the beds with the arched headboards.  Refer to La Quinta décor package for artwork selections.
207.017 Lighting

Guest room lighting must be provided by recessed can lights, decorative ceiling mounted fixtures, decorative wall mounted bedside fixtures, table and floor lamps.  Decorative lights shall be per La Quinta FF&E package.  Provide a well-lit room using appropriate fixtures.  Fixtures must be contemporary, UL listed and powder coated.  Provide matching cord covers for all wall mounted fixtures.  Select lamps to provide a consistent warm light color (2700 kelvin), long lamp life, and low energy usage. Warm white fluorescent lamps are preferred, but incandescent lamps are acceptable.  Light fixtures with prismatic lenses, egg crate diffusers or bare bulbs are not acceptable.    

A ceiling fixture or decorative wall sconce is required at the entry area.  This fixture shall be three-way switched from the entrance door and the bedside.

Provide a wall mounted bedside fixture over each night table.  A matching fixture is to be installed beside the mirror over the dresser.

Provide appropriate task lighting at the desk.  This fixture should be a, wall sconce or recessed can light.  If a desk lamp is provided, it is recommended that a data jack be provide in the lamp base.

A table or floor lamp is to be provided adjacent to any soft seating for reading.   

207.018 Microwave/Refrigerator

All guest rooms in La Quinta Inns & Suites must have a microwave and refrigerator.  These must be installed in a cabinet with doors that matches the casegoods or built into millwork.   
All suites in an Inn must have a microwave and refrigerator.  These must be installed in a cabinet with doors that matches the casegoods or built into millwork.
207.019 Telephones and Data Terminals

All guestrooms shall have a touch-tone phone.  Each single bed guest room shall have two touch-tone telephones, one at bedside and one at the desk.  In new construction, they shall be two-line telephones, capable of two conversations at once or a conversation and use of a computer.  All telephones shall have push buttons, a message warning light and a data port.
La Quinta Inns High Speed Internet Access Standards

· Minimum of Mbps, hardwired access in every guest & meeting room.

· Wireless access in the lobby / breakfast area.

· Live answer, 24 x 7 telephone support for guests & hotel staff

· Must support a minimum of 15 concurrent VPN sessions.

· System security must prevent unauthorized, room-to-room access.

· No guest PC configuration changes are required to use the system. This includes but is not limited to e-mail access.
· Remote system diagnostic capabilities.

· Minimum Internet access bandwidth (requirements can vary greatly depending on clientele & number & type of meetings):

· 100 or less guest rooms: 384KB

· 100+ guest rooms: 512KB
           Both the guest room & the wireless lobby breakfast area access will be a free to guest 

            Amenity.

208 Guest Baths

208.02 Layout

The La Quinta guest expects a well lit generous bath area with quality fixtures.  
208.03 Floors

Floors must be minimum 6” x 6” non-slip ceramic tile with a minimum static coefficient of friction of 0.6 when wet per ASTM 1028.  A solid surface threshold at the junction between carpet and tile with a 2" (5.08cm) minimum width, not to exceed 5/16” (.79cm) above bath floor level, is required.

208.04 Walls

Walls shall be moisture resistant and light reflecting. Cover all bath walls uniformly with knock down texture.  Paint walls:

Sherwin Williams
 City Loft

SW1032
Eggshell Enamel Latex

Type I vinyl wall coverings are also acceptable, except in humid climates.

208.05 Ceilings

Ceilings shall be moisture resistant, light reflecting and a minimum of 7’-4” high. Cover ceilings uniformly with orange peel texture.  Paint ceilings:

Sherwin Williams
 Luminous White
SW1900
Semi-Gloss Alkyd Enamel

208.06 Vanity

Tops shall be made of cultured marble with a raised drip edge in front and cultured marble side and back splashes.  Provide an under-counter mounted or top set porcelain enamel lavatory bowl.   

208.07 Plumbing Fixtures

Provide a tub/shower combination. Cast iron or steel tubs with sound blankets and factory finished non-slip bottoms are required. The minimum tub length is 5’-0" (1.52m).


Toilets must be white, tank type and have elongated bowls. Flush valve toilets are not permitted.

The toilet must have a white, open front plastic seat with lid. The seat and lid must remain in an upright position without being held.  Padded seats are unacceptable.

All exposed plumbing fixtures (pipes, faucets, spouts, etc.) must be high quality, chrome-plated brass as manufactured by nationally known manufacturers. Single lever mixing valves must be installed for lavatory and bath/shower. All tubs shall have stoppers.  Rubber stoppers are not permitted.

The tub/shower valve in the typical guestroom bath shall be Symmons #S-25-2-BF-X-LP with a Sloan Act-o-matic AC-51 showerhead or equal. 
The lavatory faucet for the typical guestroom or accessible bath shall be American Standard 2385.400 with pop-up drain or equal.


1. 
2. 
3. 
4. 
5. 
Each hot and cold water supply shall have an individual cut-off valve. Water supply for toilet must be mounted 10" above finished floor to avoid conflicting with tile base.

Bathtubs and lavatories must have overflow outlets.

ABS pipe is not permitted for use in La Quinta lodging facilities.

If PVC drainage piping is used above public spaces, the area must be insulated for sound control. As an option, the drainpipes may be insulated.
208.08 Tub Surrounds

Acceptable surrounds are cultured marble or solid surfacing products equal to Corian. Finish and color of surrounds shall match Corian Cameo. The tub/shower surrounds must extend to the ceiling. Ceramic tile, acrylic and fiberglass units are not acceptable. except at existing properties, and only if in like new condition.

208.09 Lighting

General illumination for dressing and color- balanced illumination for grooming are required.  Use contemporary decorative fluorescent fixtures with warm white lamps, a tube type over the vanity and a round fixture in the ceiling.  Provide a minimum of 40 foot-candles at the vanity.  When the vanity and tub/shower are in separate areas, switch each area independently.
208.010 Electrical

A GFIC duplex electrical outlet must be located at both ends of the vanity to allow convenient use of personal care appliances in front of the vanity mirror and for the required hair dryer (1500 watts) and coffee maker.  
208.011 Exhaust System
The toilet area is to be mechanically exhausted by metal ducts and continuously operating fans. The exhaust system must contain baffles to prevent sight and sound transmissions between bathrooms. Baths shall not vent into the chase ways.


Combine exhaust ducts to minimize penetrations through the building exterior.  Exhaust vents shall be placed no closer than ten feet from an operable window or air intake louver.  When possible, exhaust shall be routed through the roof on the rear of the building.

208.012 Bath Door

Provide six panel hollow core wood doors, minimum size of 3’-0” x 6’-8" x 1 3/8”. If a full-length dressing mirror is used on the bath door, a solid-core door is to be provided.

A privacy set equal to Sargent 10 line is required on bathroom doors. The finish must be US26D satin chrome finish on the bath side and the finish on the room side must match the entry door hardware. One pair of butts with US26D finish is required. Lever type handles are required.

Lock operation: The latch bolt must operate by lever from either side, except when a push-button locks the outside lever. The push-button lock must release by turning the inside lever or by closing the door. The push-button lock must also be released by an emergency key. The backset must be 2 3/8” (6.03cm) minimum with a minimum latch projection of ½” (1.27cm).
208.013 Mirrors

Vanity Mirror. A wall-mounted full wall unframed mirror is required above the vanity. The light switch and outlet must be located so they do not conflict with the mirror. The mirror must be 1/4” distortion free, polished float plate glass, manufactured according to federal specifications DDM-411 for Grade 1 mirrors.

Dressing Mirror. A full-length mirror is required in all rooms, located in or immediately adjacent to the dressing area and mounted 18" above the finished floor. Minimum size is to be 18” x 60” (.46m x 1.52m). The mirror must have polished edges. If mounted on the bath door or closet door, it is to be mounted 12” above the finished floor with mirror clips. The door must be solid core.

208.014 Bath Accessories

All accessories shall be non-corrosive, stainless steel or chrome. The following accessories shall be provided:
1. Facial Tissue Dispenser. A recessed facial tissue dispenser shall be installed in the vanity front adjacent to the bath door. The panel is to be of one piece, heavy gauge, and triple chrome-plated metal, with snap-in installation or with a dispensing slot cut into the man-made marble vanity skirt. Depth is to be a minimum of 2 ½” (6.35cm). No brand name designation is allowed. 
2. Toilet Tissue Dispenser. The toilet tissue dispenser is required equal to CSI Donner 576/587.

3. Robe Hook. The double robe hook is to be zinc and triple chrome-plated with a concealed screw and minimum hook projections of 1 5/8” (4.13cm) mounted at 6'-0” AFF.

4. Shower Rod. The shower rod must be located 6'-8" off the finished floor to the center line of the rod. The shower rod must be permanently secured.

5. Grab Bar. A chrome or stainless steel grab bar, 24" (.61m) in length, is to be installed horizontally on the tub back wall. Proper backing/blocking is to be provided for secure installation. All grab bars must be securely anchored and capable of withstanding 250 lbs. of pull. Grab bars must have flange covers to conceal the mounting screws

6. Towel Shelf. One corian towel shelf with angled support brackets (minimum length 24") (.69m) will be located over the toilet.

When the tub/shower and toilet are separate from the vanity, one chrome towel shelf, including braces and towel bar, minimum length 18" (45.7 cm), shall be located in the tub area mounted over the toilet, and one towel bar, minimum length 18" (45.7 cm), is to be located in the vanity area. Towel bar with towels shall not cover light switches/receptacle.

209 Support Areas and Systems

209.02 Electrical Panel Boxes/Circuits

All electrical panel boxes and circuits must be labeled and lockable.  Unless prohibited by local authorities, all electrical panels and circuits accessible to the public must be kept locked. Exposed electrical panel boxes must be finished to coordinate/blend with the surrounding interior finish and color scheme.

209.03 Emergency Lighting

Emergency lighting is to be provided to comply with all codes and authorities having jurisdiction. Additionally, emergency lighting fixtures shall be provided in any area that may be occupied by a guest, except guest rooms and guest baths.  Emergency lighting fixtures shall be arranged to maintain not less than one foot-candle measured at the floor at all points, including corners.  Emergency lighting fixtures shall be automatic, not requiring any manual action to put them into operation after failure of normal lighting.  Power for emergency lighting shall be supplied by approved reliable electric storage batteries with trickle charge system or equivalent.

209.04 Property Management System

209.04.2 Manager’s Office

Space must be made in General Managers’ office for one workstation with monitor and one printer. Three power outlets near the workstation are required.

209.04.3 Mainframe System

Provide a room for one 8’ x 4’ x 3’ rack to facilitate 2 file servers. Room must be accessible to auditor 24 hours a day. A total of four dedicated power outlets and one surge protected power strip will be required for installation and must be within 3 ft of rack.  
Location of file servers must be taken into consideration if any Interface equipment is going to be installed such as call accounting, movies, etc… A power source must be available for these interfaces.

Interface vendors must be contacted and made available to La Quinta IS Personnel 2 weeks prior to opening to insure proper configuration of equipment.  45 days prior to installation, a La Quinta Data Communication Technician will be contacting you to begin the procedures of installation of the T1 line at your facility.
Since location of T1 demarc will be near phone cabinet, an extended demarc may be placed within 6 ft of network router and switchbox.
Category 5 cable may be run from T1 demarc near phone cabinet to a patch panel located within 6 ft of rack.
209.04.4 Front and Back Office Requirements

A total of 5 power outlets are required in front office area. 

Front desk area will require a space for two workstations and monitors. 
If additional equipment is needed, contact La Quinta for assistance.
Space for printer (2’ x 2’ x 1’) must also be made in front office area.

Category 5 cable (CAT 5) must be run, and pinned at each end with RJ-45 connectors, from location of router and switch box to each workstation and printer at front office area.

General Manager’s office shall have 3 power outlets.

Space must be made in the General Manager’s office for one workstation, one monitor and one printer.
Category 5 cable (Cat 5) must be run, and pinned at each end with RJ-45 connectors, from location of router and switchbox to managers’ workstation and printer.

209.04.5 Telephone Requirements

One dedicated phone line with it’s own number is required for dial backup system.  This line must be located within 6 ft of network router and switch.  
209.05 Employee Break Room

If an employee break room is included, include a minimum of six (6) lockers when the hotel has 75 guestrooms or less. Lockers should be a minimum of 2.5 cubic feet (.07m3).  
Walls are to be textured and painted.  Ceilings may be textured and painted or 2x2 lay-in acoustical tile.
Provide a minimum of one employee restroom separate from the public restrooms.  
A cooking surface is not allowed in employee break rooms.

Provide a central HVAC or a thru-wall unit in the break room, designed such that there is a negative air pressure differential in the break room. Provide continuous exhaust in the employee restroom.  

209.06 Environmental Controls

Heating, ventilation (fresh air) and air conditioning must be provided for all areas of the hotel and shall be thermostatically controlled.  Thru-wall air conditioning units (PTACs) are allowed in guestrooms, two-bay meeting rooms, and employee break rooms.  When PTACs are used:

1. Provisions must be made to allow positive drainage from the unit.

2. 
3. Thru-wall units must be complete with a metal sleeve.
4. Louvers for thru-wall units shall be architectural style with finish to match the window sections or the adjacent wall, as appropriate.      

5. Thru-wall wall sleeves shall be properly sealed on the exterior and interior.

209.07 Laundry/Housekeeping Area

Floors must be non-slip sealed concrete or better, with a minimum coefficient of friction of 0.6 per ASTM 1028 in the laundry area.  Walls must be painted gypsum board, masonry or better. 
Pre- manufactured linen chutes are required in each building with three or more floors and must comply with all codes and authorities having jurisdiction. Linen chutes must have lockable doors if accessible by guests.


Provide a separate central HVAC system for the laundry room that will maintain a comfortable temperature and humidity level.

Provide a non-combustible enclosure around the dryers.

Provide a wall-mounted telephone in the laundry area.


209.08 Hot Water Supply

Domestic hot water for guestrooms must have a circulating pump or other approved system installed in-line to provide instant hot water at all fixtures. The maximum water temperature delivered to the guestroom shall be 110° - 120° F.
Hot water temperature for laundry shall meet local health codes. Temperature gauges shall be provided in all hot water supply and return lines. 

209.09 Mechanical Rooms/Electrical Rooms/Elevator Equipment Rooms

Storage of any materials in these rooms is prohibited.  The floor finish must be sealed concrete or better with vinyl base. The walls must be painted.

Fire safety detectors and alarms shall comply with all codes and authorities having jurisdiction

209.010 Storage Rooms

Adequate storage shall be provided at all properties. All storage rooms shall be provided with a hard-wired smoke detector.  Suggested minimum storage requirements are:


· 
· 
· 
· 

· Linen storage (including bedspreads, blankets, etc.).

· Replacement furniture (at least one set, including mattress and box springs).

· Stationery and miscellaneous supplies for rooms (including hand soap).

· Guest equipment (irons, ironing boards, hair dryers, etc.).

· Extra television sets and extra individual air conditioning units.

· Toilet tissue, facial tissue, coffee host supplies and glasses.

· Roll-away beds and baby cribs.

· Chemical and soap supplies.

· Maid carts, vacuums and related equipment.

Electronic locksets are to be used at all linen storage rooms.

209.011 Trash Collection

Trash chutes and trash collection rooms are not permitted. Separate service elevators are recommended for buildings over five (5) stories high.

209.012 TV/Video Equipment Room

Provide a room ten feet (10’) by twelve feet (12’). The room must be independently conditioned to handle the following heat loads:

Telephone Switch: Heat load – 600 BTU’s/hr.  Electrical power – Dedicated 120VAC 60Hz (NEMA 5-20R or –15R) standard grounded outlets.

LodgeNet System: Heat load – 5,000 BTU’s/hr.  Electrical power – Dedicated 120VAC 60Hz (NEMA 5-20R or –15R) grounded outlets: 1-15Amp twist-lock and 1- standard.

General room electrical power outlets: 3-15Amp standard grounded quad outlets (may be ganged on same NEMA 5-20 circuit)

210 Appendix

Appendix Table of Contents

210.1 Approved Décor Packages


210.01.1 
210.01.2 
210.01.3 
210.01.4 
210.2 Finish Materials

210.3 Paint Specifications

210.4 Carbon Monoxide Detection System specifications

210.5 Selected Room Floor Plans

210.05.1 Standard Double/ Double Room

210.05.2 Standard Single King Room
210.05.3 Optional Standard King
210.05.4 King Sofa Room

210.05.5 Extended King Suite

210.05.6 Two Room King Suite

210.05.7 Fixtures, Furniture and Equipment Schedule

210.6 Design Standard Drawings

210.06.1 Front Desk and Breakfast Buffet Elevation

210.06.2 Front Desk Sections

210.06.3 Breakfast Buffet Section

210.06.4 Closet Details

210.06.5 Six Panel Door Molding Layout

210.06.6 Typical Tower Elevation

210.06.7 Tower Trim

210.06.8 Porte-Cochere Column Detail

210.06.9 Column Base Trim

210.06.10 Column Crown Detail
210.06.11 Elevator Details

210.06.12 Elevator Details

210.06.13 Closet Enclosure Option
210.7 Vendor List
210.8 Signage Information
Guest Room Interior Design Scheme- AUSTIN (A)
[image: image4.jpg]




Guest Room Interior Design Scheme- CAPRI (C)
[image: image6.jpg]



Guest Room Interior Design Scheme- GRANDE (G)

[image: image9.jpg]Y]
w
=
W
I
v
w





	



	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	

	
	
	
	
	

	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	


	La Quinta Prototype 
	
	

	FINISH MATERIALS
	

	
	
	210.02
	

	
	
	

	
	
	

	Room Type
	Code
	Description
	Manufacturer
	Specifications
	 
	

	 Guestroom
	CTI
	Ceramic Tile
	Daltile
	
	Pattern: Vitrestone - VS04 Almond
	
	

	 Bath
	 
	 
	
	 
	
	Size: 8" x 8"
	
	

	
	 
	 
	
	 
	
	 
	Grout: Custom Building Products
	

	
	 
	 
	
	 
	
	 
	
	1-800-272-8786
	

	
	 
	 
	
	 
	
	 
	
	#145 Light Smoke
	

	
	 
	 
	 
	 
	
	 
	
	

	
	 
	  CTIB
	C.T. Base
	Daltile
	 
	Pattern: Vitrestone - P3689
	

	
	 
	 
	 
	 
	
	 
	Size: 6" x 8" Sanitary Cove Base
	

	
	 
	 
	
	 
	
	 
	Grout: Custom Building Products
	

	
	 
	 
	
	 
	
	 
	
	#145 Light Smoke
	

	
	 
	
	 
	 
	 
	 
	

	 Lobby/
	 
	   CT2
	C.T.
	 
	Daltile
	 
	Pattern: Antica Rome
	

	 Breakfast Area,
	 
	
	 
	
	
	Color: AM04 - Quirinale
	

	 Vending (1st
	 
	 
	 
	
	 
	Size: 12" x 12"
	
	

	 Floor Only)
	 
	 
	 
	
	 
	Finish: Polished
	
	

	
	 
	 
	
	 
	
	 
	Grout: Custom Building Products
	

	
	 
	 
	
	 
	
	 
	
	#145 Light Smoke
	

	
	 
	 
	 
	 
	
	 
	
	

	
	 
	CT2B
	C.T. Base
	Daltile
	 
	Same as Floor Tile CT2
	

	
	 
	 
	 
	 
	
	 
	Size:  6" Ht x 12"L (cut from 12x12)
	

	
	 
	 
	
	 
	
	 
	Grout: Custom Building Products
	

	
	 
	 
	
	 
	
	 
	
	#145 Light Smoke
	

	
	 
	
	 
	
	 
	
	

	Public Toilet,
	CT3
	C.T.
	 
	Daltile
	 
	Pattern: Vitrestone Abrasive
	

	Vending,
	 
	(Wall & Floor
	
	 
	Color: VS08 Brownstone
	

	Guest Laundry
	 
	as req.)
	 
	
	 
	Size: 8" x 8"
	
	

	
	 
	 
	
	 
	
	 
	Grout: Custom Building Products
	

	
	 
	 
	
	 
	
	 
	
	#185 New Taupe
	

	
	 
	 
	
	 
	
	 
	
	

	
	 
	 
	 
	 
	
	 
	
	

	
	 
	CT3B
	C.T. Base
	Daltile
	 
	Pattern: Same as CT3
	

	
	 
	 
	 
	 
	
	 
	Cove Base
	
	

	
	 
	 
	 
	 
	
	 
	Size: 6" x 8"
	
	

	
	 
	 
	
	 
	
	 
	Grout: Custom Building Products
	

	
	 
	 
	
	 
	
	 
	
	#185 New Taupe
	

	
	 
	 
	 
	 
	
	 
	
	

	Swimming Pool
	CT4
	C.T.
	 
	Daltile
	 
	Pattern: Aquarium Porcelain
	

	
	 
	 
	Band - at
	
	 
	               Pool & Spa Tile
	

	
	 
	 
	waterline
	 
	 
	Color: Coastal Blue P072
	

	
	 
	 
	 
	 
	
	 
	Size: 6" x 6"
	
	

	
	 
	 
	 
	 
	
	 
	
	

	H.C.
	 
	CT5
	C.T.
	 
	Daltile
	 
	Pattern: Keystone - Mosaic
	

	Guestroom,
	 
	 
	 
	
	 
	               Unglazed: Group 2
	

	Roll-In Shower
	 
	 
	 
	
	 
	Color: Marble DK-325
	

	
	 
	 
	 
	 
	
	
	Size: 2" x 2"
	
	

	
	 
	 
	
	 
	
	 
	Grout: Custom Building Products
	

	
	 
	 
	
	 
	
	 
	
	#145 Light Smoke
	

	Front Desk,
	CT6
	Decorative Tile
	Daltile
	 
	Pattern:  Tumbled Natural Stone
	

	Band Below the Top
	 
	 
	 
	
	 
	Color:  TS4 Emerald
	

	
	 
	 
	 
	 
	
	 
	Size:  4" x 4" Accent Strip
	

	
	 
	 
	
	 
	
	 
	Grout: Custom Building Products
	

	
	 
	 
	
	 
	
	 
	
	#145 Light Smoke
	

	
	 
	 
	
	 
	
	 
	
	

	Breakfast Counter
	CT7
	Wall
	 
	Daltile
	 
	Pattern: Goldrush Wheatland 5207
	

	Back Splash
	 
	 
	 
	
	 
	Color:  Accent
	
	

	
	
	 
	 
	 
	
	 
	Size: 8" x 10"
	
	

	
	 
	 
	
	 
	
	 
	Grout: Custom Building Products
	

	
	 
	 
	
	 
	
	 
	
	#145 Light Smoke
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	Breakfast Counter
	CT8
	Wall
	 
	Daltile
	 
	Pattern:  Tumbled Natural Stone
	

	Apron
	
	 
	 
	 
	
	 
	Color:  TS4 Emerald
	

	
	
	 
	 
	 
	
	 
	Size:  4" x 4" Accent Strip
	

	
	
	 
	 
	 
	
	 
	Grout: Custom Building Products
	

	
	
	 
	 
	 
	
	 
	
	#145 Light Smoke
	

	
	
	 
	 
	 
	
	 
	
	

	
	
	 
	 
	 
	
	 
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	Employees Break
	VCT1
	VCT Tile
	Armstrong
	Style: 51929
	
	

	Room
	
	 
	 
	 
	
	 
	Size: 12" x 12" x 1/8"
	

	
	
	 
	 
	 
	
	 
	Color: Sandy Beach
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	Back of the House
	RB1
	Rubber Base
	Johnsonite Wall
	Style: 1/8" Rubber Cove Base
	

	
	
	 
	 
	 
	Base
	 
	
	No. 49 - Beige
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	Lobby,
	 
	COR1
	Corian
	
	 Corian or
	 
	Pattern: Sierra Family
	

	Registration,
	 
	
	 Approve
	Color: Beach
	
	

	Breakfast Counter
	 
	 
	 
	 Equal
	 
	Finish: 1/2" Thickness
	

	
	 
	 
	 
	 
	
	 
	
	

	Meeting Room,
	PL1
	Plastic Lam
	Formica
	Pattern: 7022-58
	
	

	Board Room - 
	 
	 
	 
	
	 
	Color: Natural Canvas
	

	Millwork,
	 
	 
	 
	
	 
	Finish: Matte
	
	

	Front Desk, Breakfast,
	 
	 
	 
	
	 
	
	

	Computer Millwork
	 
	 
	 
	
	 
	
	

	
	 
	 
	 
	 
	
	 
	
	

	Suite Wet Bar,
	PL2
	Plastic Lam
	Formica
	Pattern: 771-58
	
	

	Back of the House
	 
	 
	 
	
	 
	Color: White Kid Leather
	

	Millwork,
	 
	 
	 
	
	 
	Finish: Matte
	
	

	Toilet Partitions
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	Elevator Wall
	PL3
	Plastic Lam
	Wilson Art
	Pattern: #4206-60
	

	
	
	 
	 
	 
	
	 
	Color: Verdigris
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	Lobby, Corridor, 
	VA
	 Vinyl 
	 
	MDC
	 
	Pattern: Satsuma II
	

	Meeting Room
	 
	 
	 
	Wallcoverings
	Color: Stone BB-SA-07
	

	Prefunction Area,
	 
	 
	 
	Tessa B. Moss
	Weight: 20 oz
	
	

	Office, Back of House,
	 
	
	 
	214-760-9300 ph
	
	

	Exercise Room
	 
	 
	 
	214-674-9693 cell
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	Meeting Room,
	VB
	Vinyl
	
	Tower Vinyl
	Pattern: Jacquard Tye II
	

	Board Room
	 
	
	 
	Wallcovering
	Color: Lightning BB-JQ-01
	

	
	 
	 
	 
	 
	1-800-222-1028
	Weight: 20 oz
	
	

	
	 
	 
	 
	 
	
	 
	
	

	
	 
	 
	 
	 
	
	 
	
	

	
	 
	 
	 
	 
	
	 
	
	

	Women -
	VC
	Vinyl
	 
	MDC
	 
	Pattern: Brielle Type I
	

	Public Toilet,
	 
	
	 
	Wallcoverings
	Color: Canopy A1-2503
	

	Employee's 
	 
	 
	 
	 
	 
	Weight: 20 oz
	
	

	Break Room
	 
	
	 
	
	 
	
	

	
	 
	 
	 
	 
	
	 
	
	

	Accent Wall,
	VD
	Vinyl
	 
	Tower Vinyl
	Austin, Baja, Capri, Grande Schemes:
	

	Front Desk
	
	 
	 
	Wallcovering
	Pattern:  Peppertone Type II
	

	
	 
	 
	 
	 
	
	 
	Color: Spruce T2-P1-13
	

	
	 
	 
	 
	 
	
	 
	Weight: 15 oz
	
	

	
	 
	 
	 
	 
	
	 
	
	

	
	 
	 
	 
	 
	National
	 
	 For Dallas Scheme:
	

	
	 
	 
	 
	 
	Wallcovering
	Pattern:  Plexus Haiku
	

	
	 
	 
	 
	 
	Kristen Harris
	Color:  Ocean Type II  71492020
	

	
	 
	 
	 
	 
	214-320-1239 ph
	Weight:  20 oz
	
	

	
	 
	 
	 
	 
	800-222-1028 w
	
	

	
	 
	 
	 
	 
	214-495-8635 fx
	
	

	
	 
	 
	 
	 
	
	 
	
	

	Men's Public
	VE
	Vinyl
	 
	MDC
	 
	Pattern: Brielle Type I
	

	Toilet
	 
	 
	 
	 
	Wallcoverings
	Color: Divine A1-2512
	

	
	 
	 
	 
	 
	
	 
	
	

	Guest Room Walls
	PIA
	P. On
	 
	Sherwin
	 
	Color: City Loft SW1032
	

	Below Trim
	 
	Knockdown
	Williams
	Finish: Latex Eggshell Enamel
	

	
	 
	 
	 
	 
	
	 
	
	

	Guest Bath Room
	PIB
	P. On 
	 
	Sherwin 
	Color: City Loft SW1032
	

	Walls
	 
	 
	Knockdown
	Williams
	Finish: Acrylic Semi-Gloss
	

	
	 
	 
	 
	 
	
	 
	
	

	Lobby
	 
	PIC
	P. On
	 
	Sherwin
	 
	Color: China Doll SW1101
	

	Breakfast Area
	 
	Knockdown
	Williams
	Finish: Acrylic Eggshell Enamel
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	Ceilings - Lobby,
	P2A
	P. On Gyp
	Sherwin
	 
	Color: Luminous White - SW1900
	

	Meting Room,
	 
	Board
	 
	Williams
	Finish: Matte Finish
	

	Meeting Room
	 
	 
	 
	
	
	 
	
	

	Prefunction,
	 
	
	 
	 
	
	
	
	

	All Public Areas
	 
	 
	 
	
	 
	
	

	
	 
	 
	 
	 
	
	 
	
	

	
	 
	 
	 
	 
	
	 
	
	

	
	 
	 
	 
	 
	
	 
	
	

	Guest Bath 
	P2B
	P. On Gyp
	Sherwin
	 
	Color: Luminous White - SW1900
	

	Ceiling And
	 
	Board
	 
	Williams
	Finish: Acrylic Semi-Gloss
	

	Back Of House
	 
	
	 
	
	
	 
	
	

	Ceiling 
	 
	 
	
	 
	
	
	 
	
	

	
	 
	 
	
	 
	
	
	 
	
	

	
	 
	 
	
	 
	
	
	 
	
	

	Guest Room -
	P2C
	P. On
	 
	Sherwin
	 
	Color: Luminous White - SW1900
	

	Above Ceiling
	 
	Gyp Board
	Williams
	Finish: Latex Flat
	

	Moulding
	
	 
	 
	
	
	 
	
	

	
	 
	 
	 
	 
	
	 
	
	

	Guest Room,
	P3
	P. On
	 
	Sherwin
	 
	Color: Luminous White - SW1900
	

	Ceiling
	 
	 
	Popcorn 
	Williams
	Finish: Latex Flat
	
	

	
	 
	 
	Texture
	 
	
	
	 
	
	

	
	 
	 
	 
	 
	
	
	 
	
	

	Lobby,
	 
	P4A
	Paint
	 
	Sherwin
	 
	Color: Luminous White - SW1900
	

	Meeting Room,
	 
	Wood Trim
	Williams
	Finish: Semi-Gloss
	

	Meeting Room
	 
	 
	 
	
	 
	
	

	Corridor, Board
	 
	 
	 
	
	 
	
	

	Room, Men's, 
	 
	 
	 
	
	 
	
	

	Ladies Room,
	 
	 
	 
	
	 
	
	

	Corridor Side
	 
	 
	 
	
	 
	
	

	Guestroom
	 
	 
	 
	
	 
	
	

	Door/Frame
	 
	 
	 
	
	 
	
	

	
	 
	 
	 
	 
	
	 
	
	

	Guest Room
	P4B
	P. On Wood
	Sherwin 
	Color: Luminous White - SW1900
	

	Bath Room - 
	 
	 
	 
	Williams
	Finish: Semi-Gloss
	

	Door/Door Frame,
	 
	 
	 
	
	 
	
	

	Ladies Room,
	 
	 
	 
	
	 
	
	 
	

	Room Side of 
	 
	 
	 
	
	 
	
	

	Guestroom Door
	 
	 
	 
	
	 
	
	

	
	 
	 
	 
	 
	
	 
	
	

	Back of the
	P5
	Paint
	 
	Sherwin
	 
	Finish: Epoxy paint grey
	

	House - Floor
	 
	 
	 
	Williams
	
	

	
	 
	 
	 
	 
	
	 
	
	

	Back of the
	P6
	Paint
	 
	Sherwin
	 
	Superpaint
	
	

	House - Walls
	 
	 
	 
	Williams
	Color: China Doll SW1101
	

	
	 
	 
	 
	 
	
	 
	Finish:  Acrylic Eggshell Enamel
	

	
	 
	 
	 
	 
	
	 
	
	

	Lobby Mat - Main
	C6
	Pedimat
	American Floor
	Austin, Capri, Grande Schemes:
	

	Entrance Vestibule
	 
	
	 
	Products Co, Inc.
	Product: Anodized Aluminum Metal
	

	
	 
	 
	 
	 
	301-987-0490
	Stratoflex (UX280) W/ Carpet Insert
	

	
	 
	 
	 
	 
	800-342-0424
	Color: #9310 Spruce
	

	
	 
	 
	 
	 
	301-987-0422 fx
	
	

	
	 
	 
	 
	 
	Marcos Reyes
	For Dallas Scheme:
	

	
	 
	 
	 
	 
	
	 
	Product: Anodized Aluminum Metal
	

	
	 
	 
	 
	 
	
	 
	Stratoflex (UX280) W/ Carpet Insert
	

	
	 
	 
	 
	 
	
	 
	Color: #9122 Denim
	

	
	 
	 
	 
	 
	
	 
	
	

	Guestroom
	CG1
	Corner Guard
	Johnsonite
	Size: 3/4" x 3/4"
	
	

	
	 
	 
	
	 
	817-461-6404
	Color: Match City Loft
	

	
	 
	 
	 
	 
	800-899-8916
	Weight: 75 oz per ft.
	

	
	 
	 
	 
	 
	Peter Button
	Pattern: Saturn (CG-2163)
	

	
	 
	 
	 
	 
	
	 
	*Use parabond Adhesive
	

	
	 
	 
	 
	
	 
	
	

	Corridor
	CG2
	Corner Guard
	Johnsonite
	Size: 1 1/2  " x 1 1/2"
	

	
	 
	 
	 
	 
	
	 
	           Vinyl Corner Guard
	

	
	 
	 
	 
	 
	
	 
	Color: Corner Wall Guard To 
	

	
	 
	 
	 
	 
	
	 
	             Match to Paint - China Doll
	

	
	 
	 
	 
	 
	
	
	             SW1101
	
	

	
	
	 
	 
	 
	
	 
	              VBGXX-A48
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	ALTERNATE FINISHES
	

	
	
	
	
	
	
	
	
	

	Front Desk
	GR1
	Granite
	 
	Alamo Tile Co.
	Color:  Absolute Black #2201
	

	
	
	 
	 
	 
	214-638-8350 ph
	Thickness:  1" Slab
	

	
	
	 
	 
	 
	214-638-8356 fx
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	Guestroom 
	VF
	Wall Vinyl
	MDC
	 
	Pattern: BBAN15 / 4610
	

	
	
	 
	 
	 
	Wallcoverings
	Color: Antique Porcelain
	

	
	
	 
	 
	 
	 
	 
	
	Calm Neutral
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	Guestroom
	VFB
	Wall Vinyl Border
	Seabrook
	Pattern: AN9739AB
	

	
	
	 
	 
	 
	Carylind
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	Suite Millwork to
	PL5
	Plastic Lam
	Formica
	Pattern: 33-58 Sail White Oxide
	

	Match to VF Vinyl
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	
	
	 
	 
	 
	
	 
	
	
	
	

	As An Alternate to paint
	 
	 
	 
	 
	 
	 
	 
	 
	

	* Guestroom can have Vinyl Wallcovering VF and Border VFB if Hotel is not in humid temperature
	 
	 
	 
	
	
	 
	
	

	
	
	
	
	
	
	
	
	
	
	
	


	
	
	
	
	
	
	
	
	210.03
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	LAQUINTA EXTERIOR PAINT SPECIFICATIONS (Feb 1, 2003)
	
	
	
	
	
	
	
	
	
	
	
	 
	 
	ArmorSeal Tread-Plex
	 
	 

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	REFER TO LA QUINTA FINISH MATERIALS FOR INTERIOR COLORS
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Name:
	La Quinta Inn Frosty White LQ5006 
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Product:
	SuperPaint Exterior Gloss Extra White
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	REX:
	A84W51
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Area
	Main Wall Color
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	Y3-4
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	N1-2
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	R4-+01
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	 
	W1-2
	One Gallon Formula
	 
	 
	 
	 
	 
	 
	 
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Name:
	La Quinta Inn Pueblo LQ1079
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Product:
	SuperPaint Exterior Gloss Extra White
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	REX:
	A84W51
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Area
	Wall/Window Trim
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	R2-4
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	B1-7+1
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	 
	Y3-29+1
	One Gallon Formula
	 
	 
	 
	 
	 
	 
	 
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	La Quinta Inn Only
	
	
	
	
	La Quinta Inn Only
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Name:
	La Quinta Inn Patina Green LQ77
	
	Name:
	La Quinta Inn Patina Green LQ77
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Product:
	SuperPaint Exterior Gloss Extra White
	
	Product:
	Industrial Enamel Pure White
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	REX:
	A84W51
	
	
	
	
	REX:
	B54W101
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Area
	Fascia/Gutter
	
	
	
	Area
	Top Rail/Doors
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	B1-49
	
	
	
	
	
	Y3-9
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	L1-34
	
	
	
	
	
	L1-27
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	G2-47
	
	
	
	
	
	G2-36
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	 
	Y3-13
	One Gallon Formula
	 
	 
	 
	B1-38
	One Gallon Formula
	 
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	La Quinta Inn & Suites Only
	
	
	
	La Quinta Inn & Suites Only
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Name:
	La Quinta Inn Trinidad LQ1109
	
	Name:
	La Quinta Inn Trinidad LQ1109
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Product:
	Industrial Enamel Ultradeep
	
	
	Product:
	A-100 Satin Deeptone
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	REX:
	B54W113
	
	
	
	
	REX:
	A82W53
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Area
	Gutters/Metal
	
	
	
	Area
	Fascia
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	B1-12+11
	
	
	
	
	
	B1-17
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	G2-27
	
	
	
	
	
	G2-36
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	Y3-12
	One Gallon Formula
	
	
	
	Y3-16
	One Gallon Formula
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	 
	 
	 
	 
	 
	 
	 
	 
	 
	 
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Name:
	La Quinta Inn Bronze LQ9800
	
	
	Name:
	La Quinta Inn Roof Tile Red
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Product:
	Industrial Enamel Ultradeep
	
	
	Product:
	Metalatex Ultradeep
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	REX:
	B54T104
	
	
	
	
	REX:
	B42T104
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Area
	Handrail Pickets
	
	
	
	Area
	Roof
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	(Environtoners)
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	R2-14
	
	
	
	
	
	R102-15
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	Y3-51
	
	
	
	
	
	B100-2
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	B1-2y30
	
	
	
	
	
	R101-20
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	W1-4y48
	One Gallon Formula
	
	
	
	W100-11
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	Y101+1
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	 
	 
	 
	 
	 
	 
	 
	Y100-1
	One Gallon Formula
	 
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	Walks and Decks
	H&C Concrete Stain Silicone Acrylic - Package Color: Silver Gray
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	NOTE THESE FORMULAS ARE FOR COLOR ONLY.  SELECTION OF THE PROPER BASE 
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	PRODUCT FOR SPECIFIC APPLICATIONS IS THE RESPONSIBILITY OF THE APPLICATOR.
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	

	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	
	


[image: image10.png]

















210.04

La Quinta Inns Carbon Monoxide Detector Project

STANDARD INSTALLATION (1) CARBON MONOXIDE DETECTOR

QTY.
DESCRIPTION

1
MA-CM15A CARBON MONOXIDE DETECTOR W/SOUNDER

1


IV-FA250 HIGH POWER TRANSMITTER

1
IV-FA416DR 16 ZONE RECEIVER W/DISPLAY

1
AC-SPX5760 ANNUCIATOR PLATE

1
SCN RB-5 AUXILLIARY RELAY MODULE

2
POWER SUPPLY DC, 12V 1-AMP

2
BATTERY 12V 7AH

2
TRANSFORMER


ANNUAL MAINTENANCE AND ANNUAL INSPECTION



IS REQUIRED

Presented By:

Mike Korbuly

ADT National Account Manager

Direct: (972) 406-4211















[image: image11.png]



210.04

La Quinta Inns Carbon Monoxide Detector Project

INSTALL HIGH POWER REPEATER FOR LONG RANGE SIGNAL OF CARBON MONOXIDE DETECTOR TO RECEIVER

QTY.
DESCRIPTION

1
IV-FA575 HIGH POWER REPEATER

1


IV-BAT603 BACKUP BATTERY


ANNUAL MAINTENANCE AND ANNUAL INSPECTION 


IS REQUIRED


Presented By:

Mike Korbuly

ADT National Account Manager

Direct: (972) 406-4211















[image: image12.png]



210.04

La Quinta Inns Carbon Monoxide Detector Project

INSTALL (1) ADDITIONAL CARBON MONOXIDE DETECTOR

QTY.
DESCRIPTION

1
MA-CM15A CARBON MONOXIDE DETECTOR W/SOUNDER

   1


IV-FA250 HIGH POWER TRANSMITTER

1
POWER SUPPLY DC, 12V 1-AMP

1
BATTERY 12V 7AH

1
TRANSFORMER


ANNUAL MAINTENANCE ANNUAL INSPECTION 



IS REQUIRED

Presented By:

Mike Korbuly

ADT National Account Manager

Direct: (972) 406-4211

210.05.1
[image: image13.png]X600
XB00A

STANDARD DOUBLE GUESTROOM

Al

W] 7o MY X501 ﬁ's@oa
X703 100 ( @
X100A X204
X606
Q X205
Vg \_
]
l\@ X405
X218 X101AD
X601 X101ADB
X101D
X101BD |
i, |
o 04/ A402N X202 °®)
< ? X604
-
< <C X605 @
3 o6/a1023 <
-
! X443
<D & S
X504
X200
X-001/C1
X-001A/C1A X101AD
X-001B/C1B X101ADB
X101D
X101DR
il FULL LENGTH MIRROR " Y
X50

X700
X701
X702
X723

MARBLE
THRESHOLD—_|

X421

TILE

: CARPET]

X201DA

X404

X201DA

@ g
al@,unmfta

. INN & SUITES

La ulnta Inn and Sltes

LA QUINTA INN & SUITES — ROOM FURNITURE LAYOUT

6/13/03




210.05.2[image: image14.png]W x704 T T xz18
X703 X100 X601 /o
X100A \@‘
x405
X501 ()
XY GAAIDS
g\ XM | -,
% X403 \@=
g x202 | °

r 3
Xe00A X101K
X101BK
X101AK

~ X101AKB
=@ X403

x200 | 06/ A403
|| X504 -
\_ 2

X201KA

X001/C1 X605
Full Length Mirror X001A/C1A (s
X0018/C1B X403

: &
X500 X202 ' j

701 |
x700A |
X702 |
X723 |
L

MARBLE
THRESHOLD R MARBLE
X421 X401 THRESHOLD
- CARPETUTILE. e O[
° CT1
CcT1B

s 1 R NG GUESTROOM 13 1nta81tes

LA QUINTA INN & SUITES — ROOM FURNITURE LAYOUT

- Jovnen Jonos





210.05.3

[image: image15.png]WWHT _ x7oa [ _ W X500
X703 %100
X100A X604 | o
x403 | (D7
X202 N
4 Y
%888 X101K
X101KR
X101AK s
. X101AKB x
N2 3
- X
3 X200
)
x X403
\_ L]
X218
X601 X605 (o
X403 \@=
X202 ~
X209
X001/C1
X001A/C1A X60!
X001B/C1B @‘ 403
Full Length Mirror *“/ X501
|
701 |
x700A |
X702 |
X723 ‘
; |
THRESHOLD R MARBLE
X421 X401 THRESHOLD
_¢_ CARPETLTILE _¢_ Q[
cT
CcT1B
O

@ _
LaQuinta

INN & SUITES

g:i;” 9£\j é L1 '_OS'-TAN DARD KING La Quinta Inn and Suites

LA QUINTA INN & SUITES — ROOM FURNITURE LAYOUT
12/10/03




210.05.4

[image: image16.png]W] X704 I
X703 X100 X405 @
X100A
S Sam———
I X215
X601 "
| 2
P >
/A404
X403
606
__ _______
*
3 < < X202
x 3 L3 X605
X204 - ~]

X600
X600A N
3 S
05‘A404 X
' X101K
X101AK
o1 )
JFULL LENGTH MIRROR 403
X001/C1 o5
X001A/C1A X500
X001B/C1B — —

—t—— MARBLE
THRESHOLD

KING SOFA ROOM

SCALE: 1/4” =1'-0" La tdSUIteS

LA QUINTA INN & SUITES — ROOM FURNITURE LAYOUT
6/13/03




210.05.5

[image: image17.png]! 1l 1]
W X704 e

X703 X100
X100A

i,
X210 04/ A405

- X301
2 } X217
X
“ X210
f
2
X601 <
Jgot {8 X213L0
& -¢-xe06
)
00 X209
X600A S
<} X403
® o 22 (O |—=t
, X604 9
X200 ( 1)
504
06/ A405 s L
| X101AK 3
%403 X101AKB
X101K
X101KR
\_ )
FULL LENGTH MIRROR X403 S
X511 X202 (@ =
X001/C1 X605
X001A/C1A X500 i
X001B/C1B

s
= i I ] L
EXTENDED KING SUITE La u1t81tes

LA QUINTA INN & SUITES — ROOM FURNITURE LAYOUT
6/13/03




210.05.6

[image: image18.png]1 =

X704
X703 X100
X100A >;‘L°.3
(®F
(P |xae = 204
) X300 Q< =
-3 o
p B I - x204 X
X606 )
X600A '
X600

X505

07/A406
0g/A406
X704

4 :
X421 g
X221 -
X001/C1 -
X001A/C1A o
X001B/C1B 4 X101K A 8
X101KR X
X101AK
X101AKB

INN & SUITES

TWO ROOM SUITE La ultes

INTERIOR CORRIDOR PROPERTY — ROOM FURNITURE LAYOUT
6/13/03





[image: image19.jpg]LaQuinta

Inns - Inn & Suites





FINISH SELECTION AND FF&E SPECIFICATION BOOK INDEX



	CARPET
	
	

	X-001/C1
	Varies per Design Scheme
	GUEST ROOM CARPET

	X-001A/C1A
	
	GUEST ROOM CARPET PAD

	X-001B/C1B
	Varies per Design Scheme
	GUEST ROOM CARPET BASE

	X-002/C2
	Varies per Design Scheme
	CORRIDOR CARPET

	X-002A/C2A
	
	CORRIDOR CARPET PAD

	X-002/C2B
	Varies per Design Scheme
	CORRIDOR CARPET BASE

	X-003/C3
	Varies per Design Scheme
	LOBBY CARPET

	X-003/C3A
	
	LOBBY CARPET PAD

	X-003/C3B
	Varies per Design Scheme
	NOT USED

	X-004/C4
	Varies per Design Scheme
	EXERCISE ROOM CARPET

	X-004/C4B
	Varies per Design Scheme
	EXERCISE ROOM CARPET BASE

	X-005/C5
	Varies per Design Scheme
	BREAKFAST, MEETING ROOM, BOARD ROOM, 

	
	
	ASSEMBLY, FRONT DESK & OFFICE CARPET

	X-005A/C5A
	
	BREAKFAST, MEETING ROOM, BOARD ROOM,

	
	
	ASSEMBLY, FRONT DESK & OFFICE CARPET PAD

	
	
	

	WINDOW TREATMENT/SHOWER CURTAIN

	X-100 
	Varies per Design Scheme
	GUESTROOM DRAPERY, INCLUDING VALANCE, SIDE

	
	
	PANEL AND OVERDRAPERY WITH BLACKOUT.

	X-100A
	Varies per Design Scheme
	GUESTROOM DRAPERY HARDWARE

	X-101A
	
	NOT USED

	X-102
	
	SHOWER CURTAIN

	X-103
	
	SHEER CURTAIN FOR CONNECTING DOOR & SIDE

	
	
	PANEL IN TWO BAY SUITE, BUSSINESS CENTER &

	
	
	CORRIDOR WINDOWS

	X-104
	Varies per Design Scheme
	LOBBY & BREAKFAST ROOM DRAPERY

	X-104A
	Varies per Design Scheme
	LOBBY & BREAKFAST ROOM SHEER

	X-105
	Varies per Design Scheme
	MEETING ROOM & BOARD ROOM DRAPERY

	X-105A
	
	NOT USED

	X-106
	
	WOOD BLINDS

	X-107
	
	SHOWER CURTAIN UPGRADE


	BEDDING
	
	

	X-101K
	Varies per Design Scheme
	KING COVERLET

	X-101AK
	
	KING MATTRESS, BOXSPRING

	X-101AKB
	
	KING MATTRESS BASE

	X-101KR
	Varies per Design Scheme
	KING DUST RUFFLE

	X-101D
	Varies per Design Scheme
	DOUBLE COVERLET

	X-101AD
	
	DOUBLE MATTRESS, BOXSPRING

	X-101ADB
	
	BOUBLE BOXSPRING BASE

	X-101DR
	Varies per Design Scheme
	DOUBLE DUST RUFFLE

	X-101KHC
	
	ADA KING MATTRESS, BOXSPRING

	X-101KHCF
	
	ADA KING MATTRESS, BOXSPRING W/CASTERS

	X-101DHC
	
	ADA DOUBLE MATTRESS, BOXSPRING

	X-101DHCF
	
	ADA DOUBLE MATTRESS, BOXSPRING W/CASTERS

	
	
	

	FURNITURE – GUESTROOM

	X-200
	
	DRESSER 60”L X 22”D X 24”H

	X-201D
	
	DOUBLE HEADBOARD

	X-201DA
	
	DOUBLE HEADBOARD ARCHED

	X-201K
	
	KING HEADBOARD

	X-201KA
	
	KING HEADBOARD ARCHED

	X-202
	
	NIGHTSTAND 24” X 16” X 24”

	X-203
	
	NIGHTSTAND 16” X 16” X 22”

	X-204
	
	DESK RECTANGULAR 48”W X 26”D X 30”H

	X-205
	Varies per Design Scheme
	DESK CHAIR W/ARMS & W/CASTERS

	X-206
	
	NOT USED (DRESSER 71.5”L X 22”D X 24”H)

	X-207
	
	NOT USED (FRAMED MIRROR)

	X-207A
	
	NOTE USED (FRAMED MIRROR ARCHED)

	X-209
	Varies per Design Scheme
	DESK CHAIR HI-BACK

	X-210
	Varies per Design Scheme
	LOUNGE CHAIR

	X-211
	Varies per Design Scheme
	RECLINER CHAIR

	X-212
	Varies per Design Scheme
	STRAIGHT BACK DESK CHAIR

	X-213L
	
	P-TOP DESK LEFT – 42”DIA X 66”L

	X-213R
	
	P-TOP DESK RIGHT – 42” DIA X 66”L

	X-214
	Varies per Design Scheme
	LOVE SEAT SLEEPER – 60”L

	X-215
	
	COFFEE TABLE

	X-216
	
	END TABLE

	X-217
	
	ROUND TABLE – 24”DIA

	X-218
	
	MICRO-FRIDGE CABINET

	X-219
	Varies per Design Scheme
	SOFA SLEEPER – 73” L

	X-220
	
	THREE-DRAWER DRESSER

	X-221
	Varies per Design Scheme
	TWO-BAY SUITE UPHOLSTERED BENCH

	X-222
	Varies per Design Scheme
	KING BARRELL CHAIR

	X-245
	
	TV ARMOIRE

	X-250
	
	DESK 42’W X 24”D X 30”H

	X-262L
	
	P-TOP DESK LEFT – 36”DIA X 60”L

	X-262R
	
	P-TOP DESK RIGHT – 36”DIA X 60”L

	
	
	

	FURNITURE – PUBLIC SPACE

	X-208
	Varies per Design Scheme
	BREAKFAST ROOM CHAIR W/ARMS

	X-223
	Varies per Design Scheme
	LOBBY SOFA

	X-224
	Varies per Design Scheme
	LOBBY BARREL CHAIR

	X-225
	
	LOBBY END TABLE

	X-226
	
	LOBBY COCKTAIL TABLE

	X-227
	Varies per Design Scheme
	BREAKFAST ROOM CHAIR W/O ARMS

	X-228
	Varies per Design Scheme
	BREAKFAST ROOM TABLE 30” DIA

	X-228A
	
	NOT USED

	X-229
	Varies per Design Scheme
	BREAKFAST ROOM TABLE 24”DIA

	X-229A
	
	NOT USED

	X-230
	Varies per Design Scheme
	BREAKFAST AREA LOVESEAT

	X-231
	Varies per Design Scheme
	OVERFLOW BREAKFAST TABLE 30” X 30”

	X-232
	
	SOFA TABLE

	X-233
	
	DECORATIVE MIRROR

	X-234
	
	MEETING ROOM PRESENTATION CABINET

	X-235
	
	BOARD ROOM TABLE

	X-236
	Varies per Design Scheme
	BOARD ROOM EXECUTIVE CHAIRS

	X-240
	Varies per Design Scheme
	LOBBY BENCH

	X-244
	Varies per Design Scheme
	OFFICE/BUSINESS CENTER CHAIR

	X-251
	
	DECORATIVE NICHE

	X-252
	
	DECORATIVE MIRROR

	X-255
	
	LOBBY ROUND TABLE

	
	
	

	FURNITURE – BACK OF HOUSE / OFFICES

	X-241
	
	OFFICE DESK

	X-242
	
	COMPUTER TABLE

	X-243
	Varies per Design Scheme
	MANAGER’S DESK CHAIR

	X-261
	Varies per Design Scheme
	SIDE CHAIR W/ARMS

	
	
	

	FURNITURE – SWIMMING POOL / COURTYARD

	X-246
	
	SWIMMING POOL CHAISE LOUNGE

	X-247
	
	SWIMMING POOL UMBRELLA TABLE 42”DIA

	X-247A
	
	SWIMMING POOL UMBRELLA

	X-247B
	
	BASE FOR MARKET UMBRELLA

	X-248
	
	SWIMMING POOL CHAIR

	X-249
	
	SWIMMING POOL COCTAIL TABLE

	X-253
	
	COURTYARD BENCH

	X-254
	
	PATIO DINING CHAIR

	X-256
	
	SWIMMING POOL TABLE W/O UMBRELLA

	X-263
	
	INDOOR SWIMMING POOL CHAIR

	X-263A
	
	INDOOR SWIMMING POOL CHAIR

	X-264
	
	INDOOR SWIMMING POOL TABLE 36” DIA

	X-264A
	
	INDOOR SWIMMING POOL TABLE 36” DIA

	X-265
	
	INDOOR SWIMMING POOL TABLE 24” DIA

	X-265A
	
	INDOOR SWIMMING POOL TABLE 24” DIA

	
	
	

	PLANTERS / TRASH RECEPTACLES

	X-237
	
	TRASH RECEPTACLE – EXTERIOR

	X-238
	
	ASH RECEPTABLE – EXTERIOR

	X-239
	
	TRASH RECEPTABLE – INTERIOR

	X-259
	
	EXTERIOR PLANTER

	X-260
	
	EXTERIOR PLANTER

	
	
	

	LIGHTING - GUESTROOMS

	X-300
	
	SUITE TABLE LAMPS

	X-301
	
	SUITE TABLE LAMPS

	X-400
	
	GUESTROOM ENTRY WALL SCONCE

	X-401
	
	GUESTROOM BATH CEILING LIGHT

	X-402
	
	GUESTROOM & PUBLIC TOILET VANITY LIGHT

	X-403
	
	SINGLE HEAD WALL SCONCE

	X-404
	
	DOUBLE HEAD WALL SCONCE

	X-405
	
	FLOOR LAMP W/GLASS TRAY

	X-406
	
	GUEST BATHROOM VANITY LIGHT 37.5”

	X-421
	
	GUESTROOM ENTRY SURFACE MTD. CEILING FIXTURE

	
	
	

	LIGHTING – PUBLIC SPACE

	X-302
	
	LOBBY TABLE LAMPS

	X-303
	
	LOBBY TABLE LAMPS

	X-306
	
	LOBBY TORCHIERE

	X-407
	
	CORRIDOR CEILING MTD. LIGHT

	X-408
	
	CORRIDOR WALL SCONCE

	X-408A
	
	CORRIDOR WALL SCONCE - ALTERNATE

	X-409
	
	LOBBY & ELEV. LOBBY WALL SCONCE

	X-410
	
	LOBBY WALL SCONCE

	X-411
	
	MEETING ROOM & BOARD ROOM WALL SCONCE

	X-412
	
	BOARD ROOM HANGING LAMP

	X-413
	
	LOBBY HANGING LAMP

	X-414
	
	BREAKFAST ROOM, ELEV. LOBBY SURFACE MTD.

	
	
	CEILING LIGHT

	X-415
	
	EXTERIOR WALL SCONCE

	X-416
	
	SWIMMING POOL EXTERIOR POST LIGHT

	X-416A
	
	POST FOR POOL LIGHT

	X-417
	
	MEETING ROOM SURFACE MTD. CEILING FIXTURE

	X-418
	
	MEETING ROOM, ASSEMBLY & UPPER FLOOR ELEVATOR

	
	
	LOBBY WALL SCONCE

	X-419
	
	HANGING LAMP

	X-420
	
	BUSINESS CENTER SURFACE MTD. CEILING FIXTURE

	X-422
	
	WALL SCONCE AT LOBBY BULKHEAD

	X-423
	
	STAIRWELL WALL SCONCE

	X-424
	
	CORRIDOR WALL SCONCE

	X-426
	
	MEETING ROOM FLOURESCENT PARABOLIC STRIP LT.

	
	
	

	ARTWORK / ACCESSORIES

	X-500
	Varies per Design Scheme
	GUESTROOM ARTWORK

	X-501
	Varies per Design Scheme
	GUESTROOM ARTWORK

	X-502
	Varies per Design Scheme
	GUESTROOM ARTWORK

	X-503
	Varies per Design Scheme
	GUESTROOM ARTWORK

	X-504
	Varies per Design Scheme
	GUESTROOM ARTWORK

	X-505
	Varies per Design Scheme
	GUESTROOM ARTWORK

	X-506
	Varies per Design Scheme
	PUBLIC SPACE ARTWORK

	X-507
	Varies per Design Scheme
	PUBLIC SPACE ARTWORK

	X-508
	Varies per Design Scheme
	PUBLIC SPACE ARTWORK

	X-509
	Varies per Design Scheme
	PUBLIC SPACE ARTWORK

	X-510
	Varies per Design Scheme
	PUBLIC SPACE ARTWORK

	X-511
	
	FULL LENGTH MIRROR

	X-512
	Varies per Design Scheme
	PUBLIC SPACE ARTWORK

	X-513
	Varies per Design Scheme
	PUBLIC SPACE ARTWORK

	X-514
	
	SILK FLOWER

	X-515
	Varies per Design Scheme
	PUBLIC SPACE ARTWORK

	X-516
	Refer Interior Signage Package in Design & Construction Standards
	LA QUINTA INN & SUITES LOGO

	X-517
	Refer Interior Signage Package in Design & Construction Standards
	LA QUINTA INN LOGO

	X-518
	
	SILK FLOWER

	X-519
	
	SILK TREE

	X-520
	
	SILK TREE

	
	
	

	MISCLLANEOUS - GUESTROOM

	X-600
	
	TELEVISION – 27”

	X-600A
	
	TELEVISION SWIVEL MOUNT

	X-601
	
	MICRO-FRIDGE UNIT

	X-603
	
	COFFEE MAKER

	X-604
	
	CLOCK RADIO

	X-605
	
	BEDSIDE TELEPHONE

	X-606
	
	SPEAKER TELEPHONE

	X-607
	
	HAIR DRYER

	X-608
	
	TOILET PAPER HOLDER

	X-609
	
	SHOWER CURTAIN ROD

	X-610
	
	TOWEL BAR

	X-611
	
	DECORATIVE TOWEL SHELF

	X-612
	
	FASCIAL TISSUE DISPENSER

	X-613
	
	ROBE HOOK

	
	
	

	X-700A
	
	SUIT HANGER MINI HOOK

	X-700B
	
	SKIRT HANGER MINI HOOK

	X-701
	
	LUGGAGE RACK

	X-702
	
	IRON, IRONING BOARD & HOOK

	X-703
	
	PTAC

	X-704
	
	PTAC DEFLECTOR

	X-723
	
	COAT CLOSET ROD

	X-742
	
	GUESTROOM DOOR LOCKS

	

	MISCALLANEOUS - OTHER AREAS

	X-614
	
	TELEVISION – 32”

	X-615
	
	READER BOARD

	X-616
	
	EXERCISE EQUIPMENT

	
	
	

	X-705
	
	LUGGAGE CARTS

	X-706
	
	SECURITY CAMERA

	X-707
	
	SECURITY CAMERA MONITOR

	X-708
	
	SECURITY CAMERA SWITCHER

	X-708A 
	
	SECURITY CAMERA CASSETTE RECORDER

	X-709
	
	EMPLOYEE LOCKERS

	X-710
	
	VCR

	X-711
	
	PIX CART WITH CASTERS

	X-712
	
	SPEAKER PODIUM

	X-713
	
	STACK CHAIR CADDY

	X-714
	
	PERSONAL COMPUTER – BUSINESS CENTER

	X-715
	
	COPIER / FAX

	X-716
	
	HOUSEHOLD REFRIGERATOR – BREAK ROOM

	X-717
	
	MEETING ROOM TABLE 18” X 72”

	X-718
	
	MEETING ROOM TABLE – 60” DIA

	X-719
	
	STACK CHAIR – MEETING ROOM / BREAK ROOM

	X-720
	
	MEETING ROOM TABLE STACK CADDY

	X-721
	
	PLANTER – 24”

	X-722
	
	PLANTER – 36”

	X-724
	
	CONTINENTAL BREAKFAST DISPLAY EQUIPMENT

	X-725
	
	OVERHEAD PROJECTOR

	X-726
	
	PORTABLE COAT RACK

	X-727
	
	50 LB. WASHER – LAUNDRY

	X-728
	
	75 LB. DRYER – LAUNDRY

	X-729
	
	ICE MACHINE – VENDING ROOM

	X-730
	
	MICROWAVE – BREAK ROOM

	X-731
	
	ROOM ATTENDANT CART

	X-732
	
	COMMERCIAL REFRIGERATOR – PANTRY

	X-733
	
	COMMERCIAL FREEZER – PANTRY

	X-734
	
	COMMERCIAL DISHWASHER – PANTRY

	X-735
	
	PRE SOAK WASHER

	X-736
	
	GUEST LAUNDRY WASHER

	X-737
	
	GUEST LAUNDRY DRYER

	X-738
	
	NOT USED

	X-739
	
	HOUSEKEEPING DESK

	X-740
	
	FILE CABINET

	X-741
	
	BREAK ROOM TABLE

	X-743
	
	SAFE

	X-744
	
	SAFE DEPOSIT BOXES

	X-745
	
	LAUNDRY CARTS

	
	
	






































































































































































































































































[image: image25.png]LA QUINTA INN OR , ¢ WOOD MOULDING, TYP.
LA QUINTA INN & SUITES

SIGN BY OWNER

LOBBY WALL
N S SCONCE, TYP. I

210.06.1

o
| 1
|

H/C ACCESSIBLE WOOD CHAIR RAIL,
COUNTER TYP.

1. REGISTRATION FRONT DESK ELEVATION
SCALE: 1/4” =1-0

»”

WOOD MOULDING, TYP.
WOOD CHAIR RAIL, TYP.
BACK SPLASH

IIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIIII

2. BREAKFAST AREA BUFFET ELEVATION “-Bast
SCALE: 1/4” =1-0"

NOTE:

1. MINIMUM LENGTH OF THE BREAKFAST BUFFET COUNTER NEEDS TO BE 16'-0".

2. RFERE LQ- FINISH MATERIAL LIST FOR DETAILS OF THE FINISHES.

3. IF BREAKFAST ROOM MILLWORK EXISTS, THEN MATCH WITH SIMILAR; IF NOT CONSTRUCT NEW PER DETAIL.
4. FINISH MAY BE ADJUSTED TO MATCH EXISTING DECOR. LQI APPROVAL REQUIRED.

La Quinta Inn And
La Quinta Inn & Suites
Design Standards

12/18/03




[image: image26.png]108"

3)—031

Y,

I La
|

Uta

12/18/03

210.06.2
UNDER COUNTER LIGHT FIXTURE
‘21-111)
COUNTERTOP

ALL INTERIOR PANELS TO BE
MELAMINE COMPONENT PANELS

T S s S

WIRE MANAGEMENT TROUGH

1 1/2" HOLE WITH GROMMET

PLASTIC LAMINATE
OPEN ELECTRICAL RACEWAY

e " 3" X 8 OPENING FOR ACCESS TO WIRE WAY
3/4" SUBSTRATE TOE KICK ADJUSTABLE SHELF WITH LIP AT BACK EDGE

3. FRONT DESK SECTION ‘

SCALE: 3/4" = 1'-0" REFER TO LA QUINTA PROTOTYPE

FOR COMPLETE DETAILS

COUNTERTOP

ALL INTERIOR PANELS TO BE
MELAMINE COMPONENT PANELS

@ : OPEN COMMUNICATION WIRE WAY
TACC i ISOLATED GROUND DEDICATED CIRCUIT OUTLET
ELECTRONIC
TIMED SAFE 5 PLASTIC LAMINATE

1 1/2" HOLE WITH GROMMET
OPEN ELECTRICAL RACEWAY

BASE

3" X 8" OPENING FOR ACCESS TO WIRE WAY
4, FRONT DESK SECTION
SCALE: 3/4" = 1'-0"
La Quinta Inn And
La Quinta Inn & Suifes INN & SUITES
Design Standards





[image: image27.png]210.06.3

PAINTED WOOD HALF ROUND
N

B X 10" DECORATIVE —‘
TILE BACK SPLASH .

112 CORIAN ‘
ON 3/4" PLYWOOD N

t\' l e e o e e ,,._,.’.,,;',_'v. il
4" X 4" TILE 2|

PLASTIC LAMINATE
ON 3/4" PLYWOOD

6" X 12" TILE BASE

5. BREAKFAST BUFFET SECTION
SCALE: 3/4" =1'-0"

NOTE:
1. MINIMUM LENGTH OF THE BREAKFAST BUFFET COUNTER NEEDS TO BE 16'-0".
2. REFER LQ - FINISH MATERIAL LIST FOR DETAILS OF FINISHES
3. IF BREAKFAST ROOM MILLWORK EXISTS, THEN MATCH WITH SIMILAR;
IF NOT CONSTRUCT NEW PER DETAIL.
4. FINISH MAY BE ADJUSTED TO MATCH EXISTING DECOR. LQI APPROVAL REQUIRED.

La Quinta Inn And
La Quinta Inn & Suites | INN & surres ]
Design Standards Ce——

12/04/01




[image: image28.png]210.06.4

NEW SIDE WALLS AND ENCLOSURE
ABOVE THE DOORS

PAIR OF MIN. 14" EMBOSED PANELED
COLONIAL DOORS

1/2" GYP BD ON 2X4 STUDS

2X4 STUDS

2 1/4" PAINTED COLONIAL
WOOD TRIM

PAINTED PANELED WOOD DOOR
IN PREHUNG FRAI

CLOSET ELEVATION DOOR_JAMB DETAIL

SCALE: 1/2" = 1'-0" SCALE: 3" = 1/-0"

La Quinta Inn And
La Quinta Inn & Suites
Design Standards

3

12/18/03




[image: image29.png]210.06.5

APPLIED MOLDING

BOREN MOLDING TYPE "D”
OR APPROVED EQUAL
5/16” x 1-1/8"

BOREN MOLDING COMPANY
4960 CRANSWICK RD.
HOUSTON, TX 77041
713-462-6154

ADA APPROVED CARD ACCESS
PAD AND LEVER

G OF DOOR HANDLE

VERIFY (6'-8” OR 7°-0")

PANEL DOOR APPLIED MOLDING LAYOUT

Scale: 3/4” = 1’-0"

La Quinta Inn And
La Quinta Inn & Suites
Design Standards

12/18/03




[image: image30.png]210.06.6

SEMIRECESSED SIGN
WITH BACK ACCESS

TYPICAL TOWER ELEVATION
SCALE: 1/8" = 1=0° __ CONVERSION.

La Quinta Inn And m@@uﬂmﬂa
La Quinta Inn & Suites L, INN & SUITES

Design Standards

12/18,/03




[image: image31.png]210.06.7

S E

S X
SRR s
D RRRRIRLIRIRY

o 202020202020 0 e 20 %0 %%
R
0020202020202 %
REKEHF
SRIXH

é?’
%
e

v"

AT EH I

<

v.v.;. )
XY

wvz
SRS

S
S

08
S

O

vv
8
%
&

TOWER TRIM DETAIL

PROTOTYPE DESIGN

La Quinta Inn And
La Quinta Inn &.SuHes
Design Standards

@ o
aQuinta|

INN & SUITES

12/18/03




[image: image32.png]210.06.8
210.06.10

wavawas

K/
X
K
<

avavivavivavivivavativavav O 0 00 026

.0
X

O

XX

¢ v"'

7 >

KRS
RO
I0%6%6%6%

£3

1” EIFS ON (2)
LAYERS ON 5/8”
GYP. BD. ON
METAL STUDS

T A AT A A A A A A A ATA XA g

STEEL COLUMN AS
PER STRUCTURAL

210.06.9

XY
. )
OO VAT N A A A N OO WS

CONCRETE PIER
CAP AS PER
STRUCTURAL

EIFS COLUMN BASE
ON CONCRETE PIER
CAP PER
STRUCTURAL

EIFS ON (2) LAYERS
5/8" GYP. BD. ON
METAL STUDS

STEEL COLUMN AS
PER STRUCTURAL

COLUMN PLAN |
PORTE COCHERE COLUMN DETAIL

SCALE: 1 1/2" = 1'-0 PROTOTYPE DESIGN

La Quinta Inn And
La Quinta Inn & Suites
Design Standards

@ N\
LaQuintaj

12/18/03




[image: image33.png]210.06.9

EIFS COLUMN BASE

AS SHOWN ON (2) LAYERS
- 5/8” GYP. BD. ON

METAL STUDS

377

O
&S
@

KT T o
|
e ‘"al v

R

N

COLUMN BASE DETAIL
La Quinta Inn And

La Quinta Inn & Suites
Design Standards

12/18/03




[image: image34.png]210.06.10

S
;

‘4
-

EIFS COLUMN CROWN

AS SHOWN, ON (2) LAYERS
5/8" GYP. BD. ON

METAL STUDS

- ——
—
0,09,:,0 )
| ’0 |
4R -
15"

La 'QuinTo Inn And
La Quinta Inn & Suites
Design Standards

12/18/03


























210.06.11

[image: image45.jpg][T

PAINT BLACK
(MATTE FIN.)

PL. WILSONART

DOOR FIN:

1ST FLOOR:
SATIN BRASS
(BRUSHED
BRASS)

2ND FLOOR:
BAKED ENAMEL
FIN. TO MATCH
PAINT: S.W.
VELLUM -
SW1116

ELEVATION - BACK & SIDE WALLS
N.T.S.

ELEVATION @ DOORS

N.T.S.

ELEVATOR DETAILS
Mt@l La Quinta Inn And
Jl I La Quinta Inn & Suites

Design Standards

COLOR: VERDIGRIS
#4206

#4 BRUSHED
STAINLESS
STEEL

DOOR FIN:
LA QUINTA INNS

BRUSHED
STAINLESS
STEEL

\_ VARIES

LaQuinta

INN & SUITES

12/04/01

LQ-11















10.06.12
[image: image46.jpg]e

— 12" x 12" TO MATCH
LOBBY FLOOR TILE

*ELEVATOR CAB
SIZES VARY

EQ. 1] EQ. 1|} EQ_\ "

EQ. 1y EQ 1qp' EQ

»
4

FLOOR PLAN

PAINT BLACK
(MATTE FIN.)

#35C04 CEILING
{:} Q CONCEALED FRAME
#38 MIRROR BRONZE

Tt
| O s

1
-

REF. CEILING PLAN

ELEVATOR DETAILS

LaQuimta

INN & SUITES

La Quinta Inn And
La Quinta Inn & Suites
Design Standards

12/04/01 LQ-12















210.06.13

[image: image47.jpg]TRANSOM TO MATCH DOORS
(USE AS AN OPTION, .
PREFER DRYWALL ABOVE DOOR)

PAIR OF 14" EMBOSED PANELED
COLONIAL DOORS

I
= 1/2” GYP BD ON 2X4 STUDS
2X4 STUDS
ollo
[—— 2 1/4” PAINTED COLONIAL
WOOD TRIM
™~

PAINTED PANELED WOOD DOOR
IN PREHUNG FRAME

6. CLOSET ELEVATION 7. DOOR JAMB DETAIL

SCALE: 1/2" = 1'-0" SCALE: 3" = 1/-0"

.
Quinta

La Quinta Inn And
La Quinta Inn & Suites
Design Standards

&

12/04,/01















210.07

	La Quinta Preferred Vender and Contractor and Consultant Listing

	La Quinta Inns, Inc. has composed this list of preferred national Venders and Contractors and Consultants for your use.

	

	National companies will tend to offer services across the USA.  Regional companies are better suited in a given City or State or close surrounding areas. 

	

	La Quinta National Venders Accounts

	Product/Service
	Company
	Contact
	National / Regional
	Vendor  /  Contractor / Consultant
	City
	State
	Phone 

	PTAC Systems
	Amana
	Sam Melear
	National
	Vendor
	Arlington
	TX
	817-465-8300

	Corian Vanity
	Vanity & Wet Wall
	Virgil Crane
	National
	Vendor
	San Antonio
	TX
	210-828-6057

	Door Hardware
	Opening Specialties
	Paul Mathis
	National
	Vendor
	San Antonio
	TX
	210-653-1234

	Door Locks
	Ving 
	Bill Mask
	National
	Vendor
	Dallas
	TX
	800-367-8094

	 
	TESA/Entry Systems
	David Ginn
	National
	Vendor
	Norcross
	GA
	800-248-6189

	Laundry Equipment
	Skyline Equipment
	Donna Weaver
	National
	Vendor
	Houston
	TX
	800-444-1227

	Paint
	Sherwin Williams
	Brian Patten
	National
	Vendor
	Houston
	TX
	713-957-4209

	Fire Alarm Systems
	Grinnell
	National Account
	National
	Contractor
	Houston
	TX
	800-299-4377

	Roofing
	Met-Tile
	National Account
	National
	Vender
	 
	 
	800-899-0311

	 
	Garrard Roofing Technologies
	Craig Winebrener
	National
	Vender
	 
	 
	800-752-0274

	 
	Decra-Tile
	Carter-Holt-Harvey Roofing Inc.
	National
	Vender
	Houston
	TX
	713-931-4032

	Pool Decking & Concrete Surfacing
	"Keystone Kool Deck" By: Mortex Manuf. Co.
	 
	National
	Vendor
	 
	 
	800-338-3225

	 
	"Sundeck" By: Sundeck Products Inc.
	 
	National
	Vendor
	Arlington
	TX
	817-265-2406

	Bathtub Surrounds
	Ambath Corp.
	David Sanders
	National
	Contractor
	Phoenix
	AZ
	888-826-2284

	 
	Corian
	Virgil Crane - Wet Wall
	National
	Vender
	San Antonio
	TX
	210-828-6057

	 
	Byars Supply
	Charles Byars 
	National
	Contractor
	Houston
	TX
	713-668-5276

	Elevator 
	Otis
	Local office Rep. (see Yellow Pages)
	Regional
	Contractor
	Farmington
	CT
	800-233-6847

	 
	Schindler
	Local office Rep. (see Yellow Pages)
	Regional
	Contractor
	Morristown
	NJ
	973-397-6500

	FF&E -Casegoods
	American of Martinsville
	Peggy Hodges
	National
	Vender
	Martinsville
	VA
	276-634-2981

	FF&E -Casegoods
	JTB Furniture
	Reau Berry
	National
	Vender
	Columbus
	MS
	800-654-3876

	FF&E -Casegoods
	Kimball
	Bob Brown
	National
	Vender
	 
	 
	281-893-8804

	FF&E -Casegoods
	Braman Furniture International
	Mark Gist
	National
	Vender
	 
	 
	214-340-4018

	FF&E - Purchasing
	Atlantic Hospitality Supply
	J. Philip O'Brien / Ralph Webb
	National
	Vender
	Garner
	NC
	919-771-2444

	FF&E - Purchasing
	Snowhite Textile, Inc. http://www.esnowhite.com
	Dick Kapadia/ Ken Qualtier
	National
	Vender
	Addison
	IL
	800-858-6565

	FF&E - Purchasing
	Supreme Hospitality
	Ray Gandhi
	National
	Vender
	Atlanta
	GA
	404-305-8555

	FF&E - Purchasing
	HSI Design Group, Inc
	Nick Carter /Lisa Petrosky
	National
	Vender
	San Antonio
	TX
	800-288-2391

	FF&E - Carpet Guest Rooms
	Templeton Carpet Mills
	Susan Clark Hepler
	National
	Vender
	Plano
	TX
	972-208-5955

	FF&E - Carpet Guest Rooms
	Shaw Hospitality
	Trish Garrett
	National
	Vender
	Allen
	TX
	214-724-7429

	FF&E - Carpet Corridors
	Shaw Hospitality
	Trish Garrett
	National
	Vender
	Allen
	TX
	214-724-7429

	Fire Alarms
	Grinnell
	National Account
	National
	Contractor
	Houston
	TX
	800-299-4377

	Fire Protection
	Grinnell
	National Account
	National
	Contractor
	Houston
	TX
	800-299-4377

	Tub Liners
	Ambath Corp.
	David Sanders
	National
	Contractor
	Phoenix
	AZ
	602-844-2596, 888-826-2284

	Signage Exterior
	Dualite Sales and Service, Inc.
	Vern Wooten
	National
	Contractor
	Williamsburg
	OH
	513-724-7100

	 
	Fairmont Sign Company
	James Kirkendall
	National
	Contractor
	Lodi
	CA
	209-365-6490 Ext. 103

	
	Persona, Inc.
	Mike Peterson
	National
	Contractor
	 
	 
	605-882-2244

	 Signage Interior
	InterSign Corporation
	Jon Wooldridge, Candace Cox
	National
	Contractor
	Chattanooga
	TN
	800-322-8426

	 
	American Image
	John Parageon
	National
	Contractor
	 
	 
	201-960-8898

	 
	Environmental Graphics
	Norma Klimek
	National
	Contractor
	 
	 
	770-227-0701

	Professional Consultants

	Architect
	3DI Inc.
	Jana Fowler
	National
	Architect
	 
	TX
	210-227-2500

	 
	Carrel Poole & Yost
	Edward Yost
	National
	Architect
	 
	TX
	214-953-0042

	 
	Automated Consulting Services, Inc.
	George Aubin
	National
	Architect
	Lakeland
	FL
	863-665-8718

	 
	Pate Design Group
	Kirby Pate
	Regional
	Architect
	Duluth
	GA
	770-814-0885

	 
	Lindsay Pope Brayfield & Associates
	Buck Lindsay
	National
	Architect
	Lawrenceville
	GA
	(770) 963-8989

	 
	Phillips Partnership
	Tony Patel
	National
	Architect
	Atlanta
	GA
	770-355-4300

	 
	Stan Gralla Architects
	Cara Hall
	National
	Architect
	Tulsa
	OK
	(918) 587-6158

	 
	Todd & Associates
	Doug Sexton
	National
	Architect
	Phoenix
	AZ
	(602) 952-8280

	 
	Truman Howell Architects
	Truman Howell
	National
	Architect
	 
	MN
	612-401-7889

	 
	DLW
	David L. Wallace
	National
	Architect
	Dunedin
	FL
	727-736-6000

	 
	Tischler Hous
	Richard Tischler
	Regional
	Architect
	Kinnelon
	NJ
	973-283-1874

	Fire Protection
	D. R. Associates
	Danny Shaw
	Regional
	Architect
	Houston
	TX
	(713)266-3304

	Indoor Air Quality
	Schirmer Engineering
	Warren Bonisch
	National
	Fire Protect Engineer
	Richardson
	TX
	(972) 234-1617

	 
	Assured Indoor Air Quality
	Bill Holder
	National
	Specialist
	Blanco
	TX
	830-833-1003

	
	
	
	
	
	
	
	







[image: image49.wmf] 

 

 

 

 

 

 

[image: image50.jpg]Ground Signs
@ 0490 |18 1198 xar 34| 3497
@ 5380 | 1rsamxio e | 20448
@ 8300 | 16 asurxigaam | 25167
@ gggg 150" X 16'8 1/4* 211.23
@ 5200 | wasexraar | 16870
@ oS0 | mrexiziiue | 12678
@ 9190 | 957 x106am | ss20
& | S 8218 X9 1° 6278
@ [?F-/ECDP 6'9"X7'57/8" 42.85
@ |22 | romerxeose | a7s7
B | S% |scaverxeoss | 2798

Building Signs [Tse_—s&  Jwmn
(-1 ) e ——
Recessed Building Signs re- @ Sk | 13436 X141 16870
quire special consideration @ Q-150
s M7 X12111/8" | 12678
during installation. Tem- SFWM
plate sleeves are available @ Q100 | 5 7/8" X 10 6 3/4"
b 9'57/8" X 10'6 3/ 85.20
for sign sizes Q-33 through SFWM
Q-100. Full size patterns are @ Q-75 8218"X9 1" 6278
available for $150.00 SEWM )
& | o | sox7ser 4285
& | 23 56360 x6058 | 2798
@& (S8, zosexar 8.57

f i Directionals assist your guests in finding not only the entrance to your property but also guides
Dl reCtI ONna | S them to specific areas. All of these signs can be customized to fit your particular needs.

3:611/16" ‘

341174 4‘

(La Qu i}]li{ Inn

ossv ENTER [l EXIT
o )





[image: image51.jpg]SIGN DETERMINATION
To maximize your signage benefits it should:

1.) Be visible before any exits or turn decisions. For the typical
interstate location, the signage needs to be visible at least

1/4 mile before the decision point to allow motorists ample time
to slow and make any needed lane changes.

2.) Be adequately sized ... height and square footage need to
work together. There is a formula you can use called the rule of
35. Here's how it works:

A. Determine the needed height of the pole sign. This is rel-
atively easy if this is a "conversion" property and there is ex-
isting signage to use for a guideline. If there is any doubt,
have a flag test performed. Be sure to drive every approach.
This not only determines the needed height but also deter-
mines the proper location. Locating the pole sign in the front
of the property may not always provide the best visibility.

B. After the height is determined, multiply the height by 35%
for the minimum square footage that should be installed.
Example:

80" above grade x 35% equals 28.00 or 280 square feet
50" above grade x 35% equals 17.50 or 175 square feet

Remember, this is only intended to act as a general guideline
and is not a guarantee of visibility. You also have to take into ac-
count the distance of the location from the traffic flow, obstruc-
tions and topography.

3.) Assist guests in finding the entrances. Clearly mark entran-
ces and provide interior directionals as needed.

4.) Increase consumer awareness of your property by using wall
signage. Does the guest in the parking lot of your competitor
know you are there?

CODES AND ORDINANCES

Now that you have some idea what signage is needed, where
they are to be located and how many you need, it's time to de-
termine what the local code will allow. Should the local codes
not allow the sign package you deem necessary, you need to
establish a list of the minimum signage you feel is needed and
determine how this compares to the code allowance. Is it close?
Depending on how close, you may decide to take what is offered
or request a variance. Variances are not that difficult and are
usually successful when a reasonable request is made. In addi-
tion, do their setback regulations allow you to place the ground
sign in the location you've spotted?

PERMITS

Once permits are in hand, you can then order the signage keep-
ing in mind there is usually a 3 to 4 week lead time before ship-
ping. To save time, have the sign vendor prepared to begin pro-
duction once permits are in hand.

INSTALLATION

Use a reputable contractor to handle your installation. You should re-
ceive from them a certificate of insurance and a copy of their workers
compensation insurance. In addition, you should obtain at least three
references from customers for whom they have completed similar
projects. In addition, some of them should be for work completed a
couple of years earlier. This is the only way to determine if their work
stands the test of time. Finally, they should obtain and provide a copy
of a sealed engineering print for all pole sign installations over 30"
above grade. The cost is usually in the $300.00 dollar range which is.
inexpensive insurance when you consider the ramifications of a failure
at a later date. In addition, you will always have this available should
you need to replace the sign in the future.

TIMELINES

* Sign Package Determination: 1 to 2 weeks

« Installation Quotes: 2to 3 weeks
* Permits (without a variance): 2 to 3 weeks
* Sign Delivery: 4 to 5 weeks

Ideally, you should allow 4 months from beginning to completion for
your sign package. Some of these items are flexible ... you may know
your sign package in 2 days ... you may only use one installer ... per-
mits may become available in 2 days. Conversely, a variance may be
needed or a Design Review Board hearing may be required ... add
30 to 90 days ... you may need special engineering or pole structures
...add 1 to 2 weeks. No one can tell you exactly how long it will take
to complete the project from beginning to end.

There are too many variables to establish a completion date without the
above information, however, there are two items that are absolutes:

1.) High rise pole structures should have a foundation that has been
cured for 28 days before installing the sign.

2.) It takes time to assemble and deliver the sign package. As you will
find when checking the local sign codes, allowable sign sizes will vary
from location to location. For that reason, signs are not built in
advance and held for shipping. In addition, the warranty on electrical
components begins on the date the signs are finally assembled
Dualite stocks components and sign faces in order to maximize our
ability to provide quick delivery.

Dualite has been and continues to be the sole supplier to La Quinta
Corporate since 1993 for several reasons ...

Service ¢ Premium Quality ¢ Value

Unlike other hospitality chains that only franchise, LaQuinta owns and
operates almost 300 properties. They not only look at the initial cost
of a product, they evaluate the long term costs including maintenance
and service. They realize saving a few percent today is not cost effec-
tive in the long run.

We have the knowledge ... we have the experience ... we have the
product. Trust the company LaQuinta has trusted over 300 times.

If you have any questions or require further assistance, call Vernon
Wooten @ 513-536-3193 or e-mail: vwooten @dualite.com




[image: image52.jpg]IDENTIFICATION REQUIREMENTS

Consistent identification coast to coast is the key to the suc-
cess of your property. Extensive marketing efforts are under-
taken in your behalf and to fully realize their benefits, the cor-

rect display of our logo is imperative. To assist you in this
process, we have compiled this handbook.

LOGO REQUIREMENTS
1. All properties are required to display the trademarked logo.

2. No copy alterations are permitted.

3. To insure the accurate reproduction of the logo, only ap-
proved vendors may be used.

4. Trademarked logos cannot be used without written au-
thorization from La Quinta corporate office.
SIGN CABINETS
The correct display of the logo requires a properly sized cabi-
net. A non-standard cabinet can only be used with the prior ap-
proval of the Corporate Sign Department.
All accessory signs on the same structure must have the same
cabinet color as the accompanying logo sign.

SUPPORT POLE/CLADDING

All support poles and pole cladding must be painted Walnut
Brown

READERBOARDS

Readerboards may be used in conjunction with the logo pro-
vided the cabinets are painted Gold to match the accompany-
ing logo sign. We recommend at least three (3) lines of 8" copy
at minimum.

GRAPHICS

The accurate reproduction of the logo is imperative to maintain-
ing a consistent image for all properties. For this reason, ONLY
APPROVED VENDORS MAY BE USED. Approval must be re-
ceived before beginning production.

FACE MATERIALS

Larger faces are to be made from Signtech Flexface with first
surface decoration by way of screen printing or heat transfer
graphics. Smaller faces are to be either GE Solar Grade lexan
or Sheffield Hyzod Sunlife polycarbonate pan formed and em-
bossed (if suitable) and decorated via second surface screen
printing.

ILLUMINATION

ONLY high-ouput cool white fluorescent lamps are approved for
illumination source. No HID's are permitted.

CUSTOM FACES/SIGNS

Consistent signage is an important element of successful mar-
keting. In those instances where restrictive codes will not allow
the installation of a new sign without loss of either size or height,
face replacements to cabinets may be allowed with prior appro-
val of the Corporate Sign Department.

The use of your name along with the logo must be approved by
the Corporate Sign Department.

NON-APPROVED VENDORS

Our approved vendors are familiar with our programs and use
stringent quality controls to insure the accurate reproduction of
our logo. Non-approved vendors may only be used upon receipt
of warranty information, proof of liability insurance, proof of
workers comp insurance, detailed engineering plans and
graphics layout. Logo specifications will be sent for their use on-
ly with the written approval of La Quinta corporate offices.

['"-‘"'IBI Dualite Sales & Service, In

WILLIAMSBURG, OHIO + CEDAR HILL, TEXAS

Contact Vern Wooten

Tel: 513 536 3193

E-Mail: vwooten@dualite.com
FAX 513-536-3116

One Dualite Lane B

Williamsburg,

Ohio g 45176




[image: image53.jpg]LaQuinta

IDENTIFICATION
PROGRAM

Proper and effective identification is a combination of many elements.
Signage, properly sized, located and maintained is a vital marketing
tool and is the least expensive advertising available per exposure.

Contact: Vern Wooten

ouaiiee’ Dyglite Sales & Service, Inc. Tel: 513-536-3193

E-Mail: vwooten@dualite.com
FAX 513-536-3116

WILLIAMSBURG, OHIO + CEDAR HILL, TEXAS




[image: image54.jpg]Ground Signs
@ oF ey 15'9" X 25'4" 338.87
5350 R
m 535 14'93/8" X 23'8 207.42
300 T
m S$:399, |38 x2rt012| 26523
S-250 SR HGGE
m 525, 12'6"X 200 21262
-2 S
m S5 12X 1711 17012
S-150 e
e} 518 98 X156 127.47
100 . aia
= S 7101/2' X 12'8 84.86
.75 S0l gig
[l S, 610 1/4' X 10' 11 6397
i) S50 57'X8 1138 4282
DF/ CP.
S-42 10" o
B |t Brce| sioxre 36.46
(-] e 4TUXT 2 2816
Building Signs e
@ $-200 12 X 17 11 170.12
SFWM
Recessed Building Signs re-
quire special consideration oiine] = 98 X15'6" 127.47
during installation. Tem-
plate sleeves are available = ssé/'vm 7101/2'X 128" 84.86
for sign sizes 5-33 through
S-100. Full size patterns are [l $75 610 1/4" X 10 117 6397
SFWM
available for $150.00
S50 R .
= S 57'X8 1138 4282
533 —
-] e 4714 X7 2 28.16
s-8
= SFWM 2284 X3 7" 6.86

DI r tl n | Directionals assist your guests in finding not only the entrance to your property but also guides
eC O a S them to specific areas. All of these signs can be customized to fit your particular needs.

-6 11/16"

31 174t

21 18—

enter [ EXIT





[image: image55.jpg]SIGN DETERMINATION
To maximize your signage benefits it should:

1.) Be visible before any exits or turn decisions. For the typical
interstate location, the signage needs to be visible at least

1/4 mile before the decision point to allow motorists ample time
to slow and make any needed lane changes.

2.) Be adequately sized ... height and square footage need to
work together. There is a formula you can use called the rule of
35. Here's how it works:

A. Determine the needed height of the pole sign. This is rel-
atively easy if this is a "conversion" property and there is ex-
isting signage to use for a guideline. If there is any doubt,
have a flag test performed. Be sure to drive every approach.
This not only determines the needed height but also deter-
mines the proper location. Locating the pole sign in the front
of the property may not always provide the best visibility.

B. After the height is determined, multiply the height by 35%
for the minimum square footage that should be installed.
Example:

80' above grade x 35% equals 28.00 or 280 square feet
50' above grade x 35% equals 17.50 or 175 square feet

Remember, this is only intended to act as a general guideline
and is not a guarantee of visibility. You also have to take into ac-
count the distance of the location from the traffic flow, obstruc-
tions and topography.

3.) Assist guests in finding the entrances. Clearly mark entran-
ces and provide interior directionals as needed.

4.) Increase consumer awareness of your property by using wall
signage. Does the guest in the parking lot of your competitor
know you are there?

CODES AND ORDINANCES

Now that you have some idea what signage is needed, where
they are to be located and how many you need, it's time to de-
termine what the local code will allow. Should the local codes
not allow the sign package you deem necessary, you need to
establish a list of the minimum signage you feel is needed and
determine how this compares to the code allowance. Is it close?
Depending on how close, you may decide to take what is offered
or request a variance. Variances are not that difficult and are
usually successful when a reasonable request is made. In addi-
tion, do their setback regulations allow you to place the ground
sign in the location you've spotted?

PERMITS

Once permits are in hand, you can then order the signage keep-
ing in mind there is usually a 3 to 4 week lead time before ship-
ping. To save time, have the sign vendor prepared to begin pro-
duction once permits are in hand.

INSTALLATION

Use a reputable contractor to handle your installation. You should re-
ceive from them a certificate of insurance and a copy of their workers
compensation insurance. In addition, you should obtain at least three
references from customers for whom they have completed similar
projects. In addition, some of them should be for work completed a
couple of years earlier. This is the only way to determine if their work
stands the test of time. Finally, they should obtain and provide a copy
of a sealed engineering print for all pole sign installations over 30"
above grade. The cost is usually in the $300.00 dollar range which is
inexpensive insurance when you consider the ramifications of a failure
at a later date. In addition, you will always have this available should
you need to replace the sign in the future.

TIMELINES

* Sign Package Determination: 1 to 2 weeks
* Installation Quotes: 2to 3 weeks
* Permits (without a variance): 2 to 3 weeks
* Sign Delivery: 4 to 5 weeks

Ideally, you should allow 4 months from beginning to completion for
your sign package. Some of these items are flexible ... you may know
your sign package in 2 days ... you may only use one installer ... per-
mits may become available in 2 days. Conversely, a variance may be
needed or a Design Review Board hearing may be required ... add
30 to 90 days ... you may need special engineering or pole structures
... add 1to 2 weeks. No one can tell you exactly how long it will take
to complete the project from beginning to end.

There are too many variables to establish a completion date without the
above information, however, there are two items that are absolutes:

1.) High rise pole structures should have a foundation that has been
cured for 28 days before installing the sign.

2.) It takes time to assemble and deliver the sign package. As you will
find when checking the local sign codes, allowable sign sizes will vary
from location to location. For that reason, signs are not built in
advance and held for shipping. In addition, the warranty on electrical
components begins on the date the signs are finally assembled
Dualite stocks components and sign faces in order to maximize our
ability to provide quick delivery.

Dualite has been and continues to be the sole supplier to La Quinta
Corporate since 1993 for several reasons ...

Service * Premium Quality * Value

Unlike other hospitality chains that only franchise, LaQuinta owns and
operates almost 300 properties. They not only look at the initial cost
of a product, they evaluate the long term costs including maintenance
and service. They realize saving a few percent today is not cost effec-
tive in the long run.

We have the knowledge ... we have the experience ... we have the
product. Trust the company LaQuinta has trusted over 300 times.

If you have any questions or require further assistance, call Vernon
Wooten @ 513-536-3193 or e-mail: vwooten @dualite.com




[image: image56.jpg]IDENTIFICATION REQUIREMENTS
Consistent identification coast to coast is the key to the suc-
cess of your property. Extensive marketing efforts are under-
taken in your behalf and to fully realize their benefits, the cor-
rect display of our logo is imperative. To assist you in this
process, we have compiled this handbook.
LOGO REQUIREMENTS

1. All properties are required to display the trademarked logo.

2. No copy alterations are permitted.

3. To insure the accurate reproduction of the logo, only ap-
proved vendors may be used.

4. Trademarked logos cannot be used without written au-
thorization from La Quinta corporate office.
SIGN CABINETS
The correct display of the logo requires a properly sized cabi-
net. A non-standard cabinet can only be used with the prior ap-
proval of the Corporate Sign Department.
All accessory signs on the same structure must have the same
cabinet color as the accompanying logo sign.

SUPPORT POLE/CLADDING

All support poles and pole cladding must be painted Frosty
White.

READERBOARDS

Readerboards may be used in conjunction with the logo pro-
vided the cabinets are painted Gold to match the accompany-
ing logo sign. We recommend at least three (3) lines of 8" copy
at minimum.

GRAPHICS

The accurate reproduction of the logo is imperative to maintain-
ing a consistent image for all properties. For this reason, ONLY
APPROVED VENDORS MAY BE USED. Approval must be re-
ceived before beginning production.

FACE MATERIALS

Larger faces are to be made from Signtech Flexface with first
surface decoration by way of screen printing or heat transfer
graphics. Smaller faces are to be either GE Solar Grade lexan
or Sheffield Hyzod Sunlife polycarbonate pan formed and em-
bossed (if suitable) and decorated via second surface screen
printing.

ILLUMINATION

ONLY high-ouput daylight fluorescent lamps are approved for il-
lumination source. No HID's are permitted.

CUSTOM FACES/SIGNS

Consistent signage is an important element of successful mar-
keting. In those instances where restrictive codes will not allow
the installation of a new sign without loss of either size or height,
face replacements to cabinets may be allowed with prior appro-
val of the Corporate Sign Department.

The use of your name along with the logo must be approved by
the Corporate Sign Department.

NON-APPROVED VENDORS

Our approved vendors are familiar with our programs and use
stringent quality controls to insure the accurate reproduction of
our logo. Non-approved vendors may only be used upon receipt
of warranty information, proof of liability insurance, proof of
workers comp insurance, detailed engineering plans and
graphics layout. Logo specifications will be sent for their use on-
ly with the written approval of La Quinta corporate offices.

One

Dualite

WILLIAN URG, OHIO « CEDAR HILL, TEXAS

Dualite Lane -

Dualite Sales & Service, Inc.

Williamsburg,

Contact Vern Wooten
Tel: 513536 3193

E-Mail: vwooten@dualite.com
FAX 513-536-3116

Ohio J 45176




[image: image57.png]ivuame; Dualite Sales & Service, Inc.

i F
WS e e Ona Dualite Lane « Wilamsburg, OH 45176 » (§10) 724-7100 + FAX (613) 724-029

Laﬁuinta “sleeved” sign
installation details.

Single faced signs that are recessed into the building
openings require special attention in two areas:

1. The leading edge of the cabinet that faces “out” of the
sleeve MUST extend three (3) inches from the building
face. This is to allow room for the drain holes and
prevent water from backing up into the sign and the
tower. '

2. The opening around the sign cabinet MUST be
completely and thoroughly caulked with GE clear
silicone. This has to be done both from the front and
the rear of the cabinet. | |

A cross sectioh drawing is enclosed for your review.

You will be held responsible for any damages that are
caused by water leaking in around the cabinets.

Please, be very careful and thorough.



[image: image58.png]70 PREVENT DRAINAGE

GE CLEAR SILICONE CAULKING
SIDES AND BOTTOM AT REAR \
INTD BUILDING.

I

—

NOTE C v
DRAIN HOLES FOR CONDENSATION

| i . ‘

HINGED BACK -
FOR SERVICE ﬁ
SIGN
e ) ) 114' x 1* x 6' ALUM. SHIM
— (3 REQD. (VELD TO SLEEVE) i ,h. /
GE CLEAR SILICUNE BY - Y B
INSTALLER AS SHOVN ABOVE} L 4 /-meu'rzn 060 ALUM. SLEEVE ﬂ : :
_L | I ——— — . * : 2=y
5/16° CLEARANCE J— _ . |
ALL ARCOUND : ‘

CLEAR EILICDNE BY
INSTALLER AS SHOWN ABOV

' 74% 1 3/4° 3 i
2 x 4 FRAMING \ ' ‘
(BY OTHERS) / \ NAILING FLANGE

\ Vs TACK WELDED
- ‘ ALL ARDUND .
E.JF.S. CBY. nmsgs:—\K' ba R
PLYVDDD (BY OTHERS | \, K4 - 4c MAILING FLANGE
' /1 <o AL




[image: image59.jpg]CUSTOM
INTERIOR
SIGNAGE

La Quinta Approved Interior Graphics Venders Listing

Company Contact Person Phone Number
1. American Image John Parageon (201) 960-8898
2.HSC Norma Klimek (770) 227-0701

3.Intersign Corporation John Wooldridge (800) 322-8426





[image: image60.jpg]SIGNAGE MENU

4-3/4"w x 4-3/8"h
Interior ADA Guestroom Number
with Braille

® =

3"wx3"h
Interior Guestroom Number
HC Symbol

© E==m

4.314"w x 1-1/2"h
No Smoking Guestroom Symbol

A

IN CASE OF
é  FRe <&
USE STAIRS

8"wx8"h
In Case of Fire

A

VENDING

6"w x5-112"h
Vending

8"wx8"h
Women's Restroom w/HC Symbol

8"wx8"h
Men'’s Restroom w/HC Symbol

6]

Push Button For
Late Check-In

8"wx 6"h
Late Check-In

Doors
Locked
24 Hours.

Please use your
room key to
re-enter this door.

6-3/4"w x 6-3/4"h
Doors Locked 24 Hours (Exterior)

®
L4

MEETING
ROOMA

8"wx 9-1/2"h
Meeting Room w/lnsert

* Indicate Name on Page 5

MEETING
ROOM B

8"wx9-1/2"h
Meeting Room w/(lnsert
* Indicate Name on Page §

(9]

A

BUSINESS
CENTER

(10

A

6"wx 5-112"h
Business Center

6"w x 5-1/2"h
Staff Only

10) Interior 10E) Exterior

®
A

FITNESS
CENTER

®

GUEST
LAUNDRY

6"w x 5-1/2"h
Fitness Center

6"w x 5-1/2"h
Guest Laundry

®

MEETING ROOM

64w x 8"h
Meeting Room Dimensional Letters

6"w x 5-1/2"h
Sales





[image: image61.jpg]SIGNAGE MENU

A

STAIRS

6"w x 5-1/2"h
Stairs

A

LUGGAGE

6"w x 5-1/2"h
Luggage

6"w x 5-1/2"h
Carts

ROOF ACCESS

1THRU 3

12"wx 12"h
Floor Stair/Level ID
18R) Roof Access  18N) No Roof Access

®

12'w x 4"h

1 Line Directional

Typical Room Directional Sign
#Enter Text on Page 2

D

= ROOM 000-000
€ ROOM 000-000

12"w x 6"h

2 Line Directional
*Enter Text on Page 2

21

A\ GUEST LAUNDRY
A VENDING
A PUBLIC TOILETS

12"wx 8"h
3 Line Directional
“Enter Text on Page 2

22

€= GUEST LAUNDRY
A VENDING

W PUBLIC TOILETS
=> ELEVATORS

12"wx 10"h

4 Line Directional
“Enter Text on Page 3

23

€ GUEST LAUNDRY
A VENDING
W PUBLIC TOILETS

=> ELEVATORS
€ ROOM 000-000

12"wx 12°"h

5 Line Directional
*Enter Text on Page 3

D

€ GUEST LAUNDRY
A VENDING

¥ PUBLIC TOILETS
=> ELEVATORS
€ ROOM 000-000
= ROOM 000

12"w x 12%h

6 Line Directional
*Enter Text on Page 4

Vending
Business Center
Guest Laundry
Breakfast Area
Public Phone
Guest Rooms

Second Floor
Guest Rooms
Vending

Third Floor
Guest Rooms
Vending

12"w x 16"h
Directory - Prototype C

*Enter Text on Page 5

D

ELECTRICA

ROOM

8"wx6"h
Electrical Room

27

8"wx6"h
Pantry

27) Interior  27E) Exterior

D

ELEVATOR

EQUIPMENT

8"wx6"h
Elevator Equipment

D

MAINTENANCE

8"wx6"h
Maintenance
29) Interior  29E) Exterior

D

STORAGE

8"wx6h
Storage
30) Interior 30E) Exterior





[image: image62.jpg]SIGNAGE MENU

MECHANICAL

ROOM

8"wx6"h
Mechanical Room
31) Interior 31E) Exterior

TELEPHONE/

CABLE ROOM

8"wx6"h
Telephone/Cable Room

®

No Smoking

8"wx 6"
No Smoking

8"wx8"h
Board Room wiinsert
* Indicate Name on Page 5

35

BOARD ROOM

55"wx 6"h
Board Room Dimensional Letters

* Indicate Name on Page 5

3¢

8"wx8"h
Maximum Occupancy

@
f

Manager
on Duty

| manaceR's NAME

12'wx 12°h
Owner/Manager on Duty
*Enter Names on Page 6

D

B"'wx2"h
Telephone Use

@

CLEARANCE XX’'X”

48"w x 5-3/4"h
Clearance

36"w x 30-1/8"h with 1/2" Thick Letters
Front Desk Logo Sign

4"wx 1-112"h
Decaffeinated Coffee Plate

4"'wx 1-1/2"h
Regular Coffee Plate

4"w x 1-1/2"h
Hot Water Plate

24"wx 16"h
Guest Laundry Liability

Fitness Center Safety Rules

28"w x 12'h
Fitness Center Rules

12"w x 18"h

Spa Rules w/Hours

* Cansul Your Local Health Deparimment for Specific Rales





[image: image63.jpg]SIGNAGE MENU

b 5
VA Respoositie

No Diving
NO LIFEGUARD
ON DUTY

49)

Pool Towels

_ Available at
~Front.Desk

Shower Beiq

12"wx 18"h ) .
Pool Rules w/Hours 12"wx 18"h 15"wx 10"h 6-3/4"w x 8-3/4"h
 Consat Your L ocal Healh Depriment orspesiic kuies | No Diving Poal Towels Area of Rescue Assistance

EVACUATION

21-1/2"w x 6"h
Evacuation Assistance

52

12'wx 18"h
Courtesy Parking

53

La Quinta
Courtesy
Van
Parking
Only

12"wx 18"h
Courtesy Van Parking

Accessible
Parking

12'wx 18"h
HC Accessible Parking

55)

Van
Accessible
Parking

12"wx 18"h
HC Van Accessible Parking

9"wx 12"h
Emergency Exit & State Laws

D

Emergency
CALL 911
You are at:
XXX XXX XXX
XXX XXX XXX
XXXXX

18"w x 14"h
911 Emergency w/Address
(Texas Only)

“Enter Information on Page 7

59

20"w x 8"h
Child Supervision
(Texas Only)

D

DO NOT USE SPA IF
'WATER TEMP IS ABOVE
104 FAHRENHEIT
(40° CENTIGRADE)

12"w x 5-1/2"h
Spa Temperature
(Texas Only)

60

WARNING:

This Area is a Designated
Smoking Area. Tobacéo

Smoking is Known to the State
of California to Cause Cancer
and Birth Defects or Other
Reproductive Harm.

10"wx 8"h
Tobacco Smoking - Area Specific
(California Only)

61

WARNING:

This Area Permits Smoking
in Designated Areas. Tobacco
Smoke is Known o the State
of California to Cause Cancer
and Birth Defects or Other
Reproductive Harm.

10"wx 8"h
Tobacco Smoking - Building Specific
(California Only)

62)

WARNING:
Smoking is Permitted in Some
of the Guest Rooms on This
Floor. Tobacco Smoke is Known
1o the State of California to
Cause Cancer and Birth Defects
or Other Reproductive Harm.

10"w x 8"h
Tobacco Smaking - Floor Specific
(California Only)





[image: image64.jpg]SIGNAGE MENU

8"wx8"h
Restrooms

SPRINKLER
ROOM

8"wx6"h
Sprinkler Room
64) Interior  64E) Exterior

®

POOL
EQUIPMENT

8"wx 6"h
Pool Equipment
63) Interior  63K) Exterior

8"wx6"h
Fire Pump

6) Interior  66K) Exterior

@

12"wx 8"h
Complimentary Breakfast

68)

CUSTOM
SIGN

8"wx6"h

Custom Sign
*Enter Text on Page 7

®

A

CUSTOM
SIGN

6"w x5-1/2"h
Custom Sign
“Enter Text on Page 7

3'wx1"h
Name Badge

“Enter Names on Page 6

@

NOW
OPEN

120"w x 48"h
“Now Open” Banner





[image: image65.jpg]CUSTOM
INTERIOR
SIGNAGE

La Quinta Approved Interior Graphics Vendors Listing

Company Contact Person Phone Number
1. American Image John Parageon (201) 960-8898
2.Environmental Graphics Norma Klimek (770) 227-0701

3. Intersign Corporation John Wooldridge (800) 322-8426





[image: image66.jpg]SIGNAGE MENU

6-1/4"w x 4"h
Interior ADA Guestroom Number
with Braille

14

3"wx3"h
Interior Guestroom Number
HC Symbol

(2]

IN CASE OF

& FIRE~$
USE STAIRS

© ==

5-3/4"w x 1-112"h
No Smoking Guestroom Symbol

7-1/4"w x 8-1/4"h
In Case of Fire

6-1/4"w x 4-5/8"h
Vending

7-1/4"w x 8-1/4"h
Women's Restroom w/HC Symbol

7-1/4"w x 8-1/4"h
Men’s Restroom w/HC Symbol

(6]

Push Button For
Late Check-In

8"wx 6"h
Late Check-In

Doors
Locked
24 Hours.

Please use your
room key to
re-enter this door.

6-3/4"w x 6-3/4"h
Doors Locked 24 Hours (Exterior)

MEETING
ROOM A

7-1/4"w x 8-1/4"h

Meeting Room w/lnsert
* Indicate Name on Page 4

o

BUSINESS

CENTER

6-1/4"w x 4-5/8"h
Business Center

(10

6-1/4"w x 4-5/8"h
Staff Only

1)

FITNESS
CENTER

7-1/4"w x 8-1/4"h
Fitness Center

@

GUEST

LAUNDRY

6-1/4"w x 4-5/8"h
Guest Laundry

®

MEETING ROOM

64-7/8"w x 6"h
Meeting Room Dimensional Letters

14

6-1/4"w x 4-5/8"h
Sales

®

STAIRS

6-1/4"w x 4-5/8"h
Stairs





[image: image67.jpg]SIGNAGE MENU

ROOF ACCESS

12"w x 12"h
Floor Stair/Level ID

16R) Roof Access  16N) No Roof Access

6-1/4"w x 4-5/8"h
Luggage

®

6-1/4"w x 4-5/8"h
Carts

12"w x 4"h
1 Line Directional

D

=» Room 000-000

€ Room 000-000

12"w x 8"h
2 Line Directional

21

€ Guest Laundry
€ Vending
<€ Public Toilets

12'wx 8"h
3 Line Directional

@

Elevator
Guest Laundry
Fitness Center

Vending

12"w x 10"h
4 Line Directional - Prototype B

23)

Meeting Rooms
Public Toilet
Business Center
Board Room
Room 000-000

12"wx 12"h
5 Line Directional - Prototype B

24

Meeting Room A
Meeting Room B
Board Room
Public Toilet
Public Phones

Business Center

12"wx 12"h
6 Line Directional

D

Public Toilet
Guest Laundry
Meeting Room
Elevator

itness Center
Business Center
Vending
Room 000-000
Breakfast Room
Public Telephones

IO O AR R

12"wx 16"h
10 Line Directional - Prototype D

DIRECTORY
First Floor
Meq

Breakfast Area
ic Phone.

Vending

Third Floor
Guest Rooms
Vending

12"w x 16"h
Directory - Prototype D

D

ELECTRICAL
ROOM

8"wx 6"h
Electrical Room

2¢]

8"wx 6"h
Pantry

29

ELEVATOR
EQUIPMENT

8"wx 6"h
Elevator Equipment

D

MAINTENANCE

8"wx 6"h
Maintenance

31

STORAGE

8"wx6"h
Storage





[image: image68.jpg]SIGNAGE MENU

MECHANICAL
ROOM

8"wx6"h
Mechanical Room

TELEPHONE/

CABLE ROOM

8"wx6"h
Telephone/Cable Room

D

No Smoking

8"wx6"h
No Smoking

7-1/4"w x 8-1/4"h
Board Room w/lnsert
* Indicate Name on Page 4

D

BOARD ROOM

57-1/2"w x 6"h
Board Room Dimensional Letters

57

MAXIMUM

OCCUPANCY
00

7-1/4"w x 8-1/4"h
Maximum Occupancy

D

Manager
on Duty

__MANAGER'S NAME _

1M"wx 12"h
Owner/Manager on Duty

@

2"hx 6"w
Telephone Use

@

ARANCE XX’X

48"w x 5-3/4"h
Clearance

)

48"w x 29"h with 1/2” Thick Letters
Front Desk Logo Sign

42)

4"wx1-1/2"h
Decaffeinated Coffee Plate

4"wx 1-1/2"h
Regular Coffee Plate

4w x 1-1/2°h
Hot Water Plate

Guest Laundry

24"w x 16"h
Guest Laundry Liability

Fitness Center Safety Rules

24"wx 12"h
Fitness Center Rules

12"wx 18"h
Spa Rules w/Hours
Indicate Hours:

= Consult Your Local Health Department for Specific Rules





[image: image69.jpg]SIGNAGE MENU

12'wx 18"h
Pool Rules w/Hours
Indicate Hours:

* Consult Your Local Health Department for Specific Rules

No Diving
NO LIFEGUARD
ON DUTY

12'wx 18"h
No Diving

50

Pool Towels

Available at
Front Desk

15"w x 10"h
Pool Towels

&

AREA OF

RESCUE
ASSISTANCE

6-3/4"w x 6-3/4"h
Area of Rescue Assistance

52)

[l Evacuamion
@) ‘ ASSISTANCEAREA

21-1/2"w x 8"h
Evacuation Assistance

53)

Courtesy Parking
for La Quinta
[

La Quinta is not
responsible for loss
of vehicle or
property.
Please remove your
valuables and lock
your vehicle.

12"w x 18"h
Courtesy Parking

54

12"w x 18"h
Courtesy Van Parking

®
&

Accessible
Parking

12"w x 18"h
HC Accessible Parking

(56
&
Van

Accessible
Parking

12w x 18"h
HC Van Accessible Parking

9"wx 12"h
Emergency Exit & State Laws

58)

Emergency
CALL 911
You are at:
XXX XXX XXX
XXX XXX XXX
XXXXX

18"w x 14"h
911 Emergency w/Address
(Texas Only)

59)

CHILDREN SHOULD NOT

USE POOL WITHOUT
ADULT SUPERVISION

20"wx 8"h
Child Supervision
(Texas Only)

60

DO NOT USE SPA IF
WATER TEMP IS ABOVE
104° FAHRENHEIT
(40° CENTIGRADE)

12"w x 5-1/2"h
Spa Temperature
(Texas Only)

WARNING:

rea is a Designated

Known to the State
0 Cause Cancer
and Birth Defects or Other
Reproductive Harm.

10"w x 8"h
Tobacco Smaking - Area Specific
(California Only)

@

WARNING:
This Area Permits Smoking
in Designated Areas. Tobacco

Smoke is Known to the State
of California to Cause Cancer
and Birth Defects or Other
Reproductive Harm.

10"w x 8"h
Tobacco Smoking - Building Specific
(California Only)

®

WARNING:
Smoking is Permitted in Some
of the Guest Rooms on This
loor. Tobacco Smoke is Know,
to the State of California to
(Cause Cancer and Birth Defects
or Other Reproductive Harm.

10"w x 8"h
Tobacco Smoking - Floor Specific
(California Only)





[image: image70.jpg]SIGNAGE MENU

3N

RESTROOMS

7-1/4"w x 8-1/4"h
Restrooms

SPRINKLER
ROOM

6"wx 8"h
Sprinkler Room

6¢)

POOL
EQUIPMENT

B"'wx8"h
Pool Equipment

6"wx 8"h
Fire Pump

68)

12"wx 8"h
Complimentary Breakfast

69

€ Public Toilets

12"w x4"h
1 Line Directional

3'wx1"h
Name Badge

“Enter Names on Page 5

120"w x 48"h
“Now Open"” Banner

72

PUBLIC

TELEPHONES

7-1/4"w x 8-1/4"h
Public Telephones





210.08.1 
210.08.2 
210.08.3 
210.08.4 
210.08.5 
210.08.6 
210.08.7 
210.08.8 
210.9 
210.09.1 
210.09.2 
210.09.3 
210.09.4 
210.09.5 
210.09.6 
210.09.7 
210.09.8 
210.09.9 
210.09.10 
� LINK Word.Document.8 "\\\\dal-apps1\\groups\\Develpment Services\\FRANCHISE\\Design and construction standards\\NOT USED_keep for record\\Design and Construction Standards.doc"  \a \b ���


� LINK Word.Document.8 "\\\\dal-apps1\\groups\\Develpment Services\\FRANCHISE\\Design and construction standards\\NOT USED_keep for record\\Design and Construction Standards.doc"  \a \b ���






                     
- 84 -       
    

Development services/Franchise/Design and Construction Standards
© La Quinta 


_1140587642.unknown

_1142751484.unknown

_1126008191.unknown

_1131799694.doc
[image: image1.jpg]la Quinta we

IDENTIFICATION
PROGRAM

Proper and effective identification is a combination of many elements.
Signage, properly sized, located and maintained is a vital marketing
tool and is the least expensive advertising available per exposure.

Contact: Vern Wooten

ouatite . Dyalite Sales & Service, INc. Tel: 513-536-3193

E-Mail: vwooten@dualite.com
FAX 513-536-3116

WILLIAMSBURG, OHIO « CEDAR HILL, TEXAS






[image: image2.jpg]Ground Signs
@ 0490 |18 1198 xar 34| 3497
@ 5380 | 1rsamxio e | 20448
@ 8300 | 16 asurxigaam | 25167
@ gggg 150" X 16'8 1/4* 211.23
@ 5200 | wasexraar | 16870
@ oS0 | mrexiziiue | 12678
@ 9190 | 957 x106am | ss20
& | S 8218 X9 1° 6278
@ [?F-/ECDP 6'9"X7'57/8" 42.85
@ |22 | romerxeose | a7s7
B | S% |scaverxeoss | 2798

Building Signs [Tse_—s&  Jwmn
(-1 ) e ——
Recessed Building Signs re- @ Sk | 13436 X141 16870
quire special consideration @ Q-150
s M7 X12111/8" | 12678
during installation. Tem- SFWM
plate sleeves are available @ Q100 | 5 7/8" X 10 6 3/4"
b 9'57/8" X 10'6 3/ 85.20
for sign sizes Q-33 through SFWM
Q-100. Full size patterns are @ Q-75 8218"X9 1" 6278
available for $150.00 SEWM )
& | o | sox7ser 4285
& | 23 56360 x6058 | 2798
@& (S8, zosexar 8.57

f i Directionals assist your guests in finding not only the entrance to your property but also guides
Dl reCtI ONna | S them to specific areas. All of these signs can be customized to fit your particular needs.

3:611/16" ‘

341174 4‘

(La Qu i}]li{ Inn

ossv ENTER [l EXIT
o )







[image: image3.jpg]SIGN DETERMINATION
To maximize your signage benefits it should:

1.) Be visible before any exits or turn decisions. For the typical
interstate location, the signage needs to be visible at least

1/4 mile before the decision point to allow motorists ample time
to slow and make any needed lane changes.

2.) Be adequately sized ... height and square footage need to
work together. There is a formula you can use called the rule of
35. Here's how it works:

A. Determine the needed height of the pole sign. This is rel-
atively easy if this is a "conversion" property and there is ex-
isting signage to use for a guideline. If there is any doubt,
have a flag test performed. Be sure to drive every approach.
This not only determines the needed height but also deter-
mines the proper location. Locating the pole sign in the front
of the property may not always provide the best visibility.

B. After the height is determined, multiply the height by 35%
for the minimum square footage that should be installed.
Example:

80" above grade x 35% equals 28.00 or 280 square feet
50" above grade x 35% equals 17.50 or 175 square feet

Remember, this is only intended to act as a general guideline
and is not a guarantee of visibility. You also have to take into ac-
count the distance of the location from the traffic flow, obstruc-
tions and topography.

3.) Assist guests in finding the entrances. Clearly mark entran-
ces and provide interior directionals as needed.

4.) Increase consumer awareness of your property by using wall
signage. Does the guest in the parking lot of your competitor
know you are there?

CODES AND ORDINANCES

Now that you have some idea what signage is needed, where
they are to be located and how many you need, it's time to de-
termine what the local code will allow. Should the local codes
not allow the sign package you deem necessary, you need to
establish a list of the minimum signage you feel is needed and
determine how this compares to the code allowance. Is it close?
Depending on how close, you may decide to take what is offered
or request a variance. Variances are not that difficult and are
usually successful when a reasonable request is made. In addi-
tion, do their setback regulations allow you to place the ground
sign in the location you've spotted?

PERMITS

Once permits are in hand, you can then order the signage keep-
ing in mind there is usually a 3 to 4 week lead time before ship-
ping. To save time, have the sign vendor prepared to begin pro-
duction once permits are in hand.

INSTALLATION

Use a reputable contractor to handle your installation. You should re-
ceive from them a certificate of insurance and a copy of their workers
compensation insurance. In addition, you should obtain at least three
references from customers for whom they have completed similar
projects. In addition, some of them should be for work completed a
couple of years earlier. This is the only way to determine if their work
stands the test of time. Finally, they should obtain and provide a copy
of a sealed engineering print for all pole sign installations over 30"
above grade. The cost is usually in the $300.00 dollar range which is.
inexpensive insurance when you consider the ramifications of a failure
at a later date. In addition, you will always have this available should
you need to replace the sign in the future.

TIMELINES

* Sign Package Determination: 1 to 2 weeks

« Installation Quotes: 2to 3 weeks
* Permits (without a variance): 2 to 3 weeks
* Sign Delivery: 4 to 5 weeks

Ideally, you should allow 4 months from beginning to completion for
your sign package. Some of these items are flexible ... you may know
your sign package in 2 days ... you may only use one installer ... per-
mits may become available in 2 days. Conversely, a variance may be
needed or a Design Review Board hearing may be required ... add
30 to 90 days ... you may need special engineering or pole structures
...add 1 to 2 weeks. No one can tell you exactly how long it will take
to complete the project from beginning to end.

There are too many variables to establish a completion date without the
above information, however, there are two items that are absolutes:

1.) High rise pole structures should have a foundation that has been
cured for 28 days before installing the sign.

2.) It takes time to assemble and deliver the sign package. As you will
find when checking the local sign codes, allowable sign sizes will vary
from location to location. For that reason, signs are not built in
advance and held for shipping. In addition, the warranty on electrical
components begins on the date the signs are finally assembled
Dualite stocks components and sign faces in order to maximize our
ability to provide quick delivery.

Dualite has been and continues to be the sole supplier to La Quinta
Corporate since 1993 for several reasons ...

Service ¢ Premium Quality ¢ Value

Unlike other hospitality chains that only franchise, LaQuinta owns and
operates almost 300 properties. They not only look at the initial cost
of a product, they evaluate the long term costs including maintenance
and service. They realize saving a few percent today is not cost effec-
tive in the long run.

We have the knowledge ... we have the experience ... we have the
product. Trust the company LaQuinta has trusted over 300 times.

If you have any questions or require further assistance, call Vernon
Wooten @ 513-536-3193 or e-mail: vwooten @dualite.com






[image: image4.jpg]IDENTIFICATION REQUIREMENTS

Consistent identification coast to coast is the key to the suc-
cess of your property. Extensive marketing efforts are under-
taken in your behalf and to fully realize their benefits, the cor-

rect display of our logo is imperative. To assist you in this
process, we have compiled this handbook.

LOGO REQUIREMENTS
1. All properties are required to display the trademarked logo.

2. No copy alterations are permitted.

3. To insure the accurate reproduction of the logo, only ap-
proved vendors may be used.

4. Trademarked logos cannot be used without written au-
thorization from La Quinta corporate office.
SIGN CABINETS
The correct display of the logo requires a properly sized cabi-
net. A non-standard cabinet can only be used with the prior ap-
proval of the Corporate Sign Department.
All accessory signs on the same structure must have the same
cabinet color as the accompanying logo sign.

SUPPORT POLE/CLADDING

All support poles and pole cladding must be painted Walnut
Brown

READERBOARDS

Readerboards may be used in conjunction with the logo pro-
vided the cabinets are painted Gold to match the accompany-
ing logo sign. We recommend at least three (3) lines of 8" copy
at minimum.

GRAPHICS

The accurate reproduction of the logo is imperative to maintain-
ing a consistent image for all properties. For this reason, ONLY
APPROVED VENDORS MAY BE USED. Approval must be re-
ceived before beginning production.

FACE MATERIALS

Larger faces are to be made from Signtech Flexface with first
surface decoration by way of screen printing or heat transfer
graphics. Smaller faces are to be either GE Solar Grade lexan
or Sheffield Hyzod Sunlife polycarbonate pan formed and em-
bossed (if suitable) and decorated via second surface screen
printing.

ILLUMINATION

ONLY high-ouput cool white fluorescent lamps are approved for
illumination source. No HID's are permitted.

CUSTOM FACES/SIGNS

Consistent signage is an important element of successful mar-
keting. In those instances where restrictive codes will not allow
the installation of a new sign without loss of either size or height,
face replacements to cabinets may be allowed with prior appro-
val of the Corporate Sign Department.

The use of your name along with the logo must be approved by
the Corporate Sign Department.

NON-APPROVED VENDORS

Our approved vendors are familiar with our programs and use
stringent quality controls to insure the accurate reproduction of
our logo. Non-approved vendors may only be used upon receipt
of warranty information, proof of liability insurance, proof of
workers comp insurance, detailed engineering plans and
graphics layout. Logo specifications will be sent for their use on-
ly with the written approval of La Quinta corporate offices.

['"-‘"'IBI Dualite Sales & Service, In

WILLIAMSBURG, OHIO + CEDAR HILL, TEXAS

Contact Vern Wooten

Tel: 513 536 3193

E-Mail: vwooten@dualite.com
FAX 513-536-3116

One Dualite Lane B

Williamsburg,

Ohio g 45176






[image: image5.jpg]LaQuinta

IDENTIFICATION
PROGRAM

Proper and effective identification is a combination of many elements.
Signage, properly sized, located and maintained is a vital marketing
tool and is the least expensive advertising available per exposure.

Contact: Vern Wooten

ouaiiee’ Dyglite Sales & Service, Inc. Tel: 513-536-3193

E-Mail: vwooten@dualite.com
FAX 513-536-3116

WILLIAMSBURG, OHIO + CEDAR HILL, TEXAS






[image: image6.jpg]Ground Signs
@ oF ey 15'9" X 25'4" 338.87
5350 R
m 535 14'93/8" X 23'8 207.42
300 T
m S$:399, |38 x2rt012| 26523
S-250 SR HGGE
m 525, 12'6"X 200 21262
-2 S
m S5 12X 1711 17012
S-150 e
e} 518 98 X156 127.47
100 . aia
= S 7101/2' X 12'8 84.86
.75 S0l gig
[l S, 610 1/4' X 10' 11 6397
i) S50 57'X8 1138 4282
DF/ CP.
S-42 10" o
B |t Brce| sioxre 36.46
(-] e 4TUXT 2 2816
Building Signs e
@ $-200 12 X 17 11 170.12
SFWM
Recessed Building Signs re-
quire special consideration oiine] = 98 X15'6" 127.47
during installation. Tem-
plate sleeves are available = ssé/'vm 7101/2'X 128" 84.86
for sign sizes 5-33 through
S-100. Full size patterns are [l $75 610 1/4" X 10 117 6397
SFWM
available for $150.00
S50 R .
= S 57'X8 1138 4282
533 —
-] e 4714 X7 2 28.16
s-8
= SFWM 2284 X3 7" 6.86

DI r tl n | Directionals assist your guests in finding not only the entrance to your property but also guides
eC O a S them to specific areas. All of these signs can be customized to fit your particular needs.

-6 11/16"

31 174t

21 18—

enter [ EXIT







[image: image7.jpg]SIGN DETERMINATION
To maximize your signage benefits it should:

1.) Be visible before any exits or turn decisions. For the typical
interstate location, the signage needs to be visible at least

1/4 mile before the decision point to allow motorists ample time
to slow and make any needed lane changes.

2.) Be adequately sized ... height and square footage need to
work together. There is a formula you can use called the rule of
35. Here's how it works:

A. Determine the needed height of the pole sign. This is rel-
atively easy if this is a "conversion" property and there is ex-
isting signage to use for a guideline. If there is any doubt,
have a flag test performed. Be sure to drive every approach.
This not only determines the needed height but also deter-
mines the proper location. Locating the pole sign in the front
of the property may not always provide the best visibility.

B. After the height is determined, multiply the height by 35%
for the minimum square footage that should be installed.
Example:

80' above grade x 35% equals 28.00 or 280 square feet
50' above grade x 35% equals 17.50 or 175 square feet

Remember, this is only intended to act as a general guideline
and is not a guarantee of visibility. You also have to take into ac-
count the distance of the location from the traffic flow, obstruc-
tions and topography.

3.) Assist guests in finding the entrances. Clearly mark entran-
ces and provide interior directionals as needed.

4.) Increase consumer awareness of your property by using wall
signage. Does the guest in the parking lot of your competitor
know you are there?

CODES AND ORDINANCES

Now that you have some idea what signage is needed, where
they are to be located and how many you need, it's time to de-
termine what the local code will allow. Should the local codes
not allow the sign package you deem necessary, you need to
establish a list of the minimum signage you feel is needed and
determine how this compares to the code allowance. Is it close?
Depending on how close, you may decide to take what is offered
or request a variance. Variances are not that difficult and are
usually successful when a reasonable request is made. In addi-
tion, do their setback regulations allow you to place the ground
sign in the location you've spotted?

PERMITS

Once permits are in hand, you can then order the signage keep-
ing in mind there is usually a 3 to 4 week lead time before ship-
ping. To save time, have the sign vendor prepared to begin pro-
duction once permits are in hand.

INSTALLATION

Use a reputable contractor to handle your installation. You should re-
ceive from them a certificate of insurance and a copy of their workers
compensation insurance. In addition, you should obtain at least three
references from customers for whom they have completed similar
projects. In addition, some of them should be for work completed a
couple of years earlier. This is the only way to determine if their work
stands the test of time. Finally, they should obtain and provide a copy
of a sealed engineering print for all pole sign installations over 30"
above grade. The cost is usually in the $300.00 dollar range which is
inexpensive insurance when you consider the ramifications of a failure
at a later date. In addition, you will always have this available should
you need to replace the sign in the future.

TIMELINES

* Sign Package Determination: 1 to 2 weeks
* Installation Quotes: 2to 3 weeks
* Permits (without a variance): 2 to 3 weeks
* Sign Delivery: 4 to 5 weeks

Ideally, you should allow 4 months from beginning to completion for
your sign package. Some of these items are flexible ... you may know
your sign package in 2 days ... you may only use one installer ... per-
mits may become available in 2 days. Conversely, a variance may be
needed or a Design Review Board hearing may be required ... add
30 to 90 days ... you may need special engineering or pole structures
... add 1to 2 weeks. No one can tell you exactly how long it will take
to complete the project from beginning to end.

There are too many variables to establish a completion date without the
above information, however, there are two items that are absolutes:

1.) High rise pole structures should have a foundation that has been
cured for 28 days before installing the sign.

2.) It takes time to assemble and deliver the sign package. As you will
find when checking the local sign codes, allowable sign sizes will vary
from location to location. For that reason, signs are not built in
advance and held for shipping. In addition, the warranty on electrical
components begins on the date the signs are finally assembled
Dualite stocks components and sign faces in order to maximize our
ability to provide quick delivery.

Dualite has been and continues to be the sole supplier to La Quinta
Corporate since 1993 for several reasons ...

Service * Premium Quality * Value

Unlike other hospitality chains that only franchise, LaQuinta owns and
operates almost 300 properties. They not only look at the initial cost
of a product, they evaluate the long term costs including maintenance
and service. They realize saving a few percent today is not cost effec-
tive in the long run.

We have the knowledge ... we have the experience ... we have the
product. Trust the company LaQuinta has trusted over 300 times.

If you have any questions or require further assistance, call Vernon
Wooten @ 513-536-3193 or e-mail: vwooten @dualite.com






[image: image8.jpg]IDENTIFICATION REQUIREMENTS
Consistent identification coast to coast is the key to the suc-
cess of your property. Extensive marketing efforts are under-
taken in your behalf and to fully realize their benefits, the cor-
rect display of our logo is imperative. To assist you in this
process, we have compiled this handbook.
LOGO REQUIREMENTS

1. All properties are required to display the trademarked logo.

2. No copy alterations are permitted.

3. To insure the accurate reproduction of the logo, only ap-
proved vendors may be used.

4. Trademarked logos cannot be used without written au-
thorization from La Quinta corporate office.
SIGN CABINETS
The correct display of the logo requires a properly sized cabi-
net. A non-standard cabinet can only be used with the prior ap-
proval of the Corporate Sign Department.
All accessory signs on the same structure must have the same
cabinet color as the accompanying logo sign.

SUPPORT POLE/CLADDING

All support poles and pole cladding must be painted Frosty
White.

READERBOARDS

Readerboards may be used in conjunction with the logo pro-
vided the cabinets are painted Gold to match the accompany-
ing logo sign. We recommend at least three (3) lines of 8" copy
at minimum.

GRAPHICS

The accurate reproduction of the logo is imperative to maintain-
ing a consistent image for all properties. For this reason, ONLY
APPROVED VENDORS MAY BE USED. Approval must be re-
ceived before beginning production.

FACE MATERIALS

Larger faces are to be made from Signtech Flexface with first
surface decoration by way of screen printing or heat transfer
graphics. Smaller faces are to be either GE Solar Grade lexan
or Sheffield Hyzod Sunlife polycarbonate pan formed and em-
bossed (if suitable) and decorated via second surface screen
printing.

ILLUMINATION

ONLY high-ouput daylight fluorescent lamps are approved for il-
lumination source. No HID's are permitted.

CUSTOM FACES/SIGNS

Consistent signage is an important element of successful mar-
keting. In those instances where restrictive codes will not allow
the installation of a new sign without loss of either size or height,
face replacements to cabinets may be allowed with prior appro-
val of the Corporate Sign Department.

The use of your name along with the logo must be approved by
the Corporate Sign Department.

NON-APPROVED VENDORS

Our approved vendors are familiar with our programs and use
stringent quality controls to insure the accurate reproduction of
our logo. Non-approved vendors may only be used upon receipt
of warranty information, proof of liability insurance, proof of
workers comp insurance, detailed engineering plans and
graphics layout. Logo specifications will be sent for their use on-
ly with the written approval of La Quinta corporate offices.

One

Dualite

WILLIAN URG, OHIO « CEDAR HILL, TEXAS

Dualite Lane -

Dualite Sales & Service, Inc.

Williamsburg,

Contact Vern Wooten
Tel: 513536 3193

E-Mail: vwooten@dualite.com
FAX 513-536-3116

Ohio J 45176







_1126008190.unknown

