Kirt

STRUCTURAL INSPECTION

Date:
November 10, 2005
Type:
Residential

RE:
Post Hurricane Katrina Structural Inspection

Materials:
Brick veneer/wood frame/composition shingles

Owner(s):
Mr. Chris Keating


Property: 
150 Moonraker Drive


924 Tidewater Drive




Eden Isles


Slidell, LA  70458




Slidell, LA  70458
History:

Mr. Roy Stubenrauch, General Contractor, was performing post-Katrina gutting of the dwelling, when he saw that the bottom six inches of the sheathing material (black board) on the exterior wall behind the brick veneer had rotted.  He asked that I investigate what was happening and the proper method to repair the black board.
Findings:

Mr. Keating pointed out the areas where the black board is rotting at the bottom.
Analysis:

The brick veneer exterior prevents getting to the black board from the outside.  Methods used to make repairs from the inside are for very small areas only, and all of this house requires replacing the black board at its bottom 12”.  The only way to properly accomplish this is to remove the brick veneer, cut and install new black board, and install new or cleaned brick.
Conclusion:

It is my professional opinion that the brick veneer must be removed and the system installed correctly.
Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


