chuck

STRUCTURAL INSPECTION

Date:
February 6, 2006

Type:
Commercial Building

RE:
Structural Inspection

Property:
2201 Williams Blvd.

Kenner, La. 70062

Structure:
Cinder Block/Wood Frame/Brick Veneer

Attention:
St. Paul Travelers

Richard Folse Jr

P.O. Box 33

Luling, La. 70070

Owner:

Kenneth Retif


Retif Oil and Fuel



527 Destrehan Ave.



Harvey, LA  70058
History:

Dammon Engineering was contacted to perform a structural inspection on the above Property.  The owner was concerned about damage to the 40-year-old structure of the building caused by a recent tornado.
Findings:

During a visual inspection of the building, the following was noted:

A) One third of the roof and ceiling had been removed by the high winds.

B) A majority of the cinder blocks was blown out and the ones remaining have cracks.

C) The ceilings are damaged from the rainwater coming through.

D) The A/C systems are complety gone off of the roof. The A/C duct work is damaged.

E) The electrical service’s are all damaged and pulled away from the walls.

F) The plumbing systems are all damaged and displaced. 

G) There are hairline cracks in the foundation.

Analysis:

1. Replacement of the roof and repairs to the supporting structure are not recommended.

2. The entire bldg. has exceeded the recommended wind load that it was designed for.

3. The entire electrical service and distribution will have to be replaced, as it does not meet the new codes.

4. All of the above and under ground plumbing has been wracked and will leak at its joints. 

5. The A/C ducts have been wet and will have mold issues if not replaced.
Conclusion:

It is my professional opinion that this building is not safe to rebuild.  It should be totally removed, including the foundation.  The new building must be designed to meet the new International Building Code.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


