STRUCTURAL INSPECTION

Date:
November 2, 2005

Type:
Residential

RE:
Post-Hurricane Katrina Structural Inspection

Structure:
Wood frame on concrete foundation/seal tab shingle roof. Texture 111 siding.

Owner(s): 
Paul Roy

Property: 
2218 Oriole St.


Slidell, La.



985-640-6844



985-892-9122

History:

Mr. Roy contacted Dammon Engineering to perform a structural inspection on the above residence.  He was concerned about damage to the foundation as well as roof damage from two fallen trees, which hit the original house and an addition to the house.
Findings:

After a visual inspection of the home, the following was noted:

1. On the north side of the house, the soffit, fascia, felt paper, decking and roof membrane are damaged.  A blue tarp covered this section of the roof

2. On the northwest side of the house the fascia, and soffit are damaged.

3. A second tree hit the east side of the addition.  On the northeast corner of the fascia board, soffit, decking, felt paper and roof membrane are damaged

4. Over 50% of the sheet rock ceiling and insulation has come down in the northwest bedroom.

5. Water damaged all of the oak parkay flooring in the bedroom

6. The addition area also suffered falling sheetrock, generally to the northeast corner.  Damage was found both in the ceiling and walls.

7. An inspection of the attic revealed severe structural damage to the north end of the roof.

8. The northwest corner beam is cracked, 2-3 rafters are cracked, and the sheathing has a hole approximately 24” in diameter.  The felt paper and the roof membrane are also damaged.

9. The complete north sides of the gable roof’s rafters are cracked.  Sheathing is damaged to over 50 %, and the felt papers as well as the roof membrane are damaged.

10. The main beam where the 2 corner beams connect on the north side is cracked.

11. Northwest and northeast support beams are cracked.

12. Daylight is visible to the west side fascia, and to the south side where the front porch meets the gable roof.

13. Level readings taken from the slab show readings from 0.0 – 1.0 throughout the house.

Analysis:
Damage from the trees that fell on the house is the result of Hurricane Katrina. Although damage to the roof appears to be less than 50%, the complete roof will need to be repaired and replaced due to the main beam that supports the rafters being cracked.

Conclusion:

It is my professional opinion that the general structure is sound, and once the above repairs to the roof system are accomplished, this dwelling will return as originally constructed.

Additional Notes:

When work is being accomplished to rebuild the structure, fastenings and nail patterns should meet the new International Building Code requirements for that work.

Information homeowners should know about flood damaged walls and floors, or those damaged by water entry from rain, etc.

The wood stud walls will need to be dried out prior to covering with new wallboard.  In order to dry out the wood studs, one side of the wall will need to be removed and allow sufficient drying time to occur.  Mold remediation should occur as soon as possible.  The American Wood Council states that wood needs to dry out to less than 19% moisture content prior to enclosing wood studs, otherwise the structural integrity could be compromised and mold growth could occur.  Contractors should not re-install wallboard until the moisture content in the stud wall is below 19%.  Houses built using thicker wood products should expect longer dry out times.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796
This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


