STRUCTURAL INSPECTION

Date:
June 22, 2006

Type:
Primary residence

RE:
Post-Hurricane Katrina Structural Inspection

Structure:
Wood frame, CMU block piers, metal siding, flat roof

Owner:

Mary Holland

Reference:
223 Carr Dr.



Slidell, La

640-0803

History:

Mrs. Holland contacted Dammon Engineering to perform a structural inspection on the above residence.  She was concerned about damages to the home cause by Hurricane Katrina and requested an inspection to determine if a third floor could be added to the existing foundation.
Findings:

During a visual inspection of the home, the following was noted:

1. The ground floor consists of four CMU block walls running north and south.

2. The first living area is approximately ten feet above the concrete foundation.

3. The first living area is constructed of four load bearing wood stud walls running north and south matching the CMU walls below.

4. The roof is flat and leaks into the first floor living area.
Analysis

There is no damage to the CMU walls on the ground floor. There is water damage to the ceiling of the ground floor and it should be removed so proper drying can occur.  The first floor living area is water damaged by the leaking roof.  Floor damage is consistent with the rainwater from the roof damage.

Conclusion:

It is my professional opinion that this home can be repaired and an additional third floor can be designed and added to the existing structure.

Sincerely,

Emmett G. (Pete) Dammon, P.E.

Louisiana License No. 8796

Attachment:
Hurricane Katrina/Effects on Lake Pontchartrain

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.

This inspection is limited to the apparent visual conditions of the structural components of this building.  It does not cover, nor attempts to cover, any components, items, and/or conditions which, by their nature or location, are concealed or are difficult or hazardous to inspect, or which require the moving of furniture, flooring materials, rugs, fixtures, appliances, or any component-part nailed, bolted, or screwed down or shut.  No opinions are expressed regarding conditions which could be discovered only by the disassembly of any component parts, special testing, or removal of any concealing objects.

Inspections are made under normal weather conditions, and are not opinions of the conditions of the property and/or structure which may exist under unusual weather conditions, such as, but not limited to floods, heavy rains or snows, high winds, temperature extremes, or any act of God.  Specific hazardous wastes, toxic substances, toxic mold, air and water quality, communicable diseases, asbestos, soil, environmental, radon, carbon monoxide, formaldehyde, building code and termite conditions are not included in this report unless otherwise stipulated.

This report is not a warranty or guarantee of the property inspected, but it is our opinion of its condition at the time inspected.  Our liability shall be limited to reimbursement of the total cost of inspection.


